3GPP TSG-RAN WG4 Meeting #92                                         R4-1908073

Ljubljana,SI, 26th – 30th May, 2019

Agenda Item:   9.1.2.2

Source:        Samsung

Title:           Spectrum utilization for NR-U

Document for:  Discussion

# Introduction

In RAN4#90 meeting it was agreed for spectrum utilization for case of CBW of 20MHz with 15 kHz and 30 kHz SCS as table below [1].

|  | 20 MHz |
| --- | --- |
| 15 kHz | 106 |
| 30 kHz | 51 |

However, there is still no agreement on SU for 20MHz with 60 kHz and SU for larger bandwidth cases. This contribution provides our view on this aspect.

# Discussion

As discussed in NR SI phase many aspects, such as unwanted emission level, average EVM, edge EVM, impact to ISI/ICI/other timing critical procedure, complexity and PA impact, should be considered for decision on spectrum utilization. Furthermore, it would be common understanding that the evaluation on spectrum utilization should be based on certain assumption of PA model and confined by emission requirements, which are referred to SEM for UE, OBUE for gNB and ACLR for both.

In the context of NR-U discussion, the agreement on BS side is captured in [2] as

- 3. 35dB ACLR requirements can be reused for NR-U single carrier <=20MHz and CA.

- 4. LAA Operating band unwanted emission mask can be reused by NR-U single carrier <=20Mhz and CA.

- 5. For NR-U for channel bandwidths above 20 MHz OBUE masks should be considered.

Obviously, the OBUE and ALCR requirement for gNB operating in unlicensed band with carrier of 10MHz &20MHz and CA is clear early this year.

However, there is still no agreement achieved for the OBUE and ACLR for CHBW larger than 20MHz for BS side. And the PA model, SEM and ACLR for all CHBW seems not fixed for UE as well. In addition, there is pending issue on how to interpret ETSI regulation for NR-U which may impact the unwanted emission level for wideband operation on NR-U.

Here is the summarized list for corresponding pending issues in table below:

| Channel BW@SCS(Hz) | Agreement on SU | BS OBUE and ACLR | UE SEM and ACLR |
| --- | --- | --- | --- |
| 10MHz@15K/30K | TBD | LAA requirement reused | TBD |
| 20MHz@15k/30k and CA | NR requirement reused | LAA requirement reused | TBD |
| 20MHz@60k | TBD | LAA requirement reused | TBD |
| CHBW>20MHz | TBD | TBD | TBD |

Considering the agreement on spectrum utilization would be applied for both UE and BS, it is suggested the spectrum utilization should be agreed based on at least tentative range of emission level on adjacent channel , which is the fundamental restriction on SU evaluation, as a package.

# Summary

In this contribution we summarized the status of SU discussion for NR-U and status for relative RF requirement. It is proposed to agree on SU based on at least tentative agreement on unwanted emission and/or ACLR.

# Reference

[1] RAN4#90 Chairman Notes

[2] R4-1902505, WF on NR-U BS RF requirements