3GPP TSG-RAN WG4 Meeting #92                                      R4-1908085

Ljubljana, Slovenia, 26th – 30th Aug, 2019

Agenda item:			7.11.1.1.2

Source:			Samsung

Title:	Discussion on Noc setup and side conditions for FR2 testing

Document for:		Discussion

# 1	Introduction

In the latest TS38.133 v15.6.0, there are still many “TBD”s remained for test conditions, such as Noc setup and side condition especially for FR2. In this contribution, we would like to provide our view on related issues.

# Noc setup and SNR range

TR38.810 has performed analysis for the achievable SNR range and corresponding Noc level to be configured in the quite zone, however we found there are still issues existed for 1AoA non-peak direction setup and 2AoA setup. Specifically, in current calculation, the maximum achievable SNR lower bound is decreased with the same amount as NoC due to rough beam and/or non-peak direction, as demonstrated in below table B.2.1.5.4-2; however, the impact of increased power for Noc is not taken into account in SNR range calculation.

<From TR38.810 v16.3.0> Inclusion of these two contributions directly reduces the maximum SNR that can be measured by a test system for a given channel bandwidth. To find the maximum SNR that can be measured by a test system with a specific Channel BW, the baseband SNR in the spreadsheet is increased until the value “Wanted signal + headroom, dBm/Ch BW” is just below the “TE Power amplifier 1dB compression, dBm” value. The resulting values are given in Table B.2.1.5.4-1. Table B.2.1.5.4-1: Predicted SNR upper bound values for Direct far field (DFF), Rx Beam peak direction Channel Bandwidth; Maximum SNR. 100MHz; [18.5dB]. Note that these are UE baseband SNR values, so the Reference point figures used in RRM test cases may be 1dB higher. Values are based on UE parameters and currently foreseen test equipment limitations, and could be improved in future. An example of SNR calculation for DFF method is provided in “Spreadsheet 1 - RRM SNR range calculator.xls” file attached to the TR. The feasible Noc and SNR range for other RRM scenarios, requirement types and S / N generation modes, can be assessed and given relative to the Noc and SNR range defined in the case above. Table B.2.1.5.4-2 summarizes the results. Table B.2.1.5.4-2: Summary of SNR range and Noc feasibility for RRM DFF setup Scenario; Requirement Type; S / N Configuration Mode; Noc; SNR Range; Notes. 1; “Fine” beams; 1 (S and N from TE); Noc; SNR; Note 1. 1; “Rough" beams; 1 (S and N from TE); Noc + Y; SNR - Y; Note 2. 2; “Fine” beams; 1 (S and N from TE); Noc + X; SNR - X; Note 3. 2; “Rough" beams; 1 (S and N from TE); Noc + X+ Z; SNR - X - Z; Note 4. Note 1: Noc as specified in section 6.2.1.4.3. Respective SNR defined as per Section B.2.1.5.4 Note 2: Y as specified in section 6.2.1.4.4. Note 3: X as specified in section 6.2.1.4.5. Note 4: X and Z as specified in section 6.2.1.4.6.

Take Scenario-2 with “rough” beams for requirement type as example, the achievable SNR at baseband is decreased as low as -2.5dB for 50MHz band n260, while decreased to the totally unusable level for 100MHz case. The reason comes from that NoC is increased to a level comparable to useful signal and increasing NoC also means less power can be allocated to useful signal, thus decreasing the achievable SNR level accordingly.

Observation 1: In TR38.810 v16.3.0, the method to calculate maximum achievable SNR lower bound (e.g., SNR-X-Z for scenario 2 with rough beam assumed) is not correct, because that NoC is increased to a level comparable to useful signal and not ignorable in SNR calculation.

Here we would like to skip the link budget calculation but directly comes to the conclusion, while the detailed link calculation is provided in the Table 5.1 and 5.2 in Appendix of this contribution. Based on the tables in Appendix, the following observations can be reached:

Observation 2: Based on the link budget calculation, with artificial noise Noc utilized, the minimum Noc level and maximum baseband SNR level are provided as below:

| Band n260, 100MHz, with Multi-band relaxation |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Scn-1, Fine (1AoA Peak, Fine) | Scn-1, Rough (1AoA Peak, Rough) | Scn-2, Fine (1AoA Non-P, Fine) | Scn-2, Rough (1AoA Non-P, Rough) |
| Min Noc (dBm/Hz) | -153.8 | -146.8 | -141.2 | -134.2 |
| Min Noc (dBm/15kHz) | -112.0 | -105.0 | -99.4 | -92.4 |
| Min Noc (dBm/SCS) | -103.0 | -96.0 | -90.4 | -83.4 |
| Max Es/Noc under this Noc | 18.4dB | 11.2dB | 5.2dB | Not Usable |
| Max Baseband SNR under this Noc | 17.4dB | 10.2dB | 4.2dB | Not Usable |

Proposal 1: For different scenarios (1AoA peak/non-peak, fine/rough beam) with artificial noise utilized, RRM test cases should be defined as being agreed with above limitations for minimum NoC and maximum Es/Noc.

As discussed in testability discussion, by not applying artificial noise, the achievable SNR at baseband can be increased considerably. As demonstrated in Appendix, the following observations can be reached:

Observation 3: Based on the link budget calculation, without artificial noise Noc utilized, the maximum baseband SNR level are provided as below:

| Band n260, 100MHz, with Multi-band relaxation |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Scn-1, Fine (1AoA Peak, Fine) | Scn-1, Rough (1AoA Peak, Rough) | Scn-2, Fine (1AoA Non-P, Fine) | Scn-2, Rough (1AoA Non-P, Rough) |
| Max Wanted Signal (dBm/Hz) | -135.3 |  |  |  |
| Max Wanted Signal (dBm/SCS) | -84.5 |  |  |  |
| Max Baseband SNR | 24.5dB | 17.5dB | 12.3dB | 5.3dB |

Proposal 2: For different scenarios (1AoA peak/non-peak, fine/rough beam) without artificial noise utilized, RRM test cases should be defined as being agreed with above limitations for maximum Es/Noc.

# Conclusion

In this paper, we have provided our view on FR2 OTA testing issues, such as Noc setup and side condition, with the following observations and proposals obtained:

Observation 1: In TR38.810 v16.3.0, the method to calculate maximum achievable SNR lower bound (e.g., SNR-X-Z for scenario 2 with rough beam assumed) is not correct, because that NoC is increased to a level comparable to useful signal and not ignorable in SNR calculation.

Observation 2: Based on the link budget calculation, with artificial noise Noc utilized, the minimum Noc level and maximum baseband SNR level are provided as below:

| Band n260, 100MHz, with Multi-band relaxation |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Scn-1, Fine (1AoA Peak, Fine) | Scn-1, Rough (1AoA Peak, Rough) | Scn-2, Fine (1AoA Non-P, Fine) | Scn-2, Rough (1AoA Non-P, Rough) |
| Min Noc (dBm/Hz) | -153.8 | -146.8 | -141.2 | -134.2 |
| Min Noc (dBm/15kHz) | -112.0 | -105.0 | -99.4 | -92.4 |
| Min Noc (dBm/SCS) | -103.0 | -96.0 | -90.4 | -83.4 |
| Max Es/Noc under this Noc | 18.4dB | 11.2dB | 5.2dB | Not Usable |
| Max Baseband SNR under this Noc | 17.4dB | 10.2dB | 4.2dB | Not Usable |

Proposal 1: For different scenarios (1AoA peak/non-peak, fine/rough beam) with artificial noise utilized, RRM test cases should be defined as being agreed with above limitations for minimum NoC and maximum Es/Noc.

Observation 3: Based on the link budget calculation, without artificial noise Noc utilized, the maximum baseband SNR level are provided as below:

| Band n260, 100MHz, with Multi-band relaxation |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Scn-1, Fine (1AoA Peak, Fine) | Scn-1, Rough (1AoA Peak, Rough) | Scn-2, Fine (1AoA Non-P, Fine) | Scn-2, Rough (1AoA Non-P, Rough) |
| Max Wanted Signal (dBm/Hz) | -135.3 |  |  |  |
| Max Wanted Signal (dBm/SCS) | -84.5 |  |  |  |
| Max Baseband SNR | 24.5dB | 17.5dB | 12.3dB | 5.3dB |

Proposal 2: For different scenarios (1AoA peak/non-peak, fine/rough beam) without artificial noise utilized, RRM test cases should be defined as being agreed with above limitations for maximum Es/Noc.

# Reference

[1] TR38.810, “NR; Study on test methods”, 3GPP.

# Appendix: Link-budget Calculation

In the following Table 5.1 and 5.2, we provide our results for Scenario-1/2 with fine/rough beam assumed respectively, in which PC3, 50MHz/100MHz bandwidth, band n260 are assumed. To find the maximum SNR that can be measured by a test system with a specific Channel BW, the baseband SNR in the spreadsheet is increased until the value “Wanted signal + headroom, dBm/Ch BW” is just below the “TE Power amplifier 1dB compression, dBm” value.

Table 5.1 Link-budget calculation (PC3, 50MHz bandwidth, band n260, with multiband relaxation, with artificial noise)

| Scenario-1: 1AoA, Peak Direction (Fine Beam) |  | Scenario-1: 1AoA, Peak Direction (Rough Beam) |  | Scenario-2: 1AoA, Non-Peak Direction (Fine) |  | Scenario-2: 1AoA, Non-Peak Direction (Rough) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Power Class | PC3 | Y-value | 7.0 | REFSENS | -85.7 | X-value = EIS Spherical - REFSENS | 12.6 |
| SCS (kHz) | 120 |  |  | EIS Spherical Coverage | -73.1 | Z-value | 7 |
| Bandwidth | 50MHz |  |  | X-value = EIS Spherical - REFSENS | 12.6 |  |  |
| Band | n260 |  |  |  |  |  |  |
| N_RB | 32 |  |  |  |  |  |  |
| PC_Band | PC3_n260 |  |  |  |  |  |  |
| REFSENS at reference point | -84.0 | REFSENS at reference point | -77.0 | REFSENS at reference point | -71.3 | REFSENS at reference point | -64.3 |
| REFSENS (per Hz) at reference point | -160.6 | REFSENS (per Hz) at reference point | -153.6 | REFSENS (per Hz) at reference point | -147.9 | REFSENS (per Hz) at reference point | -140.9 |
| UE Noise (per Hz) at reference point (virtual concept) | -159.6 | UE Noise (per Hz) at reference point (virtual concept) | -152.6 | UE Noise (per Hz) at reference point (virtual concept) | -146.9 | UE Noise (per Hz) at reference point (virtual concept) | -139.9 |
| Noc level (per Hz) at reference point | -153.6 | Noc level (Rough Beam, per Hz) at RP | -146.6 | Noc level (Non-Peak, Fine) | -141.0 | Noc level (Non-Peak, Rough) | -134.0 |
| Baseline SNR Range (PC3, n260, 50MHz) |  | Baseline SNR Range (PC3, n260, 50MHz) |  | Baseline SNR Range (PC3, n260, 50MHz) |  | Baseline SNR Range (PC3, n260, 50MHz) |  |
| Total Noise (per Hz) at reference point (virtual concept) | -152.7 | Total Noise (per Hz) at RP (virtual) | -145.7 | Total Noise (per Hz) at RP (virtual) | -140.0 | Total Noise (per Hz) at RP (virtual) | -133.0 |
| Target SNR at Baseband | 20.5 | Target SNR at Baseband | 13.0 | Target SNR at Baseband | 7.5 | Achievable SNR at Baseband | -2.5 |
| Wanted Signal (per Hz) at reference point | -132.2 | Wanted Signal (per Hz) at reference point | -132.7 | Wanted Signal (per Hz) at reference point | -132.5 | Wanted Signal (per Hz) at reference point | -135.5 |
| SNR at reference point (Wanted signal/Noc) | 21.5 | SNR at reference point (Wanted signal/Noc) | 14.0 | SNR at reference point (Wanted signal/Noc) | 8.5 | SNR at reference point (Wanted signal/Noc) | -1.5 |
| Noc level (per Hz) at Probe Ant. Input | -96.1 | Noc level (per Hz) at Probe Ant. Input | -89.1 | Noc level (per Hz) at Probe Ant. Input | -84.0 | Noc level (per Hz) at Probe Ant. Input | -77.0 |
| Wanted Signal (per Hz) at Probe Ant. Input | -74.7 | Wanted Signal (per Hz) at Probe Ant. Input | -75.2 | Wanted Signal (per Hz) at Probe Ant. Input | -75.5 | Wanted Signal (per Hz) at Probe Ant. Input | -78.5 |
| SNR at Probe Ant. Input (Wanted signal/Noc) | 21.5 | SNR at Probe Ant. Input (Wanted signal/Noc) | 14.0 | SNR at Probe Ant. Input (Wanted signal/Noc) | 8.5 | SNR at Probe Ant. Input (Wanted signal/Noc) | -1.5 |
| Noc level (per Hz) at TE amp. output | -88.1 | Noc level (per Hz) at TE amp. output | -81.1 | Noc level (per Hz) at TE amp. output | -76.0 | Noc level (per Hz) at TE amp. output | -69.0 |
| Wanted Signal (per Hz) at TE amp. output | -66.7 | Wanted Signal (per Hz) at TE amp. output | -67.2 | Wanted Signal (per Hz) at TE amp. output | -67.5 | Wanted Signal (per Hz) at TE amp. output | -70.5 |
| SNR at TE amp. output (Wanted signal/Noc) | 21.5 | SNR at TE amp. output (Wanted signal/Noc) | 14.0 | SNR at TE amp. output (Wanted signal/Noc) | 8.5 | SNR at TE amp. output (Wanted signal/Noc) | -1.5 |
| Total Signal (Wanted Signal + Noc) + headroom | 23.00 | Total Signal (Wanted Signal + Noc) + headroom | 22.64 | Total Signal (Wanted Signal + Noc) + headroom | 22.67 | Total Signal (Wanted Signal + Noc) + headroom | 22.92 |

Table 5.2 Link-budget calculation (PC3, 100MHz bandwidth, band n260, with multiband relaxation, with artificial noise)

| Scenario-1: 1AoA, Peak Direction (Fine Beam) |  |  | Scenario-1: 1AoA, Peak Direction (Rough Beam) |  |  | Scenario-2: 1AoA, Non-Peak Direction (Fine Beam) |  |  | Scenario-2: 1AoA, Non-Peak Direction (Rough Beam) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Power Class | PC3 |  | Y-value | 7 |  | REFSENS | -82.7 |  | X-value = EIS Spherical - REFSENS | 12.6 |
| SCS (kHz) | 120 |  |  |  |  | EIS Spherical Coverage | -70.1 |  | Z-value | 7 |
| Bandwidth | 100MHz |  |  |  |  | X-value = EIS Spherical - REFSENS | 12.6 |  |  |  |
| Band | n260 |  |  |  |  |  |  |  |  |  |
| N_RB | 66 |  |  |  |  |  |  |  |  |  |
| PC_Band | PC3_n260 |  |  |  |  |  |  |  |  |  |
| REFSENS at reference point | -81.0 |  | REFSENS at reference point | -74.0 |  | REFSENS at reference point | -68.3 |  | REFSENS at reference point | -61.3 |
| REFSENS (per Hz) at reference point | -160.8 |  | REFSENS (per Hz) at reference point | -153.8 |  | REFSENS (per Hz) at reference point | -148.1 |  | REFSENS (per Hz) at reference point | -141.1 |
| UE Noise (per Hz) at reference point (virtual concept) | -159.8 |  | UE Noise (per Hz) at reference point (virtual concept) | -152.8 |  | UE Noise (per Hz) at reference point (virtual concept) | -147.1 |  | UE Noise (per Hz) at reference point (virtual concept) | -140.1 |
| Min Noc level (per Hz) | -153.8 |  | Min Noc level (Rough Beam, per Hz) at RP | -146.8 |  | Min Noc level (Non-Peak, Fine) | -141.2 |  | Min Noc level (Non-Peak, Rough) | -134.2 |
| SNR Range (Target System) |  |  | SNR Range (Target System) |  |  | SNR Range (Target System) |  |  | SNR Range (Target System) |  |
| Total Noise (per Hz) at reference point (virtual concept) | -152.8 |  | Total Noise (per Hz) at RP (virtual) | -145.8 |  | Total Noise (per Hz) at RP (virtual) | -140.2 |  | Total Noise (per Hz) at RP (virtual) | -133.2 |
| Target SNR at Baseband | 17.4 |  | Target SNR at Baseband | 10.2 |  | Target SNR at Baseband | 4.2 |  | Achievable SNR at Baseband | -20.0 |
| Wanted Signal (per Hz) at reference point | -135.4 |  | Wanted Signal (per Hz) at reference point | -135.6 |  | Wanted Signal (per Hz) at reference point | -136.0 |  | Wanted Signal (per Hz) at reference point | -153.2 |
| SNR at reference point (Wanted signal/Noc) | 18.4 |  | SNR at reference point (Wanted signal/Noc) | 11.2 |  | SNR at reference point (Wanted signal/Noc) | 5.2 |  | SNR at reference point (Wanted signal/Noc) | -19.0 |
| Noc level (per Hz) at Probe Ant. Input | -96.3 |  | Noc level (per Hz) at Probe Ant. Input | -89.3 |  | Noc level (per Hz) at Probe Ant. Input | -84.2 |  | Noc level (per Hz) at Probe Ant. Input | -77.2 |
| Wanted Signal (per Hz) at Probe Ant. Input | -77.9 |  | Wanted Signal (per Hz) at Probe Ant. Input | -78.1 |  | Wanted Signal (per Hz) at Probe Ant. Input | -79.0 |  | Wanted Signal (per Hz) at Probe Ant. Input | -96.2 |
| SNR at Probe Ant. Input (Wanted signal/Noc) | 18.4 |  | SNR at Probe Ant. Input (Wanted signal/Noc) | 11.2 |  | SNR at Probe Ant. Input (Wanted signal/Noc) | 5.2 |  | SNR at Probe Ant. Input (Wanted signal/Noc) | -19.0 |
| Noc level (per Hz) at TE amp. output | -88.3 |  | Noc level (per Hz) at TE amp. output | -81.3 |  | Noc level (per Hz) at TE amp. output | -76.2 |  | Noc level (per Hz) at TE amp. output | -69.2 |
| Wanted Signal (per Hz) at TE amp. output | -69.9 |  | Wanted Signal (per Hz) at TE amp. output | -70.1 |  | Wanted Signal (per Hz) at TE amp. output | -71.0 |  | Wanted Signal (per Hz) at TE amp. output | -88.2 |
| SNR at TE amp. output (Wanted signal/Noc) | 18.4 |  | SNR at TE amp. output (Wanted signal/Noc) | 11.2 |  | SNR at TE amp. output (Wanted signal/Noc) | 5.2 |  | SNR at TE amp. output (Wanted signal/Noc) | -19.0 |
| Total Signal (Wanted Signal + Noc) + headroom | 22.92 |  | Total Signal (Wanted Signal + Noc) + headroom | 22.98 |  | Total Signal (Wanted Signal + Noc) + headroom | 22.92 |  | Total Signal (Wanted Signal + Noc) + headroom | 23.63 |

For the case without artificial noise, the calculation is simpler and demonstrated as below Table 5.3:

Table 5.3 Link-budget calculation (PC3, 100MHz bandwidth, band n260, with multiband relaxation, w/o artificial noise)

| Scenario-1: 1AoA, Peak Direction (Fine Beam) |  |  | Scenario-1: 1AoA, Peak Direction (Rough Beam) |  |  | Scenario-2: 1AoA, Non-Peak Direction (Fine Beam) |  |  | Scenario-2: 1AoA, Non-Peak Direction (Rough Beam) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted signal (per Hz) at TE amp. Output | -69.8 |  | Wanted signal (per Hz) at TE amp. Output | -69.8 |  | Wanted signal (per Hz) at TE amp. Output | -69.8 |  | Wanted signal (per Hz) at TE amp. Output | -69.8 |
| Wanted signal (per Hz) at Probe Ant. Input | -77.8 |  | Wanted signal (per Hz) at Probe Ant. Input | -77.8 |  | Wanted signal (per Hz) at Probe Ant. Input | -77.8 |  | Wanted signal (per Hz) at Probe Ant. Input | -77.8 |
| Wanted signal Es per Hz (dBm/Hz) at RP | -135.3 |  | Wanted signal Es per Hz (dBm/Hz) at RP | -135.3 |  | Wanted signal Es per Hz (dBm/Hz) at RP | -134.8 |  | Wanted signal Es per Hz (dBm/Hz) at RP | -134.8 |
| Wanted signal Es per SCS (dBm/SCS) at RP | -84.5 |  | Wanted signal Es per SCS (dBm/SCS) at RP | -84.5 |  | Wanted signal Es per SCS (dBm/SCS) at RP | -84.0 |  | Wanted signal Es per SCS (dBm/SCS) at RP | -84.0 |
| Resultant SNR at Baseband | 24.5 |  | Resultant SNR at Baseband | 17.5 |  | Resultant SNR at Baseband | 12.3 |  | Resultant SNR at Baseband | 5.3 |