3GPP TSG-RAN WG4 Meeting #92                                                               R4-1908099

Ljubljana, SI, 26th-30th August, 2019

Source:	Samsung, Bell Mobility, TELUS, Nokia

Title:	TP for TR 37.716-11-11: DC_66A_n66A

Agenda item:			10.3.2

Document for:	Approval

# 1. Introduction

In RAN#84 meeting, the revised WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band +1 NR band) was approved [1], in which the latest EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band) was attached. This contribution is a text proposal for TR 37.716-11-11 to include DC_66A_n66A only supporting single switch UL mode.

# 2. Reference

- RP-191568, Revised WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band), NTT DOCOMO, INC.

# 3. Text Proposal

<Start of Text Proposal>

# 1	Scope

The present document is a technical report for Dual Connectivity (EN-DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) under Rel-16 time frame. The purpose is to gather the relevant background information and studies in order to address Dual Connectivity (EN-DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL)for the Rel-16 band combinations in Table 1-1. The co-existence analysis and RF front end requirements such as Delta RIB,C and TIB,C are described based on the band combination basis since such information have no difference between the EN-DC configurations consisting with the same E-UTRA band and the same NR band.

Table 1-1: Release 16 EN-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) within FR1

| DC combination | Uplink EN-DC configuration |
| --- | --- |
| DC_1A_n3A | DC_1A_n3A |
| DC_1C_n3A | DC_1C_n3A |
| DC_1A_n5A | DC_1A_n5A |
| DC_1A_n7A | DC_1A_n7A |
| DC_1A_n41A | DC_1A_n41A |
| DC_1A_n50A | DC_1A_n50 |
| DC_2A-2A_n5A | DC_2A_n5A |
| DC_2A_n41A | DC_2A_n41A |
| DC_2C_n41A | DC_2A_n41A |
| DC_3A_n1A | DC_3A_n1A |
| DC_3A-3A_n1A | DC_3A_n1A |
| DC_3A_n3A | DC_3A_n3A |
| DC_3A_n5A | DC_3A_n5A |
| DC_3C_n5A | DC_3A_n5A DC_3C_n5A |
| DC_3A_n20A | DC_3A_n20A |
| DC_3A_n41A | DC_3A_n41A |
| DC_3A_n50A | DC_3A_n50A |
| DC_3A-3A_n77A | DC_3A_n77A |
| DC_3A-3A_n78A | DC_3A_n78A |
| DC_5A_n2A | DC_5A_n2A |
| DC_5A_n71A | DC_5A_n71A |
| DC_5A_n79A | DC_5A_n79A |
| DC_7A_n1A | DC_7A_n1A |
| DC_7A-7A_n1A | DC_7A_n1A |
| DC_7A_n5A | DC_7A_n5A |
| DC_7C_n5A | DC_7A_n5A DC_7C_n5A |
| DC_66A_n66A | DC_66A_n66A |
| DC_7A_n71A | DC_7A_n71A |
| DC_7A_n77A | DC_7A_n77A |
| DC_7A-7A_n77A | DC_7A_n77A |
| DC_8A_n1A | DC_8A_n1A |
| DC_8A_n3A | DC_8A_n3A |
| DC_8A_n41A | DC_8A_n41A |
| DC_8A_n41C | DC_8A_n41A DC_8A_n41C |
| DC_8A_n41(2A) | DC_8A_n41A |
| DC_12A_n2A | DC_12A_n2A |
| DC_12A_n71A | DC_12A_n71A |
| DC_13A_n5A | DC_13A_n5A |
| DC_13A_n71A | DC_13A_n71A |
| DC_20A_n1A | DC_20A_n1A |
| DC_20A_n3A | DC_20A_n3A |
| DC_20A_n41A | DC_20A_n41A |
| DC_20A_n50A | DC_20A_n50A |
| DC_25A-25A_n41A | DC_25A_n41A |
| DC_28A_n5A1 | DC_28A_n5A |
| DC_28A_n8A | DC_28A_n8A |
| DC_28A_n41A | DC_28A_n41A |
| DC_28A_n50A | DC_28A_n50 |
| DC_30A_n2A | DC_30A_n2A |
| DC_39A_n41A | DC_39A_n41A |
| DC_40A_n41A | DC_40A_n41A |
| DC_40A_n78A | DC_40A_n78A |
| DC_40C_n78A | DC_40C_n78A |
| DC_40A_n79A | DC_40A_n79A |
| DC_66A_n2A | DC_66A_n2A |
| DC_66A-66A_n5A | DC_66A_n5A |
| DC_66A-66A-66A_n5A | DC_66A_n5A |
| DC_66A_n25A | DC_66A_n25A |
| DC_71A_n5A | DC_71A_n5A |
| NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |

<Unchanged text omitted>

## 6.4.x	DC_66A_n66A

#### 6.4.x.1	Channel bandwidths per operating band for CA

Table 6.4.x.1-1: Band combinations for intra-band non-contiguous EN-DC

| EN-DC Band Uplink Combination | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_66_n66 | 66 | n66 | Yes |

Table 6.4.x.1-2: Supported channel bandwidths per DC configuration

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for LTE carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for LTE carrier (MHz) |  |  |
| DC_66A_n66A | DC_66A_n66A1 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 50 | 0 |
| NOTE1: Only single switched UL is supported |  |  |  |  |  |  |

#### 6.4.x.2	Configuration for DC

Table 6.4.x.2-1:  Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_66A_n66A | DC_66A_n66A2 | 66A | n66A |
| NOTE2: Only single switched UL is supported |  |  |  |

#### 6.4.x.3	Maximum output power for DC

Table 6.4.x.3-1: Maximum output power for  Intra-band non-contiguous EN-DC

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_66A_n66A2 |  |  | 23 | +2/-3 |
| NOTE 2: Only single switched UL is supported |  |  |  |  |

#### 6.4.x.4	Spurious emission band UE co-existence for DC

Note that only Single Tx Switched UL mode is supported for this combination, no spurious emission band UE co-existence requirement is needed

#### 6.4.x.5	MSD analysis for DC

Note that only Single Tx Switched UL mode is supported for this combination, no MSD requirement is needed

#### 6.4.x.6	∆RIBNC values

The delta R IBNC values for intra-band non-contiguous EN-DC DC_66_n66 are shown in Table 6.4.x.6-1.

Table 6.4.x.6-1: Intra-band non-contiguous EN-DC with one uplink configuration for reference sensitivity

| DC configuration | Aggregated channel bandwidth (LTE+NR) | Wgap / (MHz) | UL LTE allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| DC_66A_n66A | NOTE 4 | NOTE 8 | NOTE 9 | 0 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The table only applies when the center frequency of LTE carrier is higher than the NR carrier, and the ΔRIBNC applies to the NR DL carrier only. NOTE 4: All combinations of channel bandwidths defined in Table 5.3B.1.3-1. NOTE 5: 5 refers to the UL resource blocks shall be located at RBstart=25. NOTE 6: 6 refers to the UL resource blocks shall be located at RBstart=35. NOTE 7: 7 refers to the UL resource blocks shall be located at RBstart=50. NOTE 8: All applicable sub-block gap sizes. NOTE 9: The UL LTE allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1 in [4]. |  |  |  |  |  |

#### 6.4.x.7	MPR, AMPR requirements

Note that only Single Tx Switched UL mode is supported for this combination, no MPR or A-MPR requirement is needed

<End of Text Proposal>