3GPP TSG-RAN WG4 Meeting #92                                                               R4-1908103

Ljubljana, SI, 26th-30th August, 2019

Source:	Samsung, Bell Mobility, TELUS, Nokia

Title:	TP for TR 37.716-31-11: DC_2-7-66_n66 and DC_2-7-7-66_n66

Agenda item:			10.5.2

Document for:	Ericsson

# 1. Introduction

In RAN#84 meeting, the revised WID on EN-DC for 4 bands DL with 2 bands UL (3 LTE band + 1 NR band) was approved [1], in which the latest EN-DC for 4 bands DL with 2 bands UL (3 LTE band + 1 NR band) was attached. This contribution is a text proposal for TR 37.716-31-11 to include DC_2A-7A-66A_n66A, DC_2A-7A-7A-66A_n66A and DC_2A-7C-66A_n66A.

# 2. Reference

- RP-191195, Revised WID on EN-DC for 4bands DL with 2 bands UL (3 LTE band + 1 NR band), Huawei.

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.x	DC_2-7-66_n66, DC_2-7-7-66_n66

### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (four bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-7-66_n66 | CA_2-7-66 | n66 |  |
| DC_2-7-7-66_n66 | CA_2-7-7-66 | n66 |  |

### 5.1.x.2	Configurations for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-7A-66A_n66A DC_2A-7C-66A_n66A DC_2A-7A-7A-66A_n66A | DC_2A_n66A1 DC_7A_n66A DC_66A_n66A2 | CA_2A-7A-66A CA_2A-7C-66A CA_2A-7A-7A-66A | n66A |
| NOTE1: Single UL is allowed NOTE2: Only single switched UL is supported |  |  |  |

### 5.1.x.3	Coexistence study

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back modes. Therefore, no additional studies are needed.

### 5.1.x.4	∆TIB and ∆RIB values

DC_2-7-66_n66 and DC_2-7-7-66_n66 could reuse the TIB,c and RIB,c values of CA_2A-7A-66A-66A  as given in below tables.

Table 5.1.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-66_n66 DC_2-7-7-66_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-66_n66 DC_2-7-7-66_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

### 5.1.x.5	REFSENS requirements

There is no additional requirement for this band combination.

<End of Text Proposal>