3GPP TSG-RAN WG4 Meeting #92    	R4-1908118

Ljubljana, SI, 26th – 30th Aug, 2019

Agenda item:	7.12.2.3.1

Source:	Samsung

Title:	Discussion and simulation results for NR PUCCH with multi-slot

Document for:	Discussion

# 1	Introduction.

During RAN4#91 meeting, the WF on multi-slot NR PUCCH related the open issue on frequency hopping and the simulation assumption for requirement is approved [1].  Based on the agreement, the initial simulation results with ideal and impairment are provided.

# 2	Discussion

As indicated in the simulation assumption, only the requirement with 8 Rx is specified considering the coverage and reducing test effort. As agreed, BS class with BS type 1-H, BS type 1-O and BS-type 2-O are required for radiated requirement testing with OTA test, the requirement with up to 2Rx can be tested, which means the requirement with 8Rx can only be under conducted testing, there is no requirement for OTA test. Considering the BS test with conducted and radiated requirement, at least requirement with 2Rx should be specified. For the test applicability of requirements for different Rx antennas, a BS can declare to support a set of Rx antennas for a specific test case, is only required to test the defined requirements with lowest and highest number of supported Rx antennas.

Proposal 1:  Consider the BS with conducted and radiated test, at least the requirement with 2Rx and 8Rx antennas should be specified for PUCCH with multi-slot.

# 3	Simulation Results

In this subsection, based on the simulation assumption for PUCCH with multi-slot as follows, the ideal and impairment results are provided.

Table 1: Test parameters for PUCCH with multi-slot

| Parameters |  | PUCCH Format 1 |
| --- | --- | --- |
| Bandwidth/SCS |  | 40 MHz/ 30kHz |
| Antenna |  | 1T2R, 1T4R, 1T8R |
| Propagation condition |  | TDLC300-100 |
| Carrier frequency (GHz) |  | 4GHz |
| (#Bits, #Symbols, #PRBs) |  | (2, 14,1) |
| initialCyclicShift |  | 0 |
| StartingSymbolIndex |  | 0 |
| Index of orthogonal sequence (time-domain-OCC) |  | 0 |
| Inter/Intra-slot frequency hopping |  | Enabled inter-slot frequency hopping, disable for intra-slot frequency hopping |
| Number of slots |  | 2 |
| Test metric | DTX to ACK probability <1% | √ |
|  | Missed ACK probability < 1% | √ |
|  | NACK2ACK < 0.1% | √ |

Table 2: Ideal results for PUCCH with multi-slot

| Number of Tx antennas | Number of Rx antennas | Propagation conditions and correlation | Missed ACK probability (<1%) | NACK2ACK probability (<0.1%) |
| --- | --- | --- | --- | --- |
| 1 | 2 | TDLC300-100 Low | -9.4 | -8.6 |
|  | 4 | TDLC300-100 Low | -13.5 | -12.8 |
|  | 8 | TDLC300-100 Low | -16.5 | -16.0 |

Table 3: Impairment results for PUCCH with multi-slot

| Number of Tx antennas | Number of Rx antennas | Propagation conditions and correlation | Missed ACK probability (<1%) | NACK2ACK probability (<0.1%) |
| --- | --- | --- | --- | --- |
| 1 | 2 | TDLC300-100 Low | -7.4 | -6.6 |
|  | 4 | TDLC300-100 Low | -11.5 | -10.8 |
|  | 8 | TDLC300-100 Low | -14.5 | -14.0 |

# 4	Conclusion

In this contribution, the ideal and impairment results of PUCCH with multi-slot are provided.

Proposal 1:  Consider the BS with conducted and radiated test, at least the requirement with 2Rx and 8Rx antennas should be specified for PUCCH with multi-slot.

# References

R4-1907242, WF on multi-slot PUCCH performance requirements, ZTE.