3GPP TSG-RAN WG4 Meeting #92                 	R4-1908129

Ljubljana, Slovenia, 26th – 30th August 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	On NR multi-slot PUCCH demodulation requirements

Agenda item:	7.12.2.3.1

Document for:	Discussion

Introduction

At the end of RAN4#91 an issue was observed for multi-slot PUCCH in the form of 1T8R scenarios defined in OTA testing. This was captured in the RRM chairmen report,

R4-1907267 Draft CR on TS 38.104 Performance requirement for multi-slot PUCCH format 1 38.104 CR- rev Cat: (Rel-15) v15.5.0 Source: ZTE Wistron Telecom Abstract: Discussion: Agreement: 1T8R configuration will be revised in the next meeting. Decision: Endorsed

Furthermore, it was stated in discussion that the applicability rules for multi-slot PUCCH should be rechecked for consistency, after many new rules were added in the last meetings.

In this contribution we supplement our previous simulation results [1] with the 1T2R, inter-slot frequency hopping enabled, and intra-slot frequency hopping disabled case.
Finally, we discuss possible solutions to the 1T8R mis-configuration and our understanding of the applicability rules.

Simulation results

Following the WF of multi-slot PUCCH by ZTE [2], we ran simulations with the following setup:

Table 1: Simulation setup multi-slot PUCCH

| Parameters |  | PUCCH Format 1 |
| --- | --- | --- |
| BandWidth/SCS |  | 40 MHz/ 30kHz |
| Antenna |  | 1T8R, 1T2R |
| Propagation condition |  | TDLC300-100; |
| Carrier frequency (GHz) |  | 4GHz; |
| (#Bits, #Symbols, #PRBs) |  | (2, 14,1) |
| initialCyclicShift |  | 0 |
| StartingSymbolIndex |  | 0 |
| Index of orthogonal sequence (time-domain-OCC) |  | 0 |
| Inter/Intra-slot frequency hopping |  | Option 1: Enabled/Disabled |
|  |  | Option 2: Disabled/Enabled |
| number of slot |  | 2 |
| Test metric | DTX to Ack probability <1% | √ |
|  | Missed Ack probability < 1% | √ |
|  | NACK2ACK < 0.1% | √ |

Which led to the following results:

Table 2: Simulation results multi-slot PUCCH 1T8R

| PUCCH FR1 1T8R, TDLC300-100 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Number of slot | Inter/intra-slot frequency hopping | Test metric | SNR (dB) Ideal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Format 1 | 30kHz, 40MHz | 2 | 14 | 1 | 2 | Enabled/Disabled | ACK miss (1%) | -17,7 |
|  |  |  |  |  |  |  | NACK to ACK (0.1%) | -16,7 |
|  |  |  |  |  |  | Disabled/Enabled | ACK miss (1%) | -16,7 |
|  |  |  |  |  |  |  | NACK to ACK (0.1%) | -15,6 |

Table 3: Simulation results multi-slot PUCCH 1T2R

| PUCCH FR1 1T2R, TDLC300-100 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Number of slot | Inter/intra-slot frequency hopping | Test metric | SNR (dB) Ideal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Format 1 | 30kHz, 40MHz | 2 | 14 | 1 | 2 | Enabled/Disabled | ACK miss (1%) | -10.5 |
|  |  |  |  |  |  |  | NACK to ACK (0.1%) | -8.9 |

In our simulations, DTX to Ack probability is always ensured to be below 1%, hence we omitted the DTX to Ack test metric; similar to how this metric is not part the “standard” PUCCH simulation collection template [3].

Discussion

In order to allow for consistent test cases in conducted and OTA testing, the latter of which is constrained to two demodulation branches, we propose to change the PUCCH multi-slot scenarios from 1T8R to 1T2R for both conducted and OTA minimum performance requirements.

RAN4 to consider changing the PUCCH multi-slot configuration to 1T2R.

The current set of applicability rules relevant for multi-slot PUCCH have been reviewed and no issues have been found.

Conclusion

In this contribution we have provided our views on the remaining NR multi-slot PUCCH demodulation issues. We have made the following proposals:

- RAN4 to consider changing the PUCCH multi-slot configuration to 1T2R.

- References

- R4-1906371, “On NR multi-slot PUCCH demodulation requirements”, Nokia, Nokia Shanghai Bell, RAN4#91.

- R4-1904719, WF on Multi-slot PUCCH, ZTE, RAN4#90b.

- R4-1902830, Summary of ideal and impairment results for NR BS demodulation requirements, China Telecom, RAN4#90b.