3GPP TSG-RAN4  #92																	     	R4-1908155

Ljubljana, Sl, 26-30 Aug 2019

Source: 	Qualcomm Incorporated

Title: 	On FR2 CA MPR table anomalies

Agenda item:	7.5.8.2

Document for:	Approval

# Introduction

The FR2 CA MPR tables in TS38.101-2 v15.6 contain anomalies due to multiple disjoint efforts to reach consensus prior to close of rel. 15. In this contribution, we propose to iron out these anomalies.

# Discussion

Both, PC1 and PC2/3/4 CA MPR tables in TS38.101-2 v15.6 contain anomalies. Anomalous entries are judged based on a few simple high-level observations:

- PAPR of multiple CC CP-OFDM is comparable to that of single CC CP-OFDM [1]

- PAPR of multiple CC DFT-s waveforms in CA mode approaches that of CP-OFDM [1]. Ergo:

  - DFT-s waveforms should not need more MPR than CP-OFDM waveforms

  - MPR should be relatively insensitive to modulation type

- PC2/3/4, on account of lower TRP and lower max PSD, must have less MPR than PC1

- CA MPR must not be smaller than single CC MPR

The sections from the CA MPR tables that contain anomalous entries are reproduced below in table 2.0-1

| Waveform Type |  | Cumulative aggregated channel bandwidth ≥ 800 MHz and ≤ 1400 MHz |  |
| --- | --- | --- | --- |
|  |  | PC1 | PC2/3/4 |
| DFT-s-OFDM | Pi/2 BPSK |  |  |
|  | QPSK | [9.7] | ≤ [8.2] |
|  | 16 QAM | [9.2] | ≤ [9.3] |
|  | 64 QAM |  |  |
| CP-OFDM | QPSK | [8.7] | ≤ [8.0] |
|  | 16 QAM | [8.7] | ≤ [9.2] |
|  | 64 QAM |  |  |

Table 2.0-1: Excerpts from CA MPR tables in TS38.101-2 v15.6 with anomalous entries

We elaborate on the anomalies below:

- PC1

  - MPR for DFT-s waveforms exceeds that of CP-OFDM

  - MPR for DFT-s-QPSK exceeds that of DFT-s-16QAM

- PC3

  - MPR for DFT-s-16QAM exceeds that of CP-OFDM-16QAM

  - MPR for CP-OFDM-16QAM for PC3 exceeds that of PC1

  - Not shown in table 2.0-1, MPR for DFT-s-QPSK exceeds that of CP-OFDM QPSK for CABW between 400 and 799.99MHz (7.7 vs 7.5 dB)

A proposal to change the values to remove the anomalies is below in table 2.0-2. Some of the MPR increases are to ensure monotonicity inside the PC1 and PC3 CAMPR tables. The proposal aims to limit overall movement of MPR.

| Waveform Type |  | Cumulative aggregated channel bandwidth ≥ 800 MHz and ≤ 1400 MHz |  |
| --- | --- | --- | --- |
|  |  | PC1 | PC2/3/4 |
| DFT-s-OFDM | Pi/2 BPSK |  |  |
|  | QPSK | [9.7] 9.2 | ≤ [8.2] |
|  | 16 QAM | [9.2] | ≤ [9.3] 9.2 |
|  | 64 QAM |  |  |
| CP-OFDM | QPSK | [8.7] 9.2 | ≤ [8.0] 8.2 |
|  | 16 QAM | [8.7] 9.2 | ≤ [9.2] |
|  | 64 QAM |  |  |

Table 2.0-2: Proposal to treat anomalous entries in CA MPR tables in TS38.101-2 v15.6

## Visualizing PC1 CA MPR table changes

Table 2.1-1 captures the proposed changes in context of the existing PC1 table for easy visualization

(Table 6.2A.2.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1)

| Waveform Type |  | Cumulative aggregated channel bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.51 | 7.71 | [8.2] |
|  | QPSK | ≤ 6.51 | 8.71 | [9.7] 9.2 |
|  | 16 QAM | ≤ 6.5 | 8.7 | [9.2] |
|  | 64 QAM | ≤ 9.0 | 10.7 | [11.2] |
| CP-OFDM | QPSK | ≤ 6.5 | 8.7 | [8.7] 9.2 |
|  | 16 QAM | ≤ 6.5 | 8.7 | [8.7] 9.2 |
|  | 64 QAM | ≤ 9.0 | 10.7 | [11.2] |
| NOTE 1: The following condition applies only when the cumulative aggregated BW of the CA configuration 400MHz. For a contiguous RB allocation in a single CC of the CA configuration, the single CC MPR of subclause 6.2.2.1 applies. The cumulative aggregated bandwidth shall be used as BWchannel in Tables 6.2.2.1-1 and 6.2.2.1-2. The applicable column in Tables 6.2.2.1-1 and 6.2.2.1-2 shall be determined based on the transmission bandwidth of the CA allocation in relation to the allocation regions defined in the tables. |  |  |  |  |

Table 2.1-1: Proposed PC1 CA MPR table in TS38.101-2

Proposal 1: PC1 CA MPR table entries shall be changed per highlighted changes in table 2.1-1

## Visualizing PC2/3/4 CA MPR table changes

Table 2.2-1 captures the proposed changes in context of the existing PC3 table for easy visualization. Recall that MPRs for PC2 and PC4 are based on PC3 MPRs, so a treatment of PC3 covers PC2 and PC4.

(Table 6.2A.2.4-1: Maximum power reduction (MPRC_CA) for UE power class 3)

|  |  | Cumulative aggregated bandwidth configuration |  |  |
| --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.01 | ≤ 7.71 | ≤ [8.2] |
|  | QPSK | ≤ 5.01 | ≤ 7.71 | ≤ [8.2] |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ [9.3] 9.2 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ [11.2] |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 7.5 7.7 | ≤ [8.0] 8.2 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ [9.2] |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ [11.2] |
| NOTE 1: The following condition applies only when the cumulative aggregated BW of the CA configuration ≤ 400 MHz. For a contiguous RB allocation in a single CC of the CA configuration, the single CC MPR of subclause 6.2.2.1 applies. The cumulative aggregated bandwidth shall be used as BWchannel in Tables 6.2.2.1-1 and 6.2.2.1-2. The applicable column in Tables 6.2.2.1-1 and 6.2.2.1-2 shall be determined based on the transmission bandwidth of the CA allocation in relation to the allocation regions defined in the tables. |  |  |  |  |

Table 2.2-1: Proposed PC3 CA MPR table in TS38.101-2

Proposal 2: PC3 CA MPR table entries shall be changed per highlighted changes in table 2.2-1

# Conclusion

We identified some anomalies in the FR2 CA MPR tables and propose minor adjustments to remove them.

Proposal 1: PC1 CA MPR table entries shall be changed per highlighted changes in table 2.1-1

Proposal 2: PC3 CA MPR table entries shall be changed per highlighted changes in table 2.2-1

The changes are captured in a CR [2]

# Reference

- R4-1908151,’Rel. 15 MPR for FR2 ULCA corner case’, Qualcomm, RAN4#92, Ljubljana, Sl, Aug 2019

- R4-1908160,’CR to 38.101-2: FR2 CA MPR refinement’, Qualcomm, RAN4#92, Ljubljana, Sl, Aug 2019