3GPP TSG-RAN4  #92																	     	R4-1908165

Ljubljana, Sl, 26-30 Aug 2019

Source: 	Qualcomm Incorporated

Title: 	Work Plan for DL CA BW Enhancement for Rel. 16 FR2

Agenda item:	9.13.4

Document for:	Approval

# Introduction

For rel. 16 RAN4 must discuss UE requirements for intra-band DL CA enhanced from rel. 15 [1]. In this contribution, we share our view on how work must progress in RAN4 to enable completion of this feature for rel. 16. We also initiate discussion on how best to implement CA BW enhancement into the standard.

# Discussion

The WID, reproduced in part in section 5, outlines enhancement of the UE’s CA DL BW capability for rel. 16. Practical implementation constraints in pursuing CA BW enhancement, and how it may guide future UE requirements is addressed in a companion contribution [2]. We outline a work plan, followed by discussion on some fundamental features of the proposed enhancement.

## Work Plan

To complete the induction of the enhanced DL coverage capability, three main items must be resolved:

- Definition of the UE’s new CA DL BW enhancement (‘feature’)

- Definition of any signalling relevant to the feature, communication to other WGs

- Definition of any RF requirements relevant to the feature

Typically, signalling is addressed later in the work item’s progression in RAN4. In this case however, there is some complexity [2] in what may need to be conveyed to the network. RAN4 may also need time to whittle down the number of enhancement options, if it is deemed appropriate. These tasks must be fit into a compressed work schedule, with rel. 16 closing for RAN4 at the Feb 2020 meeting. These factors require RAN4 to set up parallel paths of development to ensure both, technical requirements and signalling requirements are addressed in time.

A high-level schedule is presented below:

| Proposed Milestone | RAN4 Meeting |
| --- | --- |
| Discussion and Concept Review | RAN4 #92, Ljubljana |
| Completion of Feature Definition | RAN4 #92Bis, Chongqing |
| Completion of Signalling Details and LS to RAN2 | RAN4 #93, Reno |
| Completion of RF requirements | RAN4 #94, Athens |

Table 2.1-1: Proposed work plan for Enhanced CA DL aggregated BW feature

## CA DL BW enhancement framework (feature definition)

In [2], we streamlined multiple UE architecture possibilities into the following categories by their UL/DL frequency spectrum coverage overlap. (‘Frequency Spectrum Coverage’ here means the range of UL or DL frequencies that can be populated with an UL or DL CC in CA mode)

- common UL+DL coverage, DL-only band on one side of bidirectional coverage

- common UL+DL coverage, DL-only coverage on both sides of bidirectional coverage

- common UL+DL coverage, with no DL-only coverage

This simple classification can form the basis of a framework to define the rel. 16 UE’s enhanced DL CA BW feature.

Figure 2.2-1 summarizes in graphic form, relative UL and DL coverage spectra described above. It shows examples of enhanced DL spectra (yellow) in relation to a UE’s UL spectrum (blue). Spectrum covered by both UL and DL is shown with yellow and blue stripes

DL spectrum examples in relation to UL spectrum (contiguous DL coverage)

UL Spectrum

DL only spectrum

Spectrum common to UL and DL

frequency

Figure 2.2-1: UL and DL spectra for UE with enhanced DL CA BW

From the figure it is evident that a UE’s CA coverage spectrum can be viewed as comprising:

- a ‘main’, bidirectional spectrum that is common to both, UL and DL (blue and yellow stripes), and

- an ‘auxiliary’ DL-only spectrum (solid yellow). This auxiliary DL spectrum can take on several forms depending on implementation choices made by the UE:

  - it may be absent altogether or

  - distributed on any one side or

  - distributed on both sides of the main spectrum.

Observation 1: Rel. 16 UEs with enhanced DL CA BW have two types of frequency spectrum coverage in CA mode:

- A ‘main’ spectrum that is available for UL and DL CCs. The main spectrum looks exactly like a rel.1 5 UE’s UL and DL coverage in CA mode

- An ‘auxiliary’ spectrum that is only available for DL CCs. The auxiliary spectrum does not have an analogue in rel. 15

We can leverage the similarity between the ‘main spectrum’ in CA mode of an enhanced rel. 16 UE and the CA feature in rel. 15 UEs to benefit signalling: Existing CA capability signalling in rel 15. can be maintained in rel. 16 to convey information about the ‘main’ (bi-directional) spectrum.

Proposal 1: For rel. 16 UEs, the existing rel. 15 signalling for CA operation remains applicable to the part of the UE frequency spectrum coverage that is common to both UL and DL.

Unfortunately, the auxiliary band (DL-only CCs allowed) does not have pre-existing constructs in rel. 15 to build on. The auxiliary spectrum must then be conveyed as additional information, with new signalling.

Proposal 2: For rel. 16 UEs, new signalling shall be instituted to convey location and size of additional UE spectrum that is covered by DL only.

Further work is required in RAN4 to resolve the content of the new signalling to accommodate all foreseen practical UE implementations

## RF Requirements in the Standard

Certain RF requirements have exposure to CA DL BW directly or indirectly. These requirements must be amended for consistency, and are listed below:

- CA MPR: Several mechanisms like spurious emissions and gain droop may require additional PA back off.

- REFSENS relaxation: as frequency separation increases, beam patterns may start to diverge from one edge of the frequency band of DL coverage to the other. From any given direction, this beam pattern divergence will manifest itself as gain reduction. There are also de-sense mechanisms that get exacerbated with the wider frequency separation.

# Conclusion

A high-level study of practical UE implementation topologies to achieve enhanced DL CA BW revealed that they all share certain characteristics pertaining to UL and DL spectrum coverage. A UE’s spectrum can be viewed as being comprised of a main, bidirectional spectrum that offers both, UL and DL capability, and an auxiliary DL-only spectrum. This type of classification is convenient because it allows us to build on existing rel. 15 signalling framework:

Proposal 1: For rel. 16 UEs, the existing rel. 15 signalling for CA operation remains applicable to the part of the UE frequency spectrum coverage that is common to both UL and DL.

Unfortunately, the auxiliary band (DL-only CCs allowed) does not have pre-existing constructs in rel. 15 to build on. The auxiliary spectrum must then be conveyed as additional information, with new signalling.

Proposal 2: For rel. 16 UEs, new signalling shall be instituted to convey location and size of additional UE spectrum that is covered by DL only.

New signalling is FFS

# Reference

- RP-190761,” NR RF requirements for FR2”, Nokia, Shenzhen, China, March 18-21, 2019

- R4-1908164, ‘UE Architecture for DL CA BW Enhancement for Rel. 16 FR2’, Qualcomm, RAN4#92, Ljubljana, Sl, Aug 2019

# Appendix

The objectives in the WID [1] covering rel. 16 FR2 RF topics are included below. The item relevant to this contribution has been highlighted: