3GPP TSG-RAN WG4 Meeting #92		R4-1908192

# Ljubljana, SI, 26th– 30th Aug, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.133 on Testcases for Beam failure detection and Link recovery procedures in FR1 for EN-DC (Section A.4.5.5) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2019-08-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test setup not complete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added additional RLM-RS to avoid OOS during test Deleted Editor’s note Corrected SSB configuration for SSB based testcases Updated figures to capture configured RLM, BFD, CBD RS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test set up not complete and OOS might be declared during test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.5.5.1 A.4.5.5.2 A.4.5.5.3 A.4.5.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of Change

### A.4.5.5	Beam Failure Detection and Link recovery procedures

#### A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode

Editor’s note: It is open whether BFD can be based on SSB. This test case will be updated accordingly.

##### A.4.5.5.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving PSCell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.4.5.5.1.1-1, A.4.5.5.1.1-2, A.4.5.5.1.1-3 and A.4.5.5.1.1-4 below. There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.4.5.5.1.1-1 shows the variation of the downlink SNR of the PCell and the SNR of the SSB in set q0 in the active PSCell to emulate SSB based beam failure. Figure A.4.5.5.1.1-1 additionally shows the variation of the downlink SNR of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [2] ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test 2.

Table A.4.5.5.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.4.5.5.1.1-2: General test parameters for FR1 PSCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  |  |  |  |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Test 1 |  |  |
| Active E-UTRA PCell |  |  |  |  |  |  |  |  | Cell 1 |  |  |
| E-UTRA RF Channel Number |  |  |  |  |  |  |  |  | 1 |  |  |
| Active PSCell |  |  |  |  |  |  |  |  | Cell 2 |  |  |
| RF Channel Number |  |  |  |  |  |  |  |  | 2 |  |  |
| Duplex mode |  |  |  |  | Config 1, 4 |  |  |  | FDD |  |  |
|  |  |  |  | Config 2, 3, 5, 6 |  |  |  |  | TDD |  |  |
| BWchannel |  |  |  |  | Config 1, 4 |  |  | MHz | 10: NRB,c = 52 |  |  |
|  |  |  |  |  | Config 2, 5 |  |  |  | 10: NRB,c = 52 |  |  |
|  |  |  |  |  | Config 3, 6 |  |  |  | 40: NRB,c = 106 |  |  |
| DL initial BWP configuration |  |  |  |  | Config 1, 2, 3, 4, 5, 6 |  |  |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  |  |  |  | Config 1, 2, 3, 4, 5, 6 |  |  |  | DLBWP.1.1 |  |  |
| UL initial BWP configuration |  |  |  |  | Config 1, 2, 3, 4, 5, 6 |  |  |  | ULBWP.0.1 |  |  |
| UL dedicated BWP configuration |  |  |  |  | Config 1, 2, 3, 4, 5, 6 |  |  |  | ULBWP.1.1 |  |  |
| TDD Configuration |  |  |  |  | Config 1, 4 |  |  |  | Not Applicable |  |  |
|  |  |  |  | Config 2, 5 |  |  |  |  | TDDConf.1.1 |  |  |
|  |  |  |  | Config 3, 6 |  |  |  |  | TDDConf.1.2 |  |  |
| CORESET Reference Channel |  |  |  |  | Config 1, 4 |  |  |  | CR. 1.1 FDD |  |  |
|  |  |  |  | Config 2, 5 |  |  |  |  | CR. 1.1 TDD |  |  |
|  |  |  |  | Config 3, 6 |  |  |  |  | CR. 2.1 TDD |  |  |
| SSB Configuration |  |  |  |  | Config 1, 4 |  |  |  | SSB.3 FR1 |  |  |
|  |  |  |  | Config 2, 5 |  |  |  |  | SSB.3 FR1 |  |  |
|  |  |  |  | Config 3, 6 |  |  |  |  | SSB.4 FR1 |  |  |
| SMTC Configuration |  |  |  |  | Config 1, 2, 4, 5 |  |  |  | SMTC.1 |  |  |
|  |  |  |  | Config 3, 6 |  |  |  |  | SMTC.1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  |  | Config 1, 2, 4, 5 |  |  |  | 15 KHz |  |  |
|  |  |  |  | Config 3, 6 |  |  |  |  | 30 KHz |  |  |
| PRACH Configuration |  |  |  |  | Config 1, 2, 4, 5 |  |  |  | Table A.3.8.2.4-1 |  |  |
|  |  |  |  | Config 3, 6 |  |  |  |  | Table A.3.8.2.4-1 |  |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  |  |  |  |  | 0 |  |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  |  |  |  |  | 1 |  |  |
| CSI-RS-Index assigned as RLM RS |  |  |  |  |  |  | 0 |  |  |  |  |
| OCNG parameters |  |  |  |  |  |  |  |  | OP.1 |  |  |
| CP length |  |  |  |  |  |  |  |  | Normal |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  |  |  |  | 2x2 Low |  |  |
| Beam failure detection transmission parameters |  | DCI format |  |  |  |  |  |  | 1-0 |  |  |
|  | Number of Control OFDM symbols |  |  |  |  |  |  |  | 2 |  |  |
|  | Aggregation level |  |  |  |  |  |  | CCE | 8 |  |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  |  |  |  |  | dB | 0 |  |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  |  |  |  |  | dB | 0 |  |  |
|  | DMRS precoder granularity |  |  |  |  |  |  |  | REG bundle size |  |  |
|  | REG bundle size |  |  |  |  |  |  |  | 6 |  |  |
| DRX |  |  |  |  |  |  |  |  | OFF |  |  |
| Gap pattern ID |  |  |  |  |  |  |  |  | gp0 |  |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  |  |  |  | absent |  | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  |  |  |  |  |  |  | dBm | [-96] |  | Threshold used for Qout_LR_SSB |
| powerControlOffsetSS |  |  |  |  |  |  |  |  | db0 |  | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  |  |  |  | n2 |  | see TS 38.321 [7], section 5.17 |
| beamFailureDetectionTimer |  |  |  |  |  |  |  |  | pbfd4 |  | see TS 38.321 [7], section 5.17 |
| CSI-RS configuration |  |  | Config 1, 4 |  |  |  |  |  | [CSI-RS.1.3 FDD] |  |  |
|  |  |  | Config 2, 5 |  |  |  |  |  | [CSI-RS.1.3 TDD] |  |  |
|  |  |  | Config 3, 6 |  |  |  |  |  | [CSI-RS.2.3 TDD] |  |  |
| CSI-RS for tracking |  |  | Config 1, 4 |  |  |  |  |  | [TRS.1.1 FDD] |  |  |
|  |  |  | Config 2, 5 |  |  |  |  |  | [TRS.1.1 TDD] |  |  |
|  |  |  | Config 3, 6 |  |  |  |  |  | [TRS.1.2 TDD] |  |  |
| T1 |  |  |  |  |  |  |  | s | 1 |  | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  |  |  |  | s | 0.4 |  |  |
| T3 |  |  |  |  |  |  |  | s | [0.6] |  |  |
| T4 |  |  |  |  |  |  |  | s | [0.4] |  |  |
| T5 |  |  |  |  |  |  |  | s | [1.4] |  |  |
| D1 |  |  |  |  |  |  |  | s | [0.44] |  |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

Table A.4.5.5.1.1-3: Cell specific test parameters for FR1 PSCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |  | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SSB of set q0 |  |  |  |  |  | SSB of set q1 |  |  |  |  |
|  |  |  | T1 | T2 | T3 |  | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |  |
| SNR | Config 1 | dB | [5] | [-3] | [-12] | [-12] |  | [-12] | [-12] | [-12] | [-12] | [-3] | [7] |
|  | Config 2 |  | [5] | [-3] | [-12] | [-12] |  | [-12] | [-12] | [-12] | [-12] | [-3] | [7] |
|  | Config 3 |  | [5] | [-3] | [-12] | [-12] |  | [-12] | [-12] | [-12] | [-12] | [-3] | [7] |
| SNR_RLM-RS | Config 1 | dB | [7] |  |  |  |  |  | [7] |  |  |  |  |
|  | Config 2 |  | [7] |  |  |  |  |  | [7] |  |  |  |  |
|  | Config 3 |  | [7] |  |  |  |  |  | [7] |  |  |  |  |
|  | Config 1 | dBm/15KHz | -98 |  |  |  |  |  | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDLC300-100 |  |  |  |  |  | TDLC300-100 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is [A.3.6]. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.5.5.1.1-4: Measurement gap configuration for FR1 PSCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Field | Test 2 |
| --- | --- |
|  | Value |
| gapOffset | 0 |

Figure A.4.5.5.1.1-1: SNR variation SSB for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.4.5.5.1.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = [TBD] ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode

Editor’s note: It is open whether BFD can be based on SSB. This test case will be updated accordingly.

##### A.4.5.5.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving PSCell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.4.5.5.2.1-1, A.4.5.5.2.1-2, A.4.5.5.2.1-3, A.4.5.5.2.1-4 and A.4.5.5.2.1-5 below. There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.4.5.5.2.1-1 shows the variation of the downlink SNR of the PCell and the SNR of the SSB in set q0 in the active PSCell to emulate SSB based beam failure. Figure A.4.5.5.2.1-1 additionally shows the variation of the downlink SNR of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [2] ms. In the test, DRX configuration is enabled in PSCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test 2.

Table A.4.5.5.2.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.4.5.5.2.1-2: General test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  |  |  |  |  | Unit |  | Value |  | Comment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Test 1 |  |  |  |
| Active E-UTRA PCell |  |  |  |  |  |  |  |  | Cell 1 |  |  |  |  |
| E-UTRA RF Channel Number |  |  |  |  |  |  |  |  | 1 |  |  |  |  |
| Active PSCell |  |  |  |  |  |  |  |  | Cell 2 |  |  |  |  |
| RF Channel Number |  |  |  |  |  |  |  |  | 2 |  |  |  |  |
| Duplex mode |  |  | Config 1, 4 |  |  |  |  |  | FDD |  |  |  |  |
|  |  |  |  | Config 2, 3, 5, 6 |  |  |  |  |  | TDD |  |  |  |
| BWchannel |  |  | Config 1, 4 |  |  |  | MHz |  | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 2, 5 |  |  |  |  |  | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3, 6 |  |  |  |  |  | 40: NRB,c = 106 |  |  |  |  |
| DL initial BWP configuration |  |  | Config 1, 2, 3, 4, 5, 6 |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3, 4, 5, 6 |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |
| UL initial BWP configuration |  |  | Config 1, 2, 3, 4, 5, 6 |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3, 4, 5, 6 |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |
| TDD Configuration |  |  | Config 1, 4 |  |  |  |  |  | Not Applicable |  |  |  |  |
|  |  |  |  | Config 2, 5 |  |  |  |  |  | TDDConf.1.1 |  |  |  |
|  |  |  |  | Config 3, 6 |  |  |  |  |  | TDDConf.1.2 |  |  |  |
| CORESET Reference Channel |  |  | Config 1, 4 |  |  |  |  |  | CR. 1.1 FDD |  |  |  |  |
|  |  |  |  | Config 2, 5 |  |  |  |  |  | CR. 1.1 TDD |  |  |  |
|  |  |  |  | Config 3, 6 |  |  |  |  |  | CR. 2.1 TDD |  |  |  |
| SSB Configuration |  |  | Config 1, 4 |  |  |  |  |  | SSB.3 FR1 |  |  |  |  |
|  |  |  |  | Config 2, 5 |  |  |  |  |  | SSB.3 FR1 |  |  |  |
|  |  |  |  | Config 3, 6 |  |  |  |  |  | SSB.4 FR1 |  |  |  |
| SMTC Configuration |  |  | Config 1, 2, 4, 5 |  |  |  |  |  | SMTC.1 |  |  |  |  |
|  |  |  |  | Config 3, 6 |  |  |  |  |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2, 4, 5 |  |  |  |  |  | 15 KHz |  |  |  |  |
|  |  |  |  | Config 3, 6 |  |  |  |  |  | 30 KHz |  |  |  |
| PRACH Configuration |  |  | Config 1, 2, 4, 5 |  |  |  |  |  | Table A.3.8.2.4-1 |  |  |  |  |
|  |  |  |  | Config 3, 6 |  |  |  |  |  | Table A.3.8.2.4-1 |  |  |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  |  |  |  |  | 0 |  |  |  |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  |  |  |  |  | 1 |  |  |  |  |
| CSI-RS-Index assigned as RLM RS |  |  |  |  |  | 0 |  |  |  |  |  |  |  |
| OCNG parameters |  |  |  |  |  |  |  |  | OP.1 |  |  |  |  |
| CP length |  |  |  |  |  |  |  |  | Normal |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  |  |  |  | 2x2 Low |  |  |  |  |
| Beam failure detection transmission parameters | DCI format |  |  |  |  |  |  |  |  | 1-0 |  |  |  |
|  | Number of Control OFDM symbols |  |  |  |  |  |  |  |  | 2 |  |  |  |
|  | Aggregation level |  |  |  |  |  |  | CCE |  | 8 |  |  |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  |  |  |  |  | dB |  | 0 |  |  |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  |  |  |  |  | dB |  | 0 |  |  |  |
|  | DMRS precoder granularity |  |  |  |  |  |  |  |  | REG bundle size |  |  |  |
|  | REG bundle size |  |  |  |  |  |  |  |  | 6 |  |  |  |
| DRX |  |  |  |  |  |  |  |  | DRX.7 |  | A.3.3.7 |  |  |
| Gap pattern ID |  |  |  |  |  |  |  |  | gp0 |  |  |  |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  |  |  |  | absent |  | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |  |  |
| rsrp-ThresholdSSB |  |  |  |  |  |  | dBm |  | [-96] |  | Threshold used for Qout_LR_SSB |  |  |
| powerControlOffsetSS |  |  |  |  |  |  |  |  | db0 |  | Used for deriving rsrp-ThresholdCSI-RS |  |  |
| beamFailureInstanceMaxCount |  |  |  |  |  |  |  |  | n2 |  | see TS 38.321 [7], section 5.17 |  |  |
| beamFailureDetectionTimer |  |  |  |  |  |  |  |  | pbfd4 |  | see TS 38.321 [7], section 5.17 |  |  |
| CSI-RS configuration |  | Config 1, 4 |  |  |  |  |  |  |  | [CSI-RS.1.3 FDD] |  |  |  |
|  |  | Config 2, 5 |  |  |  |  |  |  |  | [CSI-RS.1.3 TDD] |  |  |  |
|  |  | Config 3, 6 |  |  |  |  |  |  |  | [CSI-RS.2.3 TDD] |  |  |  |
| CSI-RS for tracking |  | Config 1, 4 |  |  |  |  |  |  |  | [TRS.1.1 FDD] |  |  |  |
|  |  | Config 2, 5 |  |  |  |  |  |  |  | [TRS.1.1 TDD] |  |  |  |
|  |  | Config 3, 6 |  |  |  |  |  |  |  | [TRS.1.2 TDD] |  |  |  |
| T1 |  |  |  |  |  |  | s |  | 1 |  | During this time the the UE shall be fully synchronized to cell 1 |  |  |
| T2 |  |  |  |  |  |  | s |  | 0.4 |  |  |  |  |
| T3 |  |  |  |  |  |  | s |  | [0.6] |  |  |  |  |
| T4 |  |  |  |  |  |  | s |  | [0.4] |  |  |  |  |
| T5 |  |  |  |  |  |  | s |  | [1.4] |  |  |  |  |
| D1 |  |  |  |  |  |  | s |  | [0.44] |  |  |  |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

Table A.4.5.5.2.1-3: Cell specific test parameters for FR1 PSCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  |  | Unit |  | Test 1 |  |  |  |  |  | Test 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | SSB of set q0 |  |  |  |  |  |  |  | SSB of set q1 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | T1 | T2 | T3 |  | T4 |  | T5 |  | T1 | T2 | T3 | T4 | T5 |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  | dB |  |  |  |  | 0 |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR | Config 1 |  |  |  | dB |  |  |  |  | [5] | [-3] | [-12] | [-12] |  |  | [-12] |  | [-12] | [-12] | [-12] | [-3] | [7] |  |
|  | Config 2 |  |  |  |  |  |  |  |  | [5] | [-3] | [-12] | [-12] |  |  | [-12] |  | [-12] | [-12] | [-12] | [-3] | [7] |  |
|  | Config 3 |  |  |  |  |  |  |  |  | [5] | [-3] | [-12] | [-12] |  |  | [-12] |  | [-12] | [-12] | [-12] | [-3] | [7] |  |
| SNR_RLM-RS |  | Config 1 | dB |  |  |  |  | [7] |  |  |  |  |  |  |  |  | [7] |  |  |  |  |  |
|  |  | Config 2 |  |  |  |  |  | [7] |  |  |  |  |  |  |  |  | [7] |  |  |  |  |  |
|  |  | Config 3 |  |  |  |  |  | [7] |  |  |  |  |  |  |  |  | [7] |  |  |  |  |  |
|  | Config 1 |  |  |  | dBm/15KHz |  |  |  |  | -98 |  |  |  |  |  |  |  | -98 |  |  |  |  |  |  |
|  | Config 2 |  |  |  |  |  |  |  |  | -98 |  |  |  |  |  |  |  | -98 |  |  |  |  |  |  |
|  | Config 3 |  |  |  |  |  |  |  |  | -98 |  |  |  |  |  |  |  | -98 |  |  |  |  |  |  |
| Propagation condition |  |  |  |  |  |  |  |  |  | TDLC300-100 |  |  |  |  |  |  |  | TDLC300-100 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is [A.3.6]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.5.5.2.1-4: Measurement gap configuration for FR1 PSCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Field | Test 2 |
| --- | --- |
|  | Value |
| gapOffset | [0] |

Table A.4.5.5.2.1-5: DRX-Configuration for FR1 PSCell for SSB-based beam failure detection and link recovery testing in DRX mode.

| Field | Test 5 | Test 6 |
| --- | --- | --- |
|  | Value | Value |
| drx-onDurationTimer | [ms6] | [ms6] |
| drx-InactivityTimer | [ms1] | [ms1] |
| drx-RetransmissionTimerDL | [sl1] | [sl1] |
| drx-RetransmissionTimerUL | [sl1] | [sl1] |
| longDRX-CycleStartOffset | [ms640] | [ms40] |
| shortDRX | disable | disable |

Figure A.4.5.5.2.1-1: SNR variation for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.4.5.5.2.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = [TBD] ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode

##### A.4.5.5.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving PSCell and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.4.5.5.3.1-1, A.4.5.5.3.1-2, and A.4.5.5.3.1-3 below. There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.4.5.5.3.1-1 shows the variation of the downlink SNR of the PSCell and the SNR of the CSI-RS in set q0 in the active PSCell to emulate CSI-RS based beam failure. Figure A.4.5.5.3.1-1 additionally shows the variation of the downlink SNR of the CSI-RS in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [2] ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurementswithout gaps.

Table A.4.5.5.3.1-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.4.5.5.3.1-2: General test parameters for FR1 PSCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  | Unit |  | Value |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  |  |  |
| Active PCell |  |  |  |  | Cell 1 |  |  |  |
| RF Channel Number |  |  |  |  | 1 |  |  |  |
| Duplex mode |  | Config 1, 4 |  |  | FDD |  |  |  |
|  |  | Config 2, 3, 5, 6 |  |  | TDD |  |  |  |
| TDD Configuration |  | Config 1, 4 |  |  | Not Applicable |  |  |  |
|  |  | Config 2, 5 |  |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3, 6 |  |  | TDDConf.1.2 |  |  |  |
| CORESET Reference Channel |  | Config 1, 4 |  |  | CR.1.1 FDD |  | A.3.1.2 |  |
|  |  | Config 2, 5 |  |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3, 6 |  |  | CR.2.1 TDD |  |  |  |
| SSB Configuration |  | Config 1, 4 |  |  | SSB.1 FR1 |  | A.3.10 |  |
|  |  | Config 2, 5 |  |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3, 6 |  |  | SSB.2 FR1 |  |  |  |
| SMTC Configuration |  | Config 1, 2, 4, 5 |  |  | SMTC.1 |  | A.3.11 |  |
|  |  | Config 3, 6 |  |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2, 4, 5 |  |  | 15 KHz |  |  |  |
|  |  | Config 3, 6 |  |  | 30 KHz |  |  |  |
| SSB-Index assigned as RLM RS |  |  |  |  |  | 0 |  |  |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  |  |  | [0] |  |  |  |
| OCNG parameters |  |  |  |  | OP.1 |  | A.3.2.1 |  |
| CP length |  |  |  |  | Normal |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |  |  |
| Beam failure detection transmission parameters |  | DCI format |  |  | 1-0 |  |  |  |
|  |  | Number of Control OFDM symbols |  |  | 2 |  |  |  |
|  |  | Aggregation level | CCE |  | 8 |  |  |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB |  | 0 |  |  |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB |  | 0 |  |  |  |
|  |  | DMRS precoder granularity |  |  | REG bundle size |  |  |  |
|  |  | REG bundle size |  |  | 6 |  |  |  |
| DRX |  |  |  |  | OFF |  |  |  |
| Gap pattern ID |  |  |  |  | N.A. |  |  |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  |  |  | 1 |  |  |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | absent |  | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |  |
| rsrp-ThresholdSSB |  |  | dBm |  | [-96] |  | Threshold used for Qin_LR_SSB |  |
| powerControlOffsetSS |  |  |  |  | db0 |  | Used for deriving rsrp-ThresholdCSI-RS |  |
| beamFailureInstanceMaxCount |  |  |  |  | n2 |  | see TS 38.321 [7], section 5.17 |  |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 |  | see TS 38.321 [7], section 5.17 |  |
| CSI-RS configuration | Config 1, 4 |  |  |  | CSI-RS.1.2 FDD |  | A.3.14 |  |
|  | Config 2, 5 |  |  |  | CSI-RS.1.2 TDD |  |  |  |
|  | Config 3, 6 |  |  |  | CSI-RS.2.2 TDD |  |  |  |
| TRS configuration | Config 1, 4 |  |  |  | TRS.1.1 FDD |  |  |  |
|  | Config 2, 5 |  |  |  | TRS.1.1 TDD |  |  |  |
|  | Config 3, 6 |  |  |  | TRS.1.2 TDD |  |  |  |
| T1 |  |  | s |  | 1 |  | During this time the the UE shall be fully synchronized to cell 1 |  |
| T2 |  |  | s |  | 0.4 |  |  |  |
| T3 |  |  | s |  | [0.6] |  |  |  |
| T4 |  |  | s |  | [0.4] |  |  |  |
| T5 |  |  | s |  | [1.4] |  |  |  |
| D1 |  |  | s |  | [0.24] |  |  |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |  |

Table A.4.5.5.3.1-3: Cell specific test parameters for FR1 PSCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  |  | Unit | Test 1 |  |  |  |  |  |  | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | CSI-RS of set q0 |  |  |  |  |  |  |  | CSI-RS of set q1 |  |  |  |  |  |
|  |  |  |  |  |  |  |  | T1 | T2 | T3 | T4 | T5 |  |  | T1 |  | T2 | T3 | T4 | T5 |  |
| EPRE ratio of PSS to SSS |  |  |  | dB |  |  |  | 0 |  |  |  |  |  |  | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR_CSI-RS |  | Config 1, 4 |  |  | dB |  |  |  | [5] | [-3] | [-12] | [-12] | [-12] |  |  | [-12 |  | [-12] | [-12] | [-3] | [7] |  |
|  |  | Config 2, 5 |  |  |  |  |  |  | [5] | [-3] | [-12] | [-12] | [-12] |  |  | [-12 |  | [-12] | [-12] | [-3] | [7] |  |
|  |  | Config 3, 6 |  |  |  |  |  |  | [5] | [-3] | [-12] | [-12] | [-12] |  |  | [-12 |  | [-12] | [-12] | [-3] | [7] |  |
| SNR_RLM-RS |  | Config 1 |  | dB |  |  |  | [7] |  |  |  |  |  |  |  |  | [7] |  |  |  |  |
|  |  | Config 2 |  |  |  |  |  | [7] |  |  |  |  |  |  |  |  | [7] |  |  |  |  |
|  |  | Config 3 |  |  |  |  |  | [7] |  |  |  |  |  |  |  |  | [7] |  |  |  |  |
|  |  | Config 1, 4 |  |  | dBm/15KHz |  |  |  | -98 |  |  |  |  |  |  | -98 |  |  |  |  |  |  |
|  |  | Config 2, 5 |  |  |  |  |  |  | -98 |  |  |  |  |  |  | -98 |  |  |  |  |  |  |
|  |  | Config 3, 6 |  |  |  |  |  |  | -98 |  |  |  |  |  |  | -98 |  |  |  |  |  |  |
| Propagation condition |  |  |  |  |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure A.4.5.5.3.1-1: SNR variation for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

##### A.4.5.5.3.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = [TBD] ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode

##### A.4.5.5.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving PSCell and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.4.5.5.4.1-1, A.4.5.5.4.1-2, A.4.5.5.4.1-3, and A.4.5.5.4.1-4 below. There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell, in the test.  The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.4.5.5.4.1-1 shows the variation of the downlink SNR of the PSCell and the SNR of the CSI-RS in set q0 in the active PSCell to emulate CSI-RS based beam failure. Figure A.4.5.5.4.1-1 additionally shows the variation of the downlink SNR of the CSI-RS in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [2] ms. In the test, DRX configuration is enabled in PSCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms).

Table A.4.5.5.4.1-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.4.5.5.4.1-2: General test parameters for FR1 PSCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  |  | Unit |  | Value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  |  |  | Cell 1 |  |  |
| RF Channel Number |  |  |  |  |  | 1 |  |  |
| Duplex mode |  | Config 1, 4 |  |  |  | FDD |  |  |
|  |  | Config 2, 3, 5, 6 |  |  |  | TDD |  |  |
| TDD Configuration |  | Config 1, 4 |  |  |  | Not Applicable |  |  |
|  |  | Config 2, 5 |  |  |  | TDDConf.1.1 |  |  |
|  |  | Config 3, 6 |  |  |  | TDDConf.1.2 |  |  |
| CORESET Reference Channel |  | Config 1, 4 |  |  |  | CR.1.1 FDD |  | A.3.1.2 |
|  |  | Config 2, 5 |  | CR.1.1 TDD |  |
|  |  | Config 3, 6 |  | CR.2.1 TDD |  |
| SSB Configuration |  | Config 1, 4 |  |  |  | SSB.1 FR1 |  | A.3.10 |
|  |  | Config 2, 5 |  | SSB.1 FR1 |  |
|  |  | Config 3, 6 |  | SSB.2 FR1 |  |
| SMTC Configuration |  | Config 1, 2, 4, 5 |  |  |  | SMTC.1 |  | A.3.11 |
|  |  | Config 3, 6 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2, 4, 5 |  |  |  | 15 KHz |  |  |
|  |  | Config 3, 6 |  |  |  | 30 KHz |  |  |
| SSB-Index assigned as RLM RS |  |  |  |  | 0 |  |  |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  |  |  |  | [0] |  |  |
| OCNG parameters |  |  |  |  |  | OP.1 |  | A.3.2.1 |
| CP length |  |  |  |  |  | Normal |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  | 2x2 Low |  |  |
| Beam failure detection transmission parameters |  | DCI format |  |  |  | 1-0 |  |  |
|  |  | Number of Control OFDM symbols |  |  |  | 2 |  |  |
|  |  | Aggregation level |  | CCE |  | 8 |  |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB |  | 0 |  |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB |  | 0 |  |  |
|  |  | DMRS precoder granularity |  |  |  | REG bundle size |  |  |
|  |  | REG bundle size |  |  |  | 6 |  |  |
| DRX |  |  |  |  |  | DRX.7 |  | A.3.3.7 |
| Gap pattern ID |  |  |  |  |  | *[gp0] |  |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  |  |  |  | 1 |  |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent |  | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  |  |  | dBm |  | [-96] |  | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  |  |  | db0 |  | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | [n2] |  | see TS 38.321 [7], section 5.17 |
| beamFailureDetectionTimer |  |  |  |  |  | [pbfd4] |  | see TS 38.321 [7], section 5.17 |
| CSI-RS configuration | Config 1, 4 |  |  |  |  | CSI-RS.1.2 FDD |  | A.3.14 |
|  | Config 2, 5 |  |  | CSI-RS.1.2 TDD |  |
|  | Config 3, 6 |  |  | CSI-RS.2.2 TDD |  |
| TRS configuration | Config 1, 4 |  |  | TRS.1.1 FDD |  |
|  | Config 2, 5 |  |  | TRS.1.1 TDD |  |
|  | Config 3, 6 |  |  | TRS.1.2 TDD |  |
| T1 |  |  |  | s |  | 1 |  | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s |  | 0.4 |  |  |
| T3 |  |  |  | s |  | [0.6] |  |  |
| T4 |  |  |  | s |  | [0.4] |  |  |
| T5 |  |  |  | s |  | [1.4] |  |  |
| D1 |  |  |  | s |  | [0.44] |  |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |  |

Table A.4.5.5.4.1-3: Cell specific test parameters for FR1 PSCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  |  |  | Unit | Test 1 |  | Test 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | CSI-RS of set q0 |  |  |  |  |  |  |  |  |  |  | CSI-RS of set q1 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | T1 | T2 | T3 | T4 | T5 |  |  |  | T1 |  |  |  | T2 | T3 | T4 | T5 |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB |  |  |  |  |  | 0 |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR_CSI-RS | Config 1,4 |  |  | dB |  |  |  |  |  | [5] | [-3] | [-12] | [-12] | [-12] |  |  |  | [-12 |  |  |  | [-12] | [-12] | [-3] | [7] |  |  |
|  | Config 2,5 |  |  |  |  |  |  |  |  | [5] | [-3] | [-12] | [-12] | [-12] |  |  |  | [-12 |  |  |  | [-12] | [-12] | [-3] | [7] |  |  |
|  | Config 3,6 |  |  |  |  |  |  |  |  | [5] | [-3] | [-12] | [-12] | [-12] |  |  |  | [-12 |  |  |  | [-12] | [-12] | [-3] | [7] |  |  |
| SNR_RLM-RS |  | Config 1 |  | dB |  |  | [7] |  |  |  |  |  |  |  |  | [7] |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  |  |  |  |  |  | [7] |  |  |  |  |  |  |  |  | [7] |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  |  |  |  |  | [7] |  |  |  |  |  |  |  | [7] |  |  |  |  |  |  |  |  |  |
|  | Config 1,4 |  |  | dBm/15KHz |  |  |  |  |  | -98 |  |  |  |  |  |  |  | -98 |  |  |  |  |  |  |  |  |  |
|  | Config 2,5 |  |  |  |  |  |  |  |  | -98 |  |  |  |  |  |  |  | -98 |  |  |  |  |  |  |  |  |  |
|  | Config 3,6 |  |  |  |  |  |  |  |  | -98 |  |  |  |  |  |  |  | -98 |  |  |  |  |  |  |  |  |  |
| Propagation condition |  |  |  |  |  |  |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.5.5.4.1-4: Measurement gap configuration for FR1 PSCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | [0] |

Table A.4.5.5.4.1-5: Void

Table A.4.5.5.4.1-6: Void

Figure A.4.5.5.4.1-1: SNR variation for CSI-RS-based beam failure detection and link recovery testing in DRX mode

##### A.4.5.5.4.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = [TBD] ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

End of Change