3GPP TSG-RAN WG4#92 Meeting	R4-1908205

Ljubljana, Slovenia, 26 – 30 August 2019

Agenda item:	8.14.3.2

Title: 	Simulation results for LTE HST bidirectional SFN scenario

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In the previous RAN4 meeting a scenario with bi-directional channel model was prioritized to define specific requirements for Rel-16 LTE HST [1].

- Introduce TM3 test HST-SFN bi-directional scenario - Agreed assumptions: maximum Doppler shift 972Hz, MCS 13, 2Rx - Interested companies can provide simulation results for 4Rx, whether to introduce 4Rx test is FFS - If needed other requirements are not precluded. If needed, signaling impact should be discussed in RAN2.

To align simulation results between interested companies a WF on the HST UE demodulation simulation assumptions was agreed [2]. In this contribution we provide simulation results for the above test case for 2 Rx and 4 Rx antennas.

# Simulation results

## Simulation assumptions

The key simulation parameters which were used for evaluations are summarized in Table 1.

Table 1. Simulation assumptions

| Parameter | Value |
| --- | --- |
| Channel model | 4-path bidirectional HST SFN with max Doppler frequency 972 Hz |
| Antenna configuration | 2x2 and 2x4 |
| Channel bandwidth | 10 MHz |
| Duplex mode | FDD and TDD |
| Transmission mode | TM3 |
| EVM | 6 % |
| PDSCH Allocation | 50 RBs |
| MCS | 13, Rank 2 |
| Allocated PDCCH symbols | 2 |
| Special subframe configuration | 4 |

## Simulation results

In Figure 1 we provide evaluation results for 2 Rx and 4 Rx antennas. In the Table 2 the summary of the simulation results is presented.

| Figure 1. Simulation results for HST scenarios |  |
| --- | --- |

Table 2. Summary of simulation results for HST scenarios, SNR @ 70% of max throughput, [dB]

|  | Alignment results |  | Impairment results |  |
| --- | --- | --- | --- | --- |
|  | 2 Rx | 4 Rx | 2 Rx | 4Rx |
| FDD | 8.5 | 5.6 | 10.5 | 7.6 |
| TDD | 9.6 | 6.7 | 11.6 | 8.7 |

# Conclusion

In this contribution we provided simulation results for HST-SFN bidirectional test case and our view on introduction

# Reference

- RP-1907745 “WF on UE demodulation enhancement for HST”, Qualcomm, RAN #91, May 2019

- R4-1907746 “WF on LTE HST UE demodulation simulation assumptions”, Intel Corporation, RAN #91, May 2019