3GPP TSG-RAN WG4 Meeting #92	R4-1908288

Ljubljana, Slovenia, 26th – 30th Aug 2019

Source: 	Nokia

Title: 					TP to TR 37.716-11-11: DC_2A_n261J and DC_2A_n261K

Agenda Item: 			10.3.3

Document for:	Approval

# 1	Introduction

This contribution adds missing fallbacks DC_2A_n261J and DC_2A_n261K into the TR 37.716-11-11. Reason that these configurations are missing is that they were erroneously left out from WID. Intention is to add these configurations into the WID in next RAN.

************************** Start of the TP *****************************************

### 6.2.2	DC_2_n261

#### 6.2.2.1	Operating bands for DC

Table 6.2.2.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2_n261 | 2 | n261 | No |

#### 6.2.2.2	Configuration for DC

Table 6.2.2.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A_n261A | DC_2A_n260A | 2A | n261A |
| DC_2A_n261(2A) | DC_2A_n260A | 2A | n261(2A) |
| DC_2A_n261(4A) | DC _2A_n261A | 2A | n261(4A) |
| DC_2A_n261G | DC_2A_n261G | 2A | n261G |
| DC_2A_n261H | DC _2A_n261H | 2A | n261H |
| DC_2A_n261H | DC _2A_n261G | 2A | n261H |
| DC_2A_n261I | DC _2A_n261I | 2A | n261I |
| DC_2A_n261I | DC_2A_n261H | 2A | n261I |
| DC_2A_n261I | DC_2A_n261G | 2A | n261I |
| DC_2A_n261J | DC_2A_n261A | 2A | n261J |
| DC_2A_n261J | DC_2A_n261G | 2A | n261J |
| DC_2A_n261J | DC _2A_n261H | 2A | n261J |
| DC_2A_n261J | DC _2A_n261I | 2A | n261J |
| DC_2A_n261K | DC_2A_n261A | 2A | n261K |
| DC_2A_n261K | DC_2A_n261G | 2A | n261K |
| DC_2A_n261K | DC _2A_n261H | 2A | n261K |
| DC_2A_n261K | DC _2A_n261I | 2A | n261K |
| DC_2A_n261M | DC _2A_n261A | 2A | n261M |
| DC_2A_n261M | DC _2A_n261I | 2A | n261M |
| DC_2A_n261M | DC _2A_n261H | 2A | n261M |
| DC_2A_n261M | DC_2A_n261G | 2A | n261M |
| DC_2A_n261(A-H) | DC _2A_n261A | 2A | n261(A-H) |
| DC_2A_n261(A-H) | DC _2A_n261H | 2A | n261(A-H) |
| DC_2A_n261(A-H) | DC _2A_n261G | 2A | n261(A-H) |
| DC_2A_n261(G-H) | DC _2A_n261A | 2A | n261(G-H) |
| DC_2A_n261(G-H) | DC _2A_n261H | 2A | n261(G-H) |
| DC_2A_n261(G-H) | DC _2A_n261G | 2A | n261(G-H) |
| DC_2A_n261(2H) | DC_2A_n261H | 2A | n261(2H) |
| DC_2A_n261(2H) | DC_2A_n261G | 2A | n261(2H) |
| DC_2A_n261(A-G-H) | DC_2A_n261A | 2A | n261(A-G-H) |
| DC_2A_n261(A-G-H) | DC _2A_n261H | 2A | n261(A-G-H) |
| DC_2A_n261(A-G-H) | DC _2A_n261G | 2A | n261(A-G-H) |
| DC_2A_n261(A-G-I) | DC _2A_n261A | 2A | n261(A-G-I) |
| DC_2A_n261(A-G-I) | DC _2A_n261I | 2A | n261(A-G-I) |
| DC_2A_n261(A-G-I) | DC _2A_n261H | 2A | n261(A-G-I) |
| DC_2A_n261(A-G-I) | DC _2A_n261G | 2A | n261(A-G-I) |
| DC_2A_n261(H-I) | DC _2A_n261A | 2A | n261(H-I) |
| DC_2A_n261(H-I) | DC _2A_n261I | 2A | n261(H-I) |
| DC_2A_n261(H-I) | DC _2A_n261H | 2A | n261(H-I) |
| DC_2A_n261(H-I) | DC _2A_n261G | 2A | n261(H-I) |

#### 6.2.3.3	Spurious emission band UE co-existence for DC

Co-existence was studied for lower order combinations DC_2_n261in Rel-16, and the results are captured in 37.716-11-11. No further studies are needed for this combination.

#### 6.2.3.4	MSD analysis for DC

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

#### 6.2.3.5	∆TIB and ∆RIB values

TIB,c and RIB,c was studied for lower order combinations DC_2_n261 in Rel-16, and the results are captured in 37.716-11-11. No further studies are needed for this combination.

************************** End of the TP *****************************************