3GPP TSG-RAN WG4 Meeting #92	R4-1908291

Ljubljana, Slovenia, 26th – 30th Aug 2019

Source: 	Nokia

Title: 					TP to TR 37.716-21-21: DC_66_n41-n261

Agenda Item: 			10.8.3

Document for:	Approval

# 1	Introduction

This contribution is a TP to introduce following EN-DC configurations into TR 37.716-21-21

| EN-DC configuration | Uplink EN-DC Configuration | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n41A-n261A | DC_66A_n41A | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | new | (ongoing) DL_66A_n41A_UL_66A_n41A |
| DC_66A_n41A-n261(2A) | DC_66A_n41A | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | new | (new) DL_66A_n41A-n261A_UL_66A_n41A |

******************************** Start of TP ******************************************

## 6.2	DC_66A_n41A-n261A

### 6.2.1	Operating bands for DC

Table 6.2.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_n41A-n261A | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz |  |

### 6.2.2	Channel bandwidths per operating band for DC

Table 6.2.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | Uplink Configurations | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_66A_n41A-n261A | DC_66A_n41A | This part can be omitted unless there is some particular reason. |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n41A-n261(2A) | DC_66A_n41A | See CA_ n261(2A) BCS0 in 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |

### 6.2.3	Co-existence studies

Co-existence studies have been done for DC_66_n41 and are captured into TR 37.716-11-11.

### 6.2.4	∆TIB and ∆RIB values

For DC_66_n41-n261, the TIB,c and RIB,c values are same as for DC_66_n41 which is already specified in TS 38.101-3. n261 TIB,c and RIB,c are 0 dB.

Table 6.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n41-n261 | 66 | 0.5 |
|  | n41 | [0.81] |
|  |  | [1.32] |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.2.4-2: ΔRIB

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66_n41-261 | 66 | 0.5 |
|  | n41 | [0.51] |
|  |  | [12] |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

### 6.2.5	MSD

Based on the studies made for DC_66_n41 MSD due to close proximity is needed and is already specified in TS 38.101-3.

******************************** End of TP ******************************************