3GPP TSG-RAN WG4 Meeting #92	R4-1908295

Ljubljana, Slovenia,  26th – 30th Aug 2019

Source: 	Nokia

Title: 					PC2 for bands 31 and 72 Emission study

Agenda Item: 			8.10.2

Document for:	Discussion

# 1	Introduction

This contribution presents simulation results for PC2 operation on bands 31 and 72 which have already been shared in previous RAN4 meeting.

# 2	Discussions

As was mentioned in [1] UE to UE co-exitance and DTV band protection needs special attention and emission studies.

Figure 1: Frequency arrangement for 450 MHz bands and DTT band

UE to UE co-existence

Band 31 uplink only needs to protect band 31 downlink and not band 72 downlink where are band 72 uplink needs to protect both band 31 and 72 downlinks. This means that the problematic case 31 UL--> 72 DL from duplex-filter point of view is not defined as a requirement.

Table 1: UE to UE co-ex

| 31 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 40, 42, 43, 65, 67, 68, 69 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 72 | E-UTRA Band 1, 7, 20, 22, 28, 31, 32, 33, 34, 38, 42, 43, 47, 52, 65, 68, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 8, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |

Hence we can just study the maximum emissions from 5 MHz distance from uplink band edge to determine the UE to UE co-ex requirement. From figure 1 we can see that emissions are -27.3 dBm at max when the channel is fully allocated which is the worst case. When we apply the minimum Tx to antenna attenuation of roughly 40 dB [2] we can see that – 50 dBm protection level is easily achieved.

Figure 1: Emissions at 5 MHz offset.

As a conclusion

Observation 1: Current Band 31 and 72 PC3 UE to UE co-ex requirement can be applied to PC2.

DTV protection:

Digital television protection starts from 4701 MHz upwards and is -42dBm/8MHz. From Figure 2 we can see that emissions are at most – 45 dBm/1MHz which equals -36 dBm/8MHz. When we take into account filter rejection it is obvious that the requirements is met.

Observation 2: DTV protection is inherently met

Figure 2: Emissions at 470 MHz.

# 3	Conclusion

In this paper we have presented emission simulation results for PC2 operation for bands 31 and 72. We have made following observations

Observation 1: Current Band 31 and 72 PC3 UE to UE co-ex requirement can be applied to PC2.

Observation 2: DTV protection is inherently met

# 4	References

[1] R4-1903528, UE aspects for New WID: Power Class 2 UE for LTE bands 31 and 72, Nokia, RAN4#90bis

[2] https://en.rf360jv.com/inf/40/ds/is/B1220.pdf