3GPP TSG-RAN WG4 Meeting #91	 R4-1908325

# Ljubljana, Slovenia,  26th – 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6552 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NB-IoT RSTD delay test case (Rel 16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh |  |  |  |  |  | Date: |  |  | 2019-08-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Rel-15 mirror CR of the Rel-14 CR in R4-1908324 In TCs A.4.7.1, A.4.7.2, A.4.7.3, A.4.7.4, during T1 the power is relatively low to allow a stable LPP signaling to be completed, although the UE is not yet requested to measure anything. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The power on the NB-IOT cell is increased significantly (SNR from -12 to -2dB) during T1 (and T6), so that the LPP signaling can be completed in a stable way. The power change does not affect the original purpose of measuring NPRS signals, as NPRS is not present during T1 (and T6) . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The very low SNR does not allow a stable completion of the LPP signalling, not allowing unfairly the DUT to execute the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.7.1, A.4.7.2, A.4.7.3, A.4.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.571-1 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage

#### A.4.7.1.1	Test Purpose and Environment

The purpose of the test is to verify that the intra frequency RSTD measurement period for HD-FDD category NB1 UE meets the delay requirements specified in Clause 4.8.2.

In the test there are three synchronous cells: nCell 1, nCell 2 and nCell 3. nCell 1 is the reference cell. nCell 2 and nCell 3 are the neighbour cells. All cells are on the same RF channel.

The test consists of six consecutive time intervals, with duration of T1, T2, T3, T4, T5, and T6. nCell 1 is active throughout T1, T2, T3, T4, T5, and T6, whilst nCell 2 and nCell 3 are activated only in the beginning of T2. nCell 2 is active until the end of T5, and nCell 3 is active until the end of T4. nCell 1 transmits NPRS in T2 and T4, while nCell 2 transmits NPRS in T3 and T5, and nCell 3 transmits NPRS only in T2 and T4. Note: The information on when NPRS is muted is conveyed to the UE using PRS muting information.

Prior to the start of the time duration T1, the UE shall be fully synchronized to nCell1.

At the start of the time duration T1, the OTDOA assistance data and OTDOA-RequestLocationInformation as defined in TS 36.355 [24], shall be provided to the UE. The duration of T1 is sufficiently long to deliver the OTDOA assistance data and OTDOA-RequestLocationInformation to the UE and is independent of the delay requirements specified in Clause 4.8.2.

After OTDOA assistance data and OTDOA-RequestLocationInformation have been successfully received, the UE is provided with a RRC connection release command. The RRC connection release command shall be received by the UE in the last TTI of interval T1. The UE shall enter RRC_IDLE state within T seconds after the receipt of the RRC connection release, where T = 10s is the maximum delay for NB-IOT UE to perform RRC connection release as define in TS36.331 [2].

The test parameters are given in Tables A.4.7.1.1-1 A.4.7.1.1-2 and A.4.7.1.1-3.

Table A.4.7.1.1-1: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Standalone |  |
| Reference cell |  | nCell 1 | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | nCell 2 and nCell 3 | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| nprsID |  | (nprsID of Cell 1 – nprsID of Cell 2)mod6=1 and (nprsID of Cell 1 – nprsID of Cell 3)mod6=2 | As defined in TS36.366 [24] |
| nprs-period | ms | 1280 | As defined in TS36.355 [24] |
| nprs-startSF |  | sf0 | Subframe offset of the NPRS positioning occasion as defined in TS36.355 [24] |
| Number of consecutive downlink positioning subframes nprs-NumSF |  | 320 | As defined in TS36.355 [24] |
| NPRS muting info |  | nCell 1: ‘11110000’ nCell 2: ‘00001111’ nCell 3: ‘11110000’ | Correponds to nprs-MutingInfoB defined in TS 36.355 [24] |
| PartA Configuration |  | N/A | NPRS is configured based on PartB but not PartA. |
| CP length |  | Normal |  |
| NPRACH Configuration |  | NPRACH.R-1 | Refer to A.3.18 |
| DRX cycle length |  | 1.28 | The value shall be used for all cells in the test. |
| Radio frame receive time offset between the cells at the UE antenna connector | s | nCell 2 to nCell 1: 1 nCell 3 to nCell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | nCell 2: 3 nCell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| T2 | s | 5.12 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 5.12 | The length of the time interval that follows immediately after time interval T2 |
| T4 | s | 5.12 | The length of the time interval that follows immediately after time interval T3 |
| T5 | s | 5.12 | The length of the time interval that follows immediately after time interval T4 |
| T6 | s | ≥ 57 | The length of the time interval that follows immediately after time interval T5 |

Table A.4.7.1.1-2: Cell-specific test parameters during T1 and T6

| Parameter | Unit | nCell 1 |  | nCell 2 |  | nCell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| NB-IoT RF Channel Number |  | 1 |  | 1 |  | 1 |
| NB-IoT Channel Bandwidth (BWchannel) | kHz | 200 |  | 200 |  | 200 |
| OCNG Pattern Note 1 |  | NOP.3 FDD |  | N/A |  | N/A |
| NPDSCH parameters Note 2 |  | R.18 HD-FDD |  | N/A |  | N/A |
| NPDCCH parameters Note 2 |  | R.30 HD-FDD |  | N/A |  | N/A |
| NPBCH_RA | dB | 0 |  | N/A |  | N/A |
| NPBCH_RB |  |  |  |  |  |  |
| NPSS_RA |  |  |  |  |  |  |
| NSSS_RA |  |  |  |  |  |  |
| NPDCCH_RA |  |  |  |  |  |  |
| NPDCCH_RB |  |  |  |  |  |  |
| NPDSCH_RA |  |  |  |  |  |  |
| NPDSCH_RB |  |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -98 |  |  |  |  |
| NPRS | dB | -Infinity |  | -Infinity |  | -Infinity |
|  | dB | -2 |  | -Infinity |  | -Infinity |
| Propagation Condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  |
| Timing offset to nCell 1 | s | N/A | 1 |  | -1 |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |  |

Table A.4.7.1.1-3: Cell-specific test parameters from T2 to T5

| Parameter | Unit | nCell 1 |  | nCell 2 |  |  |  | nCell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 and T4 | T3 and T5 | T2 and T4 |  | T3 and T5 |  | T2 and T4 | T3 and T5 |
| BWchannel | kHz | 200 |  | 200 |  |  |  | 200 |  |
| NB-IoT RF Channel Number |  | 1 |  | 1 |  |  |  | 1 |  |
| OCNG patterns |  | NOP.3 FDD |  | N/A | NOP.3 FDD |  |  | NOP.3 FDD | N/A |
| NPBCH_RA | dB | 0 |  | 0 |  |  |  | 0 | N/A |
| NPBCH_RB |  |  |  |  |  |  |  |  |  |
| NPSS_RA |  |  |  |  |  |  |  |  |  |
| NSSS_RA |  |  |  |  |  |  |  |  |  |
| NPDCCH_RA |  |  |  |  |  |  |  |  |  |
| NPDCCH_RB |  |  |  |  |  |  |  |  |  |
| NPDSCH_RA |  |  |  |  |  |  |  |  |  |
| NPDSCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |  |  |  |  |
| NPRS_RA | dB | -3 | N/A | N/A |  | 0 |  | 0 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 |  | -95 |  | -98 | -95 |
| NPRS | dB | -15 | -Infinity | -Infinity |  | -15 |  | -15 | -Infinity |
| NPRS Note 4 | dB | -15 | -Infinity | -Infinity |  | -15 |  | -15 | -Infinity |
| Io Note 4 | dBm/ 180kHz | -87.14 | -87.12 | -87.14 |  | -87.12 |  | -87.14 | -87.12 |
| NPRP Note 4 | dBm/ 15 kHz | -113 | -Infinity | -Infinity |  | -110 |  | -113 | -Infinity |
| NRSRP Note 4 | dBm/ 15 kHz | -110 | -107 | -113 |  | -110 |  | -113 | -Infinity |
| Note 4 | dB | -12 | -12 | -15 |  | -15 |  | -15 | -Infinity |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  |  |  |  |
| Timing offset to nCell 1 | s | N/A |  | 1 |  |  | -1 |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted NPRS. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If NPRS_RA is not “N/A”, , NPRS , Io, NRSRP and NPRP levels have been derived from other parameters and are given for information purpose. If NPRS_RA is “N/A”, Io and NRSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |  |  |

#### A.4.7.1.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 4.8.2.

The UE shall perform and report the RSTD measurements for nCell 2 and nCell 3 with respect to the reference cell in the OTDOA assistance data, nCell 1, within TRSTD _intra_NB-IoT-EC +TRandomAccess_NB-IoT-EC = 67.16 s starting from the beginning of time interval T4, to the moment when the UE starts to send preambles on the PRACH for sending the positioning measurement report message to nCell1.

The RSTD measurement time TRSTD _intra_NB-IoT-EC in the test is derived according to section 4.8.2. This gives the total RSTD measurement time of 11.52s for Cell 2 and Cell 3 with respect to the reference Cell 1

The random access to an already detected cell TRandomAccess_NB-IoT-EC can be expressed as: Tevaluate, NB_intra_NB-IoT-EC + TSI + TPRACH_NB-IoT,

Where:

Tevaluate, NB_intra_NB-IoT-EC	See Table 4.6.2.4-1 in clause 4.6.2.4

TSI = 41560 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT FDD cell.

TPRACH_NB-IoT = 1280 ms; it is the additional delay caused by the random access procedure.

This gives TRandomAccess_NB-IoT-EC = 55.64 s for the random access delay to an already detected cell in the test case.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3 , i.e., between RSTD_0000 and RSTD_12711.

### A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage

#### A.4.7.2.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement period for HD-FDD category NB1 UE meets the delay requirements specified in Clause 4.8.4.

In the test there are three synchronous cells: nCell 1, nCell 2 and nCell 3. nCell 1 is the reference cell. nCell 2 and nCell 3 are the neighbour cells.

The test consists of six consecutive time intervals, with duration of T1, T2, T3, T4, T5, and T6. nCell 1 is active throughout T1, T2, T3, T4, T5, and T6, whilst nCell 2 and nCell 3 are activated only in the beginning of T2. nCell 2 is active until the end of T5, and nCell 3 is active until the end of T4. nCell 1 transmits NPRS in T2 and T4, while nCell 2 transmits NPRS in T3 and T5, and nCell 3 transmits NPRS only in T2 and T4. Note: The information on when NPRS is muted is conveyed to the UE using PRS muting information.

Prior to the start of the time duration T1, the UE shall be fully synchronized to nCell1

At the start of the time duration T1, the OTDOA assistance data and OTDOA-RequestLocationInformation as defined in TS 36.355 [24], shall be provided to the UE. The duration of T1 is sufficiently long to deliver the OTDOA assistance data and OTDOA-RequestLocationInformation to the UE and is independent of the delay requirements specified in Clause 4.8.2.

After OTDOA assistance data and OTDOA-RequestLocationInformation have been successfully received, the UE is provided with a RRC connection release command. The RRC connection release command shall be received by the UE in the last TTI of interval T1. The UE shall enter RRC_IDLE state within T seconds after the receipt of the RRC connection release, where T = 10s is the maximum delay for NB-IOT UE to perform RRC connection release as define in TS36.331 [2].

The test parameters are given in Tables A.4.7.2.1-1 A.4.7.2.1-2 and A.4.7.2.1-3.

Table A.4.7.2.1-1: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Standalone |  |
| Reference cell |  | nCell 1 | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | nCell 2 and nCell 3 | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| nprsID |  | (nprsID of Cell 1 – nprsID of Cell 2)mod6=1 and (nprsID of Cell 1 – nprsID of Cell 3)mod6=2 | As defined in TS36.366 [24] |
| nprs-period | ms | 640 | As defined in TS36.355 [24] |
| nprs-startSF |  | sf0 | Subframe offset of the NPRS positioning occasion as defined in TS36.355 [24] |
| Number of consecutive downlink positioning subframes nprs-NumSF |  | 320 | As defined in TS36.355 [24] |
| nprs-SubframeOffset |  | 0 | As defined in TS36.355 [24] |
| NPRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ | Correponds to nprs-MutingInfoB defined in TS 36.355 [24] |
| PartA Configuration |  | N/A | NPRS is configured based on PartB but not PartA. |
| CP length |  | Normal |  |
| NPRACH Configuration |  | NPRACH.R-1 | Refer to A.3.18 |
| DRX |  | 1.28 | DRX parameters are further specified in Table A.8.12.1.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | nCell 2 to nCell 1: 1 nCell 3 to nCell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| T2 | s | 5.12 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 5.12 | The length of the time interval that follows immediately after time interval T2 |
| T4 | s | 5.12 | The length of the time interval that follows immediately after time interval T3 |
| T5 | s | 5.12 | The length of the time interval that follows immediately after time interval T4 |
| T6 | s | ≥ 57 | The length of the time interval that follows immediately after time interval T5 |

Table A.4.7.2.1-2: Cell-specific test parameters during T1 and T6

| Parameter | Unit | nCell 1 |  | nCell 2 |  | nCell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| NB-IoT RF Channel Number |  | 1 |  | 2 |  | 2 |
| NB-IoT Channel Bandwidth (BWchannel) | KHz | 200 |  | 200 |  | 200 |
| OCNG Pattern Note 1 |  | NOP.3 FDD |  | N/A |  | N/A |
| NPDSCH parametersNote 2 |  | R.18 HD-FDD |  | N/A |  | N/A |
| NPDCCH parameters Note 2 |  | R.30 HD-FDD |  | N/A |  | N/A |
| NPBCH_RA | dB | 0 |  | N/A |  | N/A |
| NPBCH_RB |  |  |  |  |  |  |
| NPSS_RA |  |  |  |  |  |  |
| NSSS_RA |  |  |  |  |  |  |
| NPDCCH_RA |  |  |  |  |  |  |
| NPDCCH_RB |  |  |  |  |  |  |
| NPDSCH_RA |  |  |  |  |  |  |
| NPDSCH_RB |  |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -98 |  |  |  |  |
| NPRS | dB | -Infinity |  | -Infinity |  | -Infinity |
|  | dB | -2 |  | -Infinity |  | -Infinity |
| Propagation Condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  |
| Timing offset to nCell 1 | s | N/A | 1 |  | -1 |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |  |

Table A.4.7.2.1-3: Cell-specific test parameters from T2 to T5

| Parameter | Unit | nCell 1 |  | nCell 2 |  |  |  | nCell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 and T4 | T3 and T5 | T2 and T4 |  | T3 and T5 |  | T2 and T4 | T3 and T5 |
| BWchannel | kHz | 200 |  | 200 |  |  |  | 200 |  |
| NB-IoT RF Channel Number |  | 1 |  | 2 |  |  |  | 2 |  |
| NPDSCH parameters Note 2 |  | R.18 HD-FDD |  | N/A |  |  |  | N/A |  |
| NPDCCH parameters Note 2 |  | R.30 HD-FDD |  | N/A |  |  |  | N/A |  |
| OCNG patterns |  | NOP.3 FDD |  | N/A | NOP.3 FDD |  |  | NOP.3 FDD | N/A |
| NPBCH_RA | dB | 0 |  | 0 |  |  |  | 0 | N/A |
| NPBCH_RB |  |  |  |  |  |  |  |  |  |
| NPSS_RA |  |  |  |  |  |  |  |  |  |
| NSSS_RA |  |  |  |  |  |  |  |  |  |
| NPDCCH_RA |  |  |  |  |  |  |  |  |  |
| NPDCCH_RB |  |  |  |  |  |  |  |  |  |
| NPDSCH_RA |  |  |  |  |  |  |  |  |  |
| NPDSCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |  |  |  |  |
| NPRS_RA | dB | -3 | N/A | N/A |  | 0 |  | 0 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -98 | -98 |  | -95 |  | -98 | -95 |
| NPRS | dB | -15 | -Infinity | -Infinity |  | -15 |  | -15 | -Infinity |
| NPRS Note 4 | dB | -15 | -Infinity | -Infinity |  | -15 |  | -15 | -Infinity |
| Io Note 4 | dBm/ 180kHz | -87.17 | -87.20 | -87.17 |  | -87.15 |  | -87.17 | -87.15 |
| NPRP Note 4 | dBm/ 15 kHz | -113 | -Infinity | -Infinity |  | -110 |  | -113 | -Infinity |
| NRSRP Note 4 | dBm/ 15 kHz | -110 | -110 | -113 |  | -110 |  | -113 | -Infinity |
| Note 4 | dB | -12 | -12 | -15 |  | -15 |  | -15 | -Infinity |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  |  |  |  |
| Timing offset to nCell 1 | s | N/A |  | 1 |  |  | -1 |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted NPRS. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If NPRS_RA is not “N/A”, , NPRS , Io, NRSRP and NPRP levels have been derived from other parameters and are given for information purpose. If NPRS_RA is “N/A”, Io and NRSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |  |  |

#### A.4.7.4.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 4.8.4.

The UE shall perform and report the RSTD measurements for nCell 2 and nCell 3 with respect to the reference cell in the OTDOA assistance data, nCell 1, within TRSTD _inter_NB-IoT-EC +TRandomAccess_NB-IoT-EC  = 67.16 s starting from the beginning of time interval T4, to the moment when the UE starts to send preambles on the PRACH for sending the positioning measurement report message to nCell1.

The RSTD measurement time TRSTD _inter_NB-IoT-EC  in the test is derived according to section 4.8.4. This gives the total RSTD measurement time of 11.52 s for Cell 2 and Cell 3 with respect to the reference Cell 1.

The random access to an already detected cell TRandomAccess_NB-IoT-EC  can be expressed as: Tevaluate, NB_inter_NB-IoT-EC + TSI + TPRACH_NB-IoT,

Where:

Tevaluate, NB_inter_NB-IoT-EC	See Table 4.6.2.4-1 in clause 4.6.2.4

TSI = 41560 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT FDD cell.

TPRACH_NB-IoT = 1280 ms; it is the additional delay caused by the random access procedure.

This gives TRandomAccess_NB-IoT =55.64 s for the random access delay to an already detected cell in the test case.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

### A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage

#### A.4.7.3.1	Test Purpose and Environment

The purpose of the test is to verify that the intra frequency RSTD measurement period for TDD category NB1 UE meets the delay requirements specified in Clause 4.8.2.In the test there are three synchronous cells: nCell 1, nCell 2 and nCell 3. nCell 1 is the reference cell. nCell 2 and nCell 3 are the neighbour cells. All cells are on the same RF channel.

The test consists of six consecutive time intervals, with duration of T1, T2, T3, T4, T5, and T6. nCell 1 is active throughout T1, T2, T3, T4, T5, and T6, whilst nCell 2 and nCell 3 are activated only in the beginning of T2. nCell 2 is active until the end of T5, and nCell 3 is active until the end of T4. nCell 1 transmits NPRS in T2 and T4, while nCell 2 transmits NPRS in T3 and T5, and nCell 3 transmits NPRS only in T2 and T4. Note: The information on when NPRS is muted is conveyed to the UE using PRS muting information.

Prior to the start of the time duration T1, the UE shall be fully synchronized to nCell1.

At the start of the time duration T1, the OTDOA assistance data and OTDOA-RequestLocationInformation as defined in TS 36.355 [24], shall be provided to the UE. The duration of T1 is sufficiently long to deliver the OTDOA assistance data and OTDOA-RequestLocationInformation to the UE and is independent of the delay requirements specified in Clause 4.8.2.

After OTDOA assistance data and OTDOA-RequestLocationInformation have been successfully received, the UE is provided with an RRC connection release command. The RRC connection release command shall be received by the UE in the last TTI of interval T1. The UE shall enter RRC_IDLE state within T seconds after the receipt of the RRC connection release, where T = 10s is the maximum delay for NB-IOT UE to perform RRC connection release as define in TS 36.331 [2].

The test parameters are given in Tables A.4.7.3.1-1 A.4.7.3.1-2 and A.4.7.3.1-3.

Table A.4.7.3.1-1: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Standalone |  |
| Reference cell |  | nCell 1 | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | nCell 2 and nCell 3 | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| nprsID |  | (nprsID of Cell 1 – nprsID of Cell 2)mod6=1 and (nprsID of Cell 1 – nprsID of Cell 3)mod6=2 | As defined in TS 36.355 [24] |
| nprs-period | ms | 1280 | As defined in TS 36.355 [24] |
| nprs-startSF |  | sf0 | Subframe offset of the NPRS positioning occasion as defined in TS 36.355 [24] |
| Number of consecutive downlink positioning subframes nprs-NumSF |  | 640 | As defined in TS 36.355 [24] |
| NPRS muting info |  | nCell 1: ‘11110000’ nCell 2: ‘00001111’ nCell 3: ‘11110000’ | Correponds to nprs-MutingInfoB defined in TS 36.355 [24] |
| PartA Configuration |  | N/A | NPRS is configured based on PartB but not PartA. |
| CP length |  | Normal |  |
| NPRACH Configuration |  | NPRACH.R-2 | Specified in A.3.18 |
| DRX cycle length |  | 1.28 | The value shall be used for all cells in the test. |
| Radio frame receive time offset between the cells at the UE antenna connector | s | nCell 2 to nCell 1: 1 nCell 3 to nCell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | nCell 2: 3 nCell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 2 | As specified in table 4.2-2 in TS 36.211 [16] |
| T2 | s | 5.12 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 5.12 | The length of the time interval that follows immediately after time interval T2 |
| T4 | s | 5.12 | The length of the time interval that follows immediately after time interval T3 |
| T5 | s | [5.12] | The length of the time interval that follows immediately after time interval T4 |
| T6 | s | ≥ 58.2 | The length of the time interval that follows immediately after time interval T5 |

Table A.4.7.3.1-2: Cell-specific test parameters during T1 and T6

| Parameter | Unit | nCell 1 |  | nCell 2 |  | nCell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| NB-IoT RF Channel Number |  | 1 |  | 1 |  | 1 |
| NB-IoT Channel Bandwidth (BWchannel) | kHz | 200 |  | 200 |  | 200 |
| OCNG Pattern Note 1 |  | NOP.3 TDD |  | N/A |  | N/A |
| NPDSCH parameters Note 2 |  | R.18 NB-TDD |  | N/A |  | N/A |
| NPDCCH parameters Note 2 |  | R.30 NB-TDD |  | N/A |  | N/A |
| NPBCH_RA | dB | 0 |  | N/A |  | N/A |
| NPBCH_RB |  |  |  |  |  |  |
| NPSS_RA |  |  |  |  |  |  |
| NSSS_RA |  |  |  |  |  |  |
| NPDCCH_RA |  |  |  |  |  |  |
| NPDCCH_RB |  |  |  |  |  |  |
| NPDSCH_RA |  |  |  |  |  |  |
| NPDSCH_RB |  |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -98 |  |  |  |  |
| NPRS | dB | -Infinity |  | -Infinity |  | -Infinity |
|  | dB | -2 |  | -Infinity |  | -Infinity |
| Propagation Condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  |
| Timing offset to nCell 1 | s | N/A | 1 |  | -1 |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |  |

Table A.4.7.3.1-3: Cell-specific test parameters from T2 to T5

| Parameter | Unit | nCell 1 |  | nCell 2 |  |  |  | nCell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 and T4 | T3 and T5 | T2 and T4 |  | T3 and T5 |  | T2 and T4 | T3 and T5 |
| BWchannel | kHz | 200 |  | 200 |  |  |  | 200 |  |
| NB-IoT RF Channel Number |  | 1 |  | 1 |  |  |  | 1 |  |
| OCNG patterns |  | NOP.3 TDD |  | N/A | NOP.3 TDD |  |  | NOP.3 TDD | N/A |
| NPBCH_RA | dB | 0 |  | 0 |  |  |  | 0 | N/A |
| NPBCH_RB |  |  |  |  |  |  |  |  |  |
| NPSS_RA |  |  |  |  |  |  |  |  |  |
| NSSS_RA |  |  |  |  |  |  |  |  |  |
| NPDCCH_RA |  |  |  |  |  |  |  |  |  |
| NPDCCH_RB |  |  |  |  |  |  |  |  |  |
| NPDSCH_RA |  |  |  |  |  |  |  |  |  |
| NPDSCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |  |  |  |  |
| NPRS_RA | dB | -3 | N/A | N/A |  | 0 |  | 0 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 |  | -95 |  | -98 | -95 |
| NPRS | dB | -15 | -Infinity | -Infinity |  | -15 |  | -15 | -Infinity |
| NPRS Note 4 | dB | -15 | -Infinity | -Infinity |  | -15 |  | -15 | -Infinity |
| Io Note 4 | dBm/ 180kHz | -87.14 | -87.12 | -87.14 |  | -87.12 |  | -87.14 | -87.12 |
| NPRP Note 4 | dBm/ 15 kHz | -113 | -Infinity | -Infinity |  | -110 |  | -113 | -Infinity |
| NRSRP Note 4 | dBm/ 15 kHz | -110 | -107 | -113 |  | -110 |  | -113 | -Infinity |
| Note 4 | dB | -12 | -12 | -15 |  | -15 |  | -15 | -Infinity |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  |  |  |  |
| Timing offset to nCell 1 | s | N/A |  | 1 |  |  | -1 |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted NPRS. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If NPRS_RA is not “N/A”, , NPRS , Io, NRSRP and NPRP levels have been derived from other parameters and are given for information purpose. If NPRS_RA is “N/A”, Io and NRSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |  |  |

#### A.4.7.3.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 4.8.2.

The UE shall perform and report the RSTD measurements for nCell 2 and nCell 3 with respect to the reference cell in the OTDOA assistance data, nCell 1, within TRSTD _intra_NB-IoT-EC +TRandomAccess_NB-IoT-EC = 68.44 s starting from the beginning of time interval T4, to the moment when the UE starts to send preambles on the PRACH for sending the positioning measurement report message to nCell 1.

The RSTD measurement time TRSTD _intra_NB-IoT-EC in the test is derived according to section 4.8.2. This gives the total RSTD measurement time of 11.52 s for nCell 2 and nCell 3 with respect to the reference nCell 1.

The random access to an already detected cell TRandomAccess_NB-IoT-EC can be expressed as: Tevaluate, NB_intra_NB-IoT-EC + TSI + TPRACH_NB-IoT,

Where:

Tevaluate, NB_intra_NB-IoT-EC	See Table 4.6.2.4-1 in clause 4.6.2.4

TSI = 41560 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT TDD cell.

TPRACH_NB-IoT = 2560 ms; it is the additional delay caused by the random access procedure.

This gives TRandomAccess_NB-IoT-EC = 56.92 s for the random access delay to an already detected cell in the test case.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

### A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage

#### A.4.7.4.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement period for TDD category NB1 UE meets the delay requirements specified in Clause 4.8.4. In the test there are three synchronous cells: nCell 1, nCell 2 and nCell 3. nCell 1 is the reference cell. nCell 2 and nCell 3 are the neighbour cells.

The test consists of six consecutive time intervals, with durations of T1, T2, T3, T4, T5, and T6. nCell 1 is active throughout T1, T2, T3, T4, T5, and T6, whilst nCell 2 and nCell 3 are activated only in the beginning of T2. nCell 2 is active until the end of T5, and nCell 3 is active until the end of T4. nCell 1 transmits NPRS in T2 and T4, while nCell 2 transmits NPRS in T3 and T5, and nCell 3 transmits NPRS only in T2 and T4. Note: The information on when NPRS is muted is conveyed to the UE using PRS muting information.

Prior to the start of the time duration T1, the UE shall be fully synchronized to nCell 1.

At the start of the time duration T1, the OTDOA assistance data and OTDOA-RequestLocationInformation as defined in TS 36.355 [24], shall be provided to the UE. The duration of T1 is sufficiently long to deliver the OTDOA assistance data and OTDOA-RequestLocationInformation to the UE and is independent of the delay requirements specified in Clause 4.8.2.

After OTDOA assistance data and OTDOA-RequestLocationInformation have been successfully received, the UE is provided with a RRC connection release command. The RRC connection release command shall be received by the UE in the last TTI of interval T1. The UE shall enter RRC_IDLE state within T seconds after the receipt of the RRC connection release, where T = 10s is the maximum delay for NB-IOT UE to perform RRC connection release as define in TS 36.331 [2].

The test parameters are given in Tables A.4.7.4.1-1 A.4.7.4.1-2 and A.4.7.4.1-3.

Table A.4.7.4.1-1: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Standalone |  |
| Reference cell |  | nCell 1 | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | nCell 2 and nCell 3 | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| nprsID |  | (nprsID of Cell 1 – nprsID of Cell 2)mod6=1 and (nprsID of Cell 1 – nprsID of Cell 3)mod6=2 | As defined in TS 36.355 [24] |
| nprs-period | ms | 1280 | As defined in TS 36.355 [24] |
| nprs-startSF |  | sf0 | Subframe offset of the NPRS positioning occasion as defined in TS 36.355 [24] |
| Number of consecutive downlink positioning subframes nprs-NumSF |  | 640 | As defined in TS 36.355 [24] |
| nprs-SubframeOffset |  | 0 | As defined in TS 36.355 [24] |
| NPRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ | Correponds to nprs-MutingInfoB defined in TS 36.355 [24] |
| PartA Configuration |  | N/A | NPRS is configured based on PartB but not PartA. |
| CP length |  | Normal |  |
| NPRACH Configuration |  | NPRACH.R-2 | Refer to A.3.18 |
| DRX |  | 1.28 | DRX parameters are further specified in Table A.8.12.1.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | nCell 2 to nCell 1: 1 nCell 3 to nCell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 2 | As specified in table 4.2-2 in TS 36.211 [16] |
| T2 | s | 5.12 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 5.12 | The length of the time interval that follows immediately after time interval T2 |
| T4 | s | 10.24 | The length of the time interval that follows immediately after time interval T3 |
| T5 | s | 10.24 | The length of the time interval that follows immediately after time interval T4 |
| T6 | s | ≥ 58.2 | The length of the time interval that follows immediately after time interval T5 |

Table A.4.7.4.1-2: Cell-specific test parameters during T1 and T6

| Parameter | Unit | nCell 1 |  | nCell 2 |  | nCell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| NB-IoT RF Channel Number |  | 1 |  | 2 |  | 2 |
| NB-IoT Channel Bandwidth (BWchannel) | KHz | 200 |  | 200 |  | 200 |
| OCNG Pattern Note 1 |  | NOP.3 TDD |  | N/A |  | N/A |
| NPDSCH parametersNote 2 |  | R.18 NB-TDD |  | N/A |  | N/A |
| NPDCCH parameters Note 2 |  | R.30 NB-TDD |  | N/A |  | N/A |
| NPBCH_RA | dB | 0 |  | N/A |  | N/A |
| NPBCH_RB |  |  |  |  |  |  |
| NPSS_RA |  |  |  |  |  |  |
| NSSS_RA |  |  |  |  |  |  |
| NPDCCH_RA |  |  |  |  |  |  |
| NPDCCH_RB |  |  |  |  |  |  |
| NPDSCH_RA |  |  |  |  |  |  |
| NPDSCH_RB |  |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -98 |  |  |  |  |
| NPRS | dB | -Infinity |  | -Infinity |  | -Infinity |
|  | dB | -2 |  | -Infinity |  | -Infinity |
| Propagation Condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  |
| Timing offset to nCell 1 | s | N/A | 1 |  | -1 |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |  |

Table A.4.7.4.1-3: Cell-specific test parameters from T2 to T5

| Parameter | Unit | nCell 1 |  | nCell 2 |  |  |  | nCell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 and T4 | T3 and T5 | T2 and T4 |  | T3 and T5 |  | T2 and T4 | T3 and T5 |
| BWchannel | kHz | 200 |  | 200 |  |  |  | 200 |  |
| NB-IoT RF Channel Number |  | 1 |  | 2 |  |  |  | 2 |  |
| NPDSCH parameters Note 2 |  | R.18 NB-TDD |  | N/A |  |  |  | N/A |  |
| NPDCCH parameters Note 2 |  | R.30 NB-TDD |  | N/A |  |  |  | N/A |  |
| OCNG patterns |  | NOP.3 TDD |  | N/A | NOP.3 TDD |  |  | NOP.3 TDD | N/A |
| NPBCH_RA | dB | 0 |  | 0 |  |  |  | 0 | N/A |
| NPBCH_RB |  |  |  |  |  |  |  |  |  |
| NPSS_RA |  |  |  |  |  |  |  |  |  |
| NSSS_RA |  |  |  |  |  |  |  |  |  |
| NPDCCH_RA |  |  |  |  |  |  |  |  |  |
| NPDCCH_RB |  |  |  |  |  |  |  |  |  |
| NPDSCH_RA |  |  |  |  |  |  |  |  |  |
| NPDSCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |  |  |  |  |
| NPRS_RA | dB | -3 | N/A | N/A |  | 0 |  | 0 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -98 | -98 |  | -95 |  | -98 | -95 |
| NPRS | dB | -15 | -Infinity | -Infinity |  | -15 |  | -15 | -Infinity |
| NPRS Note 4 | dB | -15 | -Infinity | -Infinity |  | -15 |  | -15 | -Infinity |
| Io Note 4 | dBm/ 180kHz | -87.17 | -87.20 | -87.17 |  | -87.15 |  | -87.17 | -87.15 |
| NPRP Note 4 | dBm/ 15 kHz | -113 | -Infinity | -Infinity |  | -110 |  | -113 | -Infinity |
| NRSRP Note 4 | dBm/ 15 kHz | -110 | -110 | -113 |  | -110 |  | -113 | -Infinity |
| Note 4 | dB | -12 | -12 | -15 |  | -15 |  | -15 | -Infinity |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  |  |  |  |
| Timing offset to nCell 1 | s | N/A |  | 1 |  |  | -1 |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted NPRS. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If NPRS_RA is not “N/A”, , NPRS , Io, NRSRP and NPRP levels have been derived from other parameters and are given for information purpose. If NPRS_RA is “N/A”, Io and NRSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |  |  |

#### A.4.7.4.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 4.8.4.

The UE shall perform and report the RSTD measurements for nCell 2 and nCell 3 with respect to the reference cell in the OTDOA assistance data, nCell 1, within TRSTD _inter_NB-IoT-EC +TRandomAccess_NB-IoT-EC  = 78.68 s starting from the beginning of time interval T4, to the moment when the UE starts to send preambles on the PRACH for sending the positioning measurement report message to nCell 1.

The RSTD measurement time TRSTD _inter_NB-IoT-EC  in the test is derived according to section 4.8.4. This gives the total RSTD measurement time of 21.76 s for nCell 2 and nCell 3 with respect to the reference nCell 1.

The random access to an already detected cell TRandomAccess_NB-IoT-EC  can be expressed as: Tevaluate, NB_inter_NB-IoT-EC + TSI + TPRACH_NB-IoT,

Where:

Tevaluate, NB_inter_NB-IoT-EC	See Table 4.6.2.4-1 in clause 4.6.2.4

TSI = 41560 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT TDD cell.

TPRACH_NB-IoT = 2560 ms; it is the additional delay caused by the random access procedure.

This gives TRandomAccess_NB-IoT =56.92 s for the random access delay to an already detected cell in the test case.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

< End of changes >