3GPP TSG-RAN WG4 Meeting #92                                     R4-1908371

# Ljubljana, SI, 26th– 30th Aug, 2019

Source: 	Charter Communications

Title: 	Emission Mask Considerations for NR-U DL single wideband carrier operation modes

Agenda item:	9.1.2.1

Document for:	Approval

Introduction

In RAN4 #91, we discussed emission mask considerations in the 5 GHZ band for NR-U wideband carrier operation modes [1]. Four observations were made:

Observation1: With the ETSI mask, the emission limits become relaxed as the bandwidth widens.

Observation 2: Using the 802.11ac mask or a tighter mask will ensure that NR-U would not impact already deployed Wi-Fi generations more than an additional Wi-Fi network of the same generation on the same carrier.

Observation 3: For 802.11ax preamble punctured mask, the signal leakage to the punctured channel from the occupied sub channels is less than or equal to –20 dBr.

- This mask does not seem to comply with ETSI mask for transmission on non-adjacent channel.

- This mask could be harmful to existing incumbents in 5 GHz and therefore is not recommended.

Observation 4: Currently, there are no 11ax preamble punctured feature in Wi-Fi 6 wave one. Discussions on the next generation Wi-Fi 6 wave two has not started; therefore it is not expected in a foreseeable future to have certified 802.11ax devices with this feature.

As a result of this discussion paper and others, an LS [2] was sent to ETSI to confirm some of our understandings. In this LS we asked the following questions:

Question 1: 	RAN4 would kindly request ETSI TC BRAN understanding whether preamble punctured mask for a wide band carrier larger than 20MHz having a relaxed mask requirements with a constant reference level of -20 dBr in the punctured region(s) is compliant to ETSI mask?

Question 2: 	RAN4 respectfully ask ETSI TC BRAN to confirm our understanding of the ETSI transmit spectral mask widening with increase bandwidth configurations, particularly at the -20dBr limit

and explain the reasoning’s for this behavior

- In this paper we are planning to highlight several proposals that can address the concerns shown above and insure that the NR-U design enables fair coexistence between already deployed Wi-Fi generations and NR-U, between NR-U and LTE-LAA, and between different NR-U systems in the 5 GHz band.  NR-U should not impact already deployed Wi-Fi generations more than an additional Wi-Fi network of the same generation on the same carrier.

Discussion

- Two proposals are highlighted in this paper to address the possible impact to co-existence with current incumbents in the 5 GHz band applying ETSI like emission mask

  - ETSI Emission Mask

Figure1: ETSI 5GHz transmission mask [3]

As previously shown in [3], Figure1 demonstrates the behavior of the ETSI emission mask limits.  In the ETSI mask as the channel bandwidth increases, the -20dbr limit widens.

- For 20 MHz channel bandwidth, the -20 dbr limit is 11 MHz

- For 40 MHz bandwidth, the -20dbr limit is 22 MHz

- For 80 MHz channel bandwidth, the -20 dbr limit s is 44 MHz

Observation 1: With the ETSI mask, the emission limits become relaxed as the bandwidth widens.

Proposal 1: For wideband transmission over contiguous bands with successful LBT (Mode 1 & 2), keep the spectral emission of -20dBr limit for wider bandwidths with the same relative distance from the edge as the spectral mask of the 20 MHz channel bandwidth.

2.2 ETSI transmission mask for non-contiguous punctured transmissions

Figure2: ETSI transmission mask for an 80MHz channel with a 20MHz punctured channel

Figure 2 shows an example of an 80MHz channel transmission with a punctured 20MHz channel. The required ETSI transmission mask is shown in red which would be several dB less that -20dBr.

Figure 3: Comparison of the overall spectral emission masks for the [1 0 0 1] channel puncturing pattern

Figure3 shows an example of transmission on an 80 MHz channel with 2 contiguous channels punctured.  If 2 or more contiguous channels are punctured the gap in between punctured channels will be significantly > -20dbr limit.

Proposal 2: For wideband operation with punctured channels (Modes 2 & 3), use the OOBE mask at level of -28dBr and -40dBr wherever applicable (e.g. 10MHz and 20MHz from the edge respectively for Fig. 3).

Proposal 3: Neither proposal can be approved until ETSI responds to questions 1 and 2 in the LS [2] and clarifies our understanding of the ETSI mask

Conclusions

- In this paper, we have made two proposals to mitigate the impact of NR-U wider bandwidth operations enabling fair coexistence between already deployed Wi-Fi generations and NR-U, between NR-U and LTE-LAA, and between different NR-U systems in the 5 GHz band.

Observation 1: With the ETSI mask, the emission limits become relaxed as the bandwidth widens.

Proposal 1: For wideband transmission over contiguous bands with successful LBT (Mode 1 & 2), keep the spectral emission of -20dBr limit for wider bandwidths with the same relative distance from the edge as the spectral mask of the 20 MHz channel bandwidth.

Proposal 2: For wideband operation with punctured channels (Modes 2 & 3), use the OOBE mask at level of -28dBr and -40dBr wherever applicable (e.g. 10MHz and 20MHz from the edge respectively for Fig. 3).

Proposal 3: Neither proposal can be approved until ETSI responds to questions 1 and 2 in the LS [2] and clarifies our understanding of the ETSI mask

# References

[1] R4-1907124	Emission Mask Considerations for single wideband carrier operation modes

[2] R4-1907850	LS to ETSI TC BRAN on Interpretations of EN 301 893 for NR-U

[3] ETSI EN 301 893, “5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”