3GPP TSG-RAN WG4 Meeting #92                                                                R4-1908393

# Ljubljana, Slovenia, 26th – 30th August, 2019

Source: 	CATT

Title: 	Discussion on the simulation assumption for HST PUSCH

Agenda item:	9.16.2.3

Document for:	Agreement

- Introduction

In the recent RAN#84 meeting, a new WID [1] on NR support for high speed train was approved. One of the objectives is to investigate the scenarios for NR support for high speed train shown as follows:

- Following scenarios will be investigated: - The channel model: - HST-SFN scenarios, e.g. the channel mode captured in Section B.3A of 3GPP TS 36.101 - HST scenarios with high Doppler frequency, e.g. the channel mode captured in Section B.3 of 3GPP TS 36.101 - TDL channel with high Doppler frequency - Other deployment scenarios are not precluded - The target speed is up to 500km/h - The carrier frequency is up to 6GHz

This paper will give our view on  HST scenario  and its parameters for NR HST PUSCH based on simuation results for HST PUSCH.

- Discussion

Generally the maximum Doppler shift for UE is fd while that for BS is 2fd without the offset correction. The fd and 2fd for 300km/h and 350km/h at 2.07GHz, 2.1GHz and 3.6GHz are shown in Table 2-1 respectively.

Table 2-1 fd and 2fd for 300km/h and 350km/h at 2.07GHz, 2.1GHz, and 3.6GHz

| Speed() | 2.07GHz |  | 2.1GHz |  | 3.6GHz |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | (Hz) | 2(Hz) | (Hz) | 2(Hz) | (Hz) | 2(Hz) |
| 300 km/h | 575 | 1150 | 583 | 1166 | 1000 | 2000 |
| 350 km/h | 670 | 1340 | 680 | 1360 | 1166 | 2332 |

The HST Scenario 1 and HST Scenario 3 defined in Table B.3-1 in TS 36.104 are required as propagation conditions for LTE HST PUSCH. The parameters for HST Scenario 1 and HST Scenario 3 are shown as follows:

Table B.3-1: Parameters for high speed train conditions Parameter; Value. Scenario 1; Scenario 3. 1000 m; 300 m. 50 m; 2 m. 350 km/h; 300 km/h. 1340 Hz; 1150 Hz. NOTE1: Parameters for HST conditions in table B.3-1 including and Doppler shift trajectories presented on figures B.3-1 and B.3-2 were derived from Band1 and are applied for performance verification in all frequency bands.

The maximum Doppler shift for 300km/h in HST scenario 3 and 350km/h in HST scenario 1 are calculated at 2.07GHz according to Table 2-1. The contribution [2] indicates the band n77(3.6GHz) and band n1(2.1GHz) are candidate bands for NR HST. For 2.1GHz, the scenarios and speed supported by LTE HST PUSCH are sufficient to be used for NR HST PUSCH, because the 2fd at 2.1GHz is close to that at 2.07GHz according to Table 2-1. But for 3.6GHz, BS maximum Doppler shifts for 300km/h in HST scenario 3 and 350km/h in HST scenario 1 at 3.6GHz are required as 2000Hz and 2332Hz respectively according to Table2-1. Whether the NR BS can support those BS maximum Doppler shift at 3.6GHz or not need to be evaluated. The detailed parameters for high speed train conditions for evaluation of NR HST PUSCH are summarized in Table 2-2.

Table 2-2: Parameters for high speed train conditions

| Parameter | Value |  |
| --- | --- | --- |
|  | Scenario 1 | Scenario 3 |
|  | 1000 m | 300 m |
|  | 50 m | 2 m |
|  | 350 km/h | 300 km/h |
|  | 1340 Hz for 2.1GHz(for 15kHz SCS), 2332 Hz for 3.6GHz(for 30kHz SCS) | 1150 Hz for 2.1GHz (for 15kHz SCS), 2000Hz for 3.6GHz(for 30kHz SCS) |

Based on our proposed propagation parameters for high speed train conditions, we provide our evaluation results for NR HST PUSCH shown in Table 2-3 with simulation assumption for NR HST PUSCH shown in Table 5-1 in Annex.

Table 2-3 evaluation results for NR HST PUSCH

| waveform | FR and mapping type | MCS level | SCS-BW | Frequency domain resource | Carrier Frequency (GHz) | Propagation conditions | SNR @70% of maximum throughput (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 Mappying type A | MCS2 | 15kHz, 10MHz | Full applicable test bandwidth | 2.1 | HST- Scenario 3 1150Hz | -5.25 |
|  |  |  |  |  |  | HST- Scenario 1 1340Hz | [TBD] |
|  |  |  | 30kHz, 40MHz | Full applicable test bandwidth | 3.6 | HST- Scenario 3 2000Hz | -4.62 |
|  |  |  |  |  |  | HST- Scenario 1 2332Hz | [TBD] |
| DFT-s-OFDM | FR1 Mappying type A | MCS2 | 15kHz, 5MHz | 15kHz: 25 PRB | 2.1 | HST- Scenario 3 1150Hz | -4.83 |
|  |  |  |  |  |  | HST- Scenario 1 1340Hz | [TBD] |
|  |  |  | 30kHz, 10MHz | 30kHz: 24 PRB (middle of test BW) | 3.6 | HST- Scenario 3 2000Hz | -4.88 |
|  |  |  |  |  |  | HST- Scenario 1 2332Hz | [TBD] |

- Conclusion

This paper discussed HST scenario and its parameters for NR HST PUSCH, Based on discussed HST scenario, we provide our evaluation results for NR HST PUSCH.

- References

- RP-191219,New WID on NR support for high speed train scenario, CMCC, RAN #84

- R4-1907243,WF on high speed related requirements for NR BS demodulation in Rel.15, NTT DOCOMO, INC.,RAN4#91

- Annex

Table 5-1 Simulation assumption for HST PUSCH

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 |  |
| Transform precoding | Disabled | Enabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Number of layers | 1 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| symbols length | 14 | 14 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A | type A |
| Frequency domain resource | Full applicable test bandwidth | 15kHz: 25 PRB; 30kHz: 24 PRB (middle of test BW) |
| MCS index | 2 | 2 |
| PTRS | Not configured | Not configured |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| RV | {0,2,3,1} | {0,2,3,1} |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1+1+1 | 1+1+1 |
| Front loaded DMRS start symbol | l0= 3 | l0= 3 |
| SCS and BW | 15kHz: 10MHz; 30kHz: 40MHz | 15kHz: 5 MHz; 30kHz: 10 MHz |
| Carrier frequency (GHz) | For 15kHz SCS: 2.1GHz For 30kHz SCS: 3.6GHz | For 15kHz SCS: 2.1GHz For 30kHz SCS: 3.6GHz |
| Propagation condition | HST -Scenario 1 low, HST- Scenario 3 (table 5-2) | HST -Scenario 1 low, HST- Scenario 3 (table 5-2) |