3GPP TSG-RAN WG4 Meeting #92	R4-1908516

# Ljubljana, Slovenia, 26th – 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | xxxx | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on NR-DC requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Release Rel-15 NR DC supports only “NR DC between FR1 and FR2” scenarios. For these scenarios demodulation and CSI requirements are rather similar to “EN-DC including FR2” (because testing is applied on FR2 carriers) and SDR requirements are rather similar to “NR CA between FR1 and FR2”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined NR-DC requirements for PDSCH, PDSCH, SDR, CQI, PMI and RI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR-DC requirements are not defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2B.2, 9.3B.2, 9.4B.2, 10.2B.2, 10.3B.2, 10.4B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 9.2B	PDSCH demodulation for DC

### 9.2B.1	EN-DC

<SKIP UNCHANGED PART>

### 9.2B.2	NR DC between FR1 and FR2

The test setup for FR1 cell group is specified in Section 5.2. The test setup for FR2 cell group is specified in Section 7.2. During the test, only the PDSCH performance on the NR cell(s) on FR2 carriers shall be verified. The PDSCH demodulation performance requirements for NR FR2 carriers are specified in Section 7.2.

<SKIP UNCHANGED PART>

## 9.3B	PDCCH demodulation for DC

### 9.3B.1	EN-DC

<SKIP UNCHANGED PART>

### 9.3B.2	NR DC between FR1 and FR2

The test setup for FR1 cell group is specified in Section 5.3. The test setup for FR2 cell group is specified in Section 7.3. During the test, only the PDCCH performance on the NR cell(s) on FR2 carriers shall be verified. The PDCCH demodulation performance requirements for NR FR2 carriers are specified in Section 7.3.

<SKIP UNCHANGED PART>

## 9.4B	SDR test for DC

### 9.4B.1	EN-DC

<SKIP UNCHANGED PART>

### 9.4B.2	NR DC between FR1 and FR2

The methodology for selection of tested NR DC bandwidth combination and the requirements are specified in Section 9.4A.1.

<SKIP UNCHANGED PART>

## 10.2B	Reporting of Channel Quality Indicator (CQI) for DC

### 10.2B.1	EN-DC

<SKIP UNCHANGED PART>

### 10.2B.2	NR DC between FR1 and FR2

The test setup for FR1 cell group is specified in Sections 6.1.2 and 6.2. The test setup for FR2 cell group is specified in Sections 8.1.2 and 8.2. During the test, only the performance based on NR requirements on the NR cell(s) on FR2 carriers shall be verified. The NR CQI reporting requirements for NR FR2 carriers are specified in Clause 8.2.

<SKIP UNCHANGED PART>

## 10.3B	Reporting of Precoding Matrix Indicator (PMI) for DC

### 10.3B.1	EN-DC

<SKIP UNCHANGED PART>

### 10.3B.2	NR DC between FR1 and FR2

The test setup for FR1 cell group is specified in Sections 6.1.2 and 6.3. The test setup for FR2 cell group is specified in Section s 8.1.2 and 8.3. During the test, only the performance based on NR requirements on the NR cell(s) on FR2 carriers shall be verified. The NR PMI reporting requirements for NR FR2 carriers are specified in Clause 8.3.

<SKIP UNCHANGED PART>

## 10.4B	Reporting of Rank Indicator (RI) for DC

### 10.4B.1	EN-DC

<SKIP UNCHANGED PART>

### 10.4B.2	NR DC between FR1 and FR2

The test setup for FR1 cell group is specified in Sections 6.1.2 and 6.4. The test setup for FR2 cell group is specified in Section s 8.1.2 and 8.4. During the test, only the performance based on NR requirements on the NR cell(s) on FR2 carriers shall be verified. The NR RI reporting requirements for NR FR2 carriers are specified in Clause 8.4.

END OF CHANGE