3GPP TSG-RAN WG4 Meeting #92 	R4-1908525

# Ljubljana, Slovenia, August 26th – 30th, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-3: Corrections to reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| 77 |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | - n71(UL)-2(DL) MSD is due to UL harmonic, not Rx harmonic mixing. - n79(UL)-21(DL) MSD is due to 3rd order harmonic mixing, not 4th order harmonic mixing. - No combination MSD is due to 4th order harmonic mixing. - In Table 7.3B.2.3.2-1, inconsistency was found among NOTES for describing the UL/DL frequency ranges. - In Table 7.3B.2.3.2-1 NOTE 8, UL and DL frequency should be swapped. - In Table 7.3B.2.3.2-2, UL configurations for 41(UL)-n78(DL), 41(UL)-n77(DL), n77(UL)-41(DL), and n78(UL)-41(DL) need to be scaled according to DL channel BW. - In Table 7.3B.2.3.2-2, NOTE 1 is specific for 3rd order harmonic mixing which cannot be applied to the entire table. There is no need for this note as NOTE 1 in Table 7.3B.2.3.2-1 is suffiicent to describe the applicable scenario. - In Table 7.3B.2.3.2-2, NOTE 2 is specific for 3rd order harmonic mixing which cannot be applied to the entire table. There is no need for this note as the corresponding conditions have been decribes in the notes in Table 7.3B.2.3.2-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove n71(UL)-2(DL) from Table 7.3B.2.3.2-1. - Change NOTE index for n79(UL)-21(DL) from 3 to 4 in Table 7.3B.2.3.2-1. - Void NOTE 3 in Table 7.3B.2.3.2-1. - Remove UL/DL frequency ranges for NOTE 2, NOTE 4, NOTE 7, and NOTE 8 and improve the wordings in Table 7.3B.2.3.2-1. - Swap UL and DL frequency in Table 7.3B.2.3.2-1 NOTE 8. - Change UL configurations for 41(UL)-n78(DL), 41(UL)-n77(DL), n77(UL)-41(DL), and n78(UL)-41(DL) according to DL channel BW and UL harmonic order. - Void NOTE 1 and NOT 2 in Table 7.3B.2.3.2-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1 are not correctly specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<< Start of changed sections>>>

##### 7.3B.2.3.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (low) are specified in Table 7.3B.2.3.2-1 with uplink configuration of the agressor band (high) specified in Table 7.3B.2.3.2-2.

Table 7.3B.2.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 2 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| n41 | 264 | 24.3 | 24.3 | 22.5 | N/A |  |  |  |  |  |  |  |
| 41 | n777 |  | 8.3 | 8.0 | 6.9 |  | 3.9 | 3 | 2.3 | 1.2 |  | 0.4 |
| 41 | n787 |  | 8.3 | 8.0 | 6.9 |  | 3.9 | 3 | 2.3 | 1.2 |  | 0.4 |
| n77 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n77 | 282 | 28 | 25 | 23.2 | 22 |  |  |  |  |  |  |  |
| n78 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n79 | 192 | 29.5 | 26.5 | 24.7 |  |  |  |  |  |  |  |  |
| n79 | 214 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 262 | 27 | 24 | 22.2 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void NOTE 6: Void NOTE 7: The requirements should be verified for DL NR-ARFCN of the victim (higher) band (superscript HB) such that fDLHB=50×fULLB0.03 with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 2 | n71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |
| n41 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| 414 |  |  | n77 |  |  | 15 |  |  |  |  |  | 16 |  | 25 |  | 32 |  |  |  | 64 |  | 80 |  | 100 |  | 144 |  |  |  | 180 |
| 414 |  | n78 |  |  | 15 |  |  |  |  |  | 16 |  | 25 |  | 32 |  |  |  | 64 |  | 80 |  | 100 |  | 144 |  |  |  | 180 |  |
| n77 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n774 | 41 | 15 |  | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n784 | 41 | 15 |  | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n79 | 19 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 21 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| NOTE 1: Void NOTE 2: Void NOTE 3: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2]. NOTE 4: The UL resource blocks allocation is applied at the center of the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<< End of changed sections>>>