3GPP TSG RAN WG4 Meeting #92	R4-1908602

Ljubljana, Slovenia, 26 - 30 Aug 2019

Agenda item:	10.4.2

Source:	SoftBank Corp.

Title:	TP for TR 37.716-21-11: EN-DC_8-42_n77

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_8-42_n77 was approved in RAN#84[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_8-42_n77 | CA_8-42 | n77 | No |

Table 2: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_8A-42A_n77A | DC_8A_n77A | CA_8A-42A | n77A |
| DC_8A-42C_n77A | DC_8A_n77A | CA_8A-42C | n77A |

3.  Co-existence

The co-existence studies for the lower order 2DL/2UL DC configurations are already captured in TR 37.863-01-01. For the additional 3rd receiver band, there is no MSD issue for this DC configuration based on the following conditions.

- LTE Band 42 and NR Band n77 are synchronized.

- Simultaneous transmission of LTE Band 42 and NR Band n77 are not supported.

4.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked from [2].

Table 3: ΔTIB of constituent 2 band combo.

| ΔTIB | B8+B42 | B8+n77 | B42+n77 | Proposed Value |
| --- | --- | --- | --- | --- |
| B8 | 0.6 | 0.6 |  | 0.6 |
| B42 | 0.8 |  |  | 0.8 |
| n77 |  | 0.8 |  | 0.8 |

Table 4: ΔRIB of constituent 2 band combo.

| ΔRIB | B8+B42 | B8+n77 | B42+n77 | Proposed Value |
| --- | --- | --- | --- | --- |
| B8 | 0.2 | 0.2 |  | 0.2 |
| B42 | 0.5 |  |  | 0.5 |
| n77 |  | 0.5 |  | 0.5 |

Then the proposal for this 2LTE-1NR EN-DC is:

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-42_n77 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-42_n77 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

5.  REFSENS

As mentioned in section 3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.  Conclusion

This paper proposes study results of EN-DC 8-42_n77.

TP is shown in Annex.

7.  Reference

[1]  RP-191124  WID revision: EN-DC for 3 bands DL with 2 bands UL (2 LTE bands+ 1 NR band); rapporteur: Huawei

[2]  R4-1907459  Updated TR 37.716-11-11_V0.4.0_Rel16_DC band combo of 1 LTE band + 1 NR
 band

Annex.   Proposed text for TR37.716-21-11:

[Unchanged Parts Skipped]

5.1.x	DC_8-42_n77

### 5.1.x.1	Operating bands for DC_8-42_n77

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_8-42_n77 | CA_8-42 | n77 | No |

### 5.1.x.2	Configurations for DC_8-42_n77

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_8A-42A_n77A | DC_8A_n77A | CA_8A-42A | n77A |
| DC_8A-42C_n77A | DC_8A_n77A | CA_8A-42C | n77A |

### 5.1.x.3	Co-existence studies

The co-existence studies for the lower order 2DL/2UL DC configurations are already captured in TR 37.863-01-01. For the additional 3rd receiver band, there is no MSD issue for this DC configuration based on the following conditions.

- LTE Band 42 and NR Band n77 are synchronized.

- Simultaneous transmission of LTE Band 42 and NR Band n77 are not supported.

### 5.1.x.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.x.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-42_n77 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 5.1.x.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-42_n77 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

### 5.1.x.5	REFSENS requirements

As mentioned in 5.1.x.3, there is no need to specify additional MSD requirement for this UL DC configuration.

[Unchanged Parts Skipped]