3GPP TSG-RAN WG4 Meeting #92	R4-1908646

# Ljubljana, Slovenia,  26th Aug, 2019 - 30th Aug, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0048 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-02: Background on the EVM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | EVM measurement in the Annex F are not explained and documented which are quite important. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 1. Add the background on the FR1 EVM measurement in the Annex F in TS38.141-1 - 2.Add the background on the FR2 EVM measurement in Annex F in TS38.141-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Background information on the Annex F are missing in the TR38.817-02. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.4, 9.6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************************* BEGIN CHANGE 1 ********************************

6.5.4	Modulation Quality

The modulation quality requirement is defined by the difference between the measured carrier signal and a reference signal.  Modulation quality can be expressed Error Vector Magnitude (EVM).

The core requirement for the conducted modulation quality for FR1 requirement will be reused from conducted modulation quality requirement from 3GPP TS 37.104 [7] specification.  Although some aspects, such as the requirement level is reused other aspects such as EVM window and equalization algorithms will need to differ to suit NR aspects such as spectrum utilization, reference signal structure, and numerologies. The requirements apply per TAB connector.

A DM-RS pattern will be transmitted, and a standardized receiver will be used to mitigate some linear aspects of the EVM. With the removal of Common Reference Signals (CRS) present in LTE and the new design of demodulation reference signals (DM-RS) the specifics of pattern configurations are needed to be considered as part of the overall requirement conditions.  The overall link performance was studied in a fading channel and also in an AWGN channel for different DM-RS pattern designs.  In fading channels during operation (as opposed to just test operation), the simulations showed that a minimum DM-RS density configuration of comb 2 (every other subcarrier) in symbols 3 and 11 would be required to achieve similar performance with CRS in LTE.

Figure 6.5.4-1: DM-RS patterns for DL Single Layer Front Loaded and Front and Back Loaded (Pattern 1 and 2)

| EVM 8% for 64 QAM | EVM 3.5% for 256 QAM |
| --- | --- |

Figure 6.5.4-1: Throughput for PDSCH vs SNR simulation results [R4-1711162, Ericsson]

| EVM impacts on throughput performance, NR DMRS Pattern 1 | EVM impacts on throughput performance, NR DMRS Pattern 2 |
| --- | --- |

Figure 6.5.4-1: Throughput for PDCCH vs SNR simulation results [R4-1713672, NEC]

6.5.4.1	EVM measurement in the Annex

In the following section, background on the EVM measurement are introduced in details.

6.5.4.2	Example NRB Values

In the existing spec TS36.141, NRB=100 is based on LTE maximum channel bandwidth 20 MHz. For NR spec, it is agreed to use 100 MHz channel bandwidth with SCS 30 kHz for FR1 as example cases.

6.5.4.3	TX Power (RSTP) and OFDM Symbol TX power (OSTP)

As we know that RS TX power (RSTP) is mainly used for testing the DL RS power accuracy requirement as specified for UTRA and E-UTRA system, however for NR specification, DL RS power accuracy requirement is not specified anymore, therefore it is agreed to remove the RSTP in the Annex F.

The OFDM symbol TX power (OSTP) is required for testing Total power dynamic range. In the E-UTRA TS 36.141 specification, the RETP and OSTP is defined as following with considerations on the supported SCS.

RETP=Z't,f2Δf

where Δf is the subcarrier spacing in Hz.

In the E-UTRA TS 36.141 specification, from RETP the OFDM Symbol TX power (OSTP) is derived as follows:

Figure 6.5.4.3-1. LTE CRS pattern

As shown in Figure 6.5.4.3-1, CRS will spanning over all DL RBs. Therefore it is impossible to measure the single RB power at symbol containing the CRS signals or synchronization signals, thus it’s proposed to skip the symbols containing reference signals and synchronization signals to make sure that single RB power could be measured. However for NR system,  DMRS is used which will not spanning over all DL RBs, therefore RBs with DMRS could also be used for single RB power measurement and it’s agreed to use the 4th symbol to explicitly indicate which OFDM symbols should be used for single RB power measurement and whole RB power measurement.

From RETP the OFDM Symbol TX power (OSTP) is derived as follows:

OSTP=RETP

Where the summation accumulates NRBNscRB RETP values of the [4th] OFDM symbol. The [4th] (out of 14 OFDM symbols within a slot in case of normal CP length) contains exclusively PDSCH. NscRB=12

6.5.4.4	Number of DL frames in Averaged EVM

The averaged EVM with the minimum averaging length of at least 10ms is then achieved by further averaging of the  results

,

For FDD LTE, for Ndl is 10 subframes per frame, then Nframe=ceil(10/Ndl)=1

For TDD LTE without in-band NB-IoT and guardband NB-IoT operation, TDD configuration for test model is configuration 3. Then Ndl is 6 subframes per frame, then Nframe=ceil(10/Ndl)=2.

Table 6.5.4.4-1: Configurations of TDD eNB test models

| TDD Configuration | Downlink-to-Uplink Switch-point periodicity | Number of UL/DL sub-frames per radio frame (10 ms) |  | DwPTS | GP | UpPTS |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | DL | UL |  |  |  |
| 1 | 5 ms | 4 | 4 |  |  |  |
| 3 | 10ms | 6 | 3 |  |  |  |

For FR1 NR TDD configuration defined in Table 4.9.2.2-1 in TS38.141-01, here Ndl is the numeber of downlink slots, therefore considering different SCS and TDD periodicity for NR system, some updates are needed as following according to the TDD configuration in TS 38.141-01.

For NR system, for SCS 15kHz case,  Nframe=ceil(10/Ndl)=ceil(10/(3*2))=2

For NR system, for SCS 30kHz case, Nframe=ceil(10/Ndl)=ceil(10/(7*2*0.5))=2

For NR system, for SCS 60kHz case, Nframe=ceil(10/Ndl)=ceil(10/(14*2*0.25))=2

Where as TDD periodicity for NR system is specified as 5ms, then 2 times of downlink slots should be considered, meanwhile with the increasing SCS, the time duration per slot should also be scaled correspondingly.

Based on the above considerations, the formula for Nframe should be defined as following

EVM=1Nframek=1NframeEVMframe,k2

Nframe=10NdlFDD

Nframe=10NdlTDD

9.6.7	OTA modulation quality for FR2

The EVM requirement for FR2 shall apply also to the range of directions which is described in section 9.6.6.

The intention of the following description is to summarize the parameters used to determine appropriate EVM levels.

A DM-RS pattern will be transmitted, and a standardized receiver will be used to mitigate some linear aspects of the EVM. Additionally, common phase error compensation algorithm is also needed to be applied in addition to the simple equalization algorithm. For this, an additional reference signal is needed, the so-called phase tracking reference signal (PT-RS).

To balance between reference signal overhead and throughput a balance of DM-RS and PT-RS along with achievable EVM levels were studied.

PTRS every 4th OFDM symbol

PDCCH     DM-RS       PT-RS     PDSCH

Figure 9.6.7-1: TYPE-1 Pattern front loaded DM-RS (1 DM-RS symbol)

9.6.7.1	EVM measurement in the Annex

In the following section, background on the EVM measurement are introduced in details.

9.6.7.2	Example NRB Values

In the E-UTRA TS 36.141 specification, NRB=100 is based on LTE maximum channel bandwidth 20 MHz. For NR specification, it is agreed to use 100 MHz channel bandwidth with SCS 30 kHz for FR1 and 400 MHz channel bandwidth with 120 kHz for FR2 as example cases.

9.6.7.3	TX Power (RSTP) and OFDM Symbol TX power (OSTP)

The considerations in section 6.5.4.3 could be applied for FR2.

9.6.7.4	Number of DL frames in Averaged EVM

The averaged EVM with the minimum averaging length of at least 10ms is then achieved by further averaging of the  results

,

For FDD LTE, for Ndl is 10 subframes per frame, then Nframe=ceil(10/Ndl)=1

For TDD LTE without in-band NB-IoT and guardband NB-IoT operation, TDD configuration for test model is configuration 3. Then Ndl is 6 subframes per frame, then Nframe=ceil(10/Ndl)=2.

Table 9.6.7.4-1: Configurations of TDD eNB test models

| TDD Configuration | Downlink-to-Uplink Switch-point periodicity | Number of UL/DL sub-frames per radio frame (10 ms) |  | DwPTS | GP | UpPTS |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | DL | UL |  |  |  |
| 1 | 5 ms | 4 | 4 |  |  |  |
| 3 | 10ms | 6 | 3 |  |  |  |

For FR2 NR TDD configuration defined in Table 4.9.2.2-1 in TS38.141-02, here Ndl is the numeber of downlink slots, therefore for different SCS,some updates are needed as following:

For NR system, for SCS 60kHz case,  Nframe=ceil(10/Ndl)=ceil(10/(3*8*0.25))=2

For NR system, for SCS 120kHz case, Nframe=ceil(10/Ndl)=ceil(10/(7*8*0.125))=2

Where as TDD periodicity for NR system is specified as 1.25ms, then 8 times of downlink slots should be considered, meanwhile with the increasing SCS, time duration per slot should also be scaled correspondingly.

Based on the above considerations, the formula for Nframe should be defined as following

EVM=1Nframek=1NframeEVMframe,k2

Nframe=10NdlTDD

NdlTDD is the downlink slots per frame.

9.6.7.5	TDD EVM Measurements:

Moving average length 19 from E-UTRA specification is agreed to be reuse for FR2 in NR.

9.6.7.6	EVM window

For the configurations, the example case for FR1 is 400MHz BW with 120kHz SCS, then sampling rate should be  491.52Msps, therefore is 3 where is 64 for Tc (1/480kHz*4096).

is on Tf=144 () within  the CP of length 288 ()  (in OFDM symbol    or  )

is on Tf=400 () within  the CP of length 352 ()  (in OFDM symbol   or  ).

******************************* END OF CHANGE 1 ********************************