3GPP TSG-RAN WG4 Meeting #92	R4-1908695

Ljubljana, Slovenia, 26 - 30 August, 2019

Agenda item:	10.10.2

Source:	Dish Network

Title:	TP for TR38.716-03-01: Requirements for CA_n66A-n70A-n71A, CA_n66B-n70A-n71A, and CA_n66(2A)-n70A-n71A

Document for:	Approval

# 1	Introduction

This TP captures the requirements for CA_n66A-n70A-n71A, CA_n66B-n70A-n71A, and CA_ n66(2A)-n70A-n71A.

# 2	TP for TR38.716-03-01

## 6.X	CA_n66-n70-n71

#### 6.X.1		Operating bands for CA

# Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR CA BAnd | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n66-n70-n71 | n66 | 1710 | – | 1780 | 2110 | – | 2200 | FDD |
|  | n70 | 1695 | – | 1710 | 1995 | – | 2020 | FDD |
|  | n71 | 663 | – | 698 | 617 | – | 652 | FDD |

#### 6.X.2		Channel bandwidths per operating band for CA

# Table 6.X.2-1: Supported channel bandwidths per CA combination for 3DL Inter-band CA

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n66A-n70A-n71A | - | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
|  |  | n70 | 15 | Yes | Yes | Yes | Yes1 | Yes1 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes1 | Yes1 |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes1 | Yes1 |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66B-n70A-n71A | - | n66 | See CA_n66B Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n70 | 15 | Yes | Yes | Yes | Yes1 | Yes1 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes1 | Yes1 |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes1 | Yes1 |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n70A-n71A |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n70 | 15 | Yes | Yes | Yes | Yes1 | Yes1 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes1 | Yes1 |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes1 | Yes1 |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.X.3		UE co-existence studies

Table 6.X..3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n66-n70-n71.

# Table 6.X.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 |  |  |
| n70 | 1695 | 1710 | 1995 | 2020 | 3390 | 3420 | 5085 | 5130 |  |  |
| n71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 |  |  |

Based on the table above, there is 3rd harmonic relation between n71 UL and n70 DL. However, this relation is already specified in CA_n70A-n71A so no additional requirements are needed for higher order CA.

# Table 6.X.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 |  |  |
| n70 | 1695 | 1710 | 1995 | 2020 | 3990 | 4040 | 5985 | 6060 |  |  |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 |  |  |

Based on the table above, the is no harmonic mixing relation.

#### 6.X.4		∆TIB and ∆RIB values

For CA_n66-n70-n71, the TIB,c and RIB,c values are given in the tables below.

# Table 6.4.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n66-n70-n71 | n66 | 0.5 |
|  | n70 | 0.5 |
|  | n71 | 0.6 |

# Table 6.1.x.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n66-n70-n71 | n66 | 0 |
|  | n70 | 0 |
|  | n71 | 0 |

#### 6.4.1.5		REFSENS requirements

Specific REFSENS requirement due to 3rd harmonic for CA_n70-n71 has already been accounted in two band CA combination CA_n70A-n71A.

# 4	References