3GPP TSG-RAN WG4 Meeting #92	R4-1908757

Ljubljana, EU, 26 – 30 August 2019

Source:	Ericsson

Title:	TP to TR 38.820: PA parameter dependency for different example frequencies in subclause 5.4.3

Agenda item:	11.4.7

Document for:	Approval

# Introduction

In [1], the trends for power amplifier semi-conductor technologies in terms of achievable peak saturated power and power added efficiency was presented. In addition, scaling principles for PA RF power over frequency when considering BS based on an antenna array which is the logical BS type for 7-24 GHz instead of a single PA was discussed in [2]. This is necessary to support scaling of PA performance metrics over frequency.

Due to complex dependencies between output power, ACLR/unwanted emission and efficiency, it is important to properly document theses aspects including performance and feasibility during the SI to ensure that proper considerations when WI for standardizing specific frequency bands in 7-24 GHz range is taken. It was described in detail in [3].

The intention with this paper is to propose text to capture the PA dependencies in relation to power, ACLR and PAE when BS operating point is concerned.

# Proposal

It is proposed that the attached text proposal is included in TR 38.820 [4].

# References

[1]	R4-1900832, “7-24 GHz technical aspects and considerations”, Ericsson

[2] 	R4-1906184, “Elaboration on 7-24 GHz PA power scaling and AAS dependency”, Ericsson

[3]	R4-1906180, “Further elaboration on 7-24 GHz PA dependencies for different example frequencies”, Ericsson

[4]	TR 38.820, “7-24 GHz frequency range”

TEXT PROPOSAL:

5.4.3	PA Output power, ACLR and PAE dependencies

The current technological capability to achieve a certain average output power level at a prescribed ACLR level for power amplifiers operating in a weakly non-linear regime (e.g. in back-off), the relation between the 3rd order intercept point (IP3) and ACLR can be used. For an OFDM signal with a large number of sub-carriers (more than 10) the average output power as function of targeted ACLR can be calculated from:

Pout=IP3 - 12ACLR + 1.3 - 6

If the IP3 levels are not available they can be estimated from the 1dB gain compression point, and the average output power can then be calculated as:

Pout=9.6 +P1dB - 12ACLR + 1.3 - 6

In figure 5.4.3-1 the peak power added efficiency is plotted versus 1-dB output power compression point (P1dB) for amplifiers operating between 10 GHz to 30 GHz. Considering available data (see figure 5.4.1-1 and 5.4.1-2), a reasonable trade-off between linearity and efficiency is achieved around a P1 dB of 100 mW (with associated peak PAE of > 40%). This observation holds regardless of semiconductor technology. In figure 5.4.3-2 the effect on achievable ACLR of the efficiency - linearity trade-off is investigated. Three typical points from the maximum P1dB-PAE trajectory is chosen: P1dB = 10 dBm, 17 dBm and 20 dBm. For each point the ACLR is computed as function of average output power. The PAE numbers given in the figure are indicative of the average PAE resulting from the peak PAE for these example points (4%, 5% and 6%) under the assumption that the amplifiers operate in class-A.

Figure 5.4.3-1: Peak power added efficiency versus 1 dB gain compression point for power amplifiers using Silicon transistors (Left) and GaAs and GaN transistors (right). The data points show power amplifiers operating in the 10 GHz to 30 GHz frequency range

Figure 5.4.3-2: Average output power as function of ACLR with PAE as parameter

To verify the relations between output power vs. ACLR, we re-use to the results and approach derived earlier in the work for FR2 based on simulations of a GaN and a CMOS PA, both designed for operation around 30 GHz. Similar to the FR2 approach where the PA models at 45 GHz and 70 GHz FR2 proxy frequencies used the same nonlinear characteristics but with the output power scaled as -20 dB/decade while PAE scaled as -6 dB/decade, the same approach has been used for 7-24 GHz example frequencies. The results for relation between output power and ACLR as well as ACLR vs PAE for example frequencies of 10 GHz, 15 GHz and 20 GHz are presented in Figure 5.4.3-3 and Figure 5.4.3-4 respectively, but now with PAE scaling as -5 dB/decade as supported by the larger updated data set used in this work.

Figure 5.4.3-3: ACLR versus output power at 10, 15, and 20 GHz example frequencies, and 30 GHz based on 30 GHz PA model for reference

Figure 5.4.3-4: PAE versus output power at 10, 15, and 20 GHz example frequencies, and 30 GHz based on 30 GHz PA model for reference. Constant PAE for CMOS and GaN PA, respectively, also shown

As discussed in sub-clause 5.4.2, assuming the DC power as well as the RF power associated with the PAs should be constant per antenna array unit area, the PAE scaling of -5 dB/decade can be exploited to a yield a -10 dB/decade ACLR scaling. Thus, from 30 GHz to 20,15,10 GHz, PAE should improve by 0.9,1.5, 2.4 dB and consequently the ACLR can be improved by 1.8, 3.0, 4.8 dB.

Considering the 28 dB ACLR for 30 GHz, the possible ACLR for 7-24 GHz example frequencies can be considered as in Table 5.4.3-1.

Table 5.4.3-1: BS ACLR as function of frequency

| Example frequency (GHz) | ACLR (dB) |
| --- | --- |
| 10 | ~33 dB |
| 15 | ~31 dB |
| 20 | ~30 dB |

The analysis in this paper outlines the achievable raw linearity of the PA and does not consider any linearization. The analysis is applicable for BS PA while for UE PA, a different selection of operating point could be made where higher PAE is achieved given constrains on battery consumptions, although at the expense of further reduced ACLR.