3GPP TSG-RAN WG4 Meeting #92	R4-1908886

# Ljubljana, Slovenia, August 26th-30th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | - | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.141-2 – PUSCH requirements with CP-OFDM for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update the requirements for PUSCH FR2 with CP-OFDM |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the requirements for PUSCH FR2 with CP-OFDM tests based on simulation results |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No up to date requirements for PUSCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.104 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<start of the change>

##### 8.2.1.5.2	Test requirement for BS type 2-O

The throughput measured according to subclause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in table 8.2.1.5.2-1 to 8.2.1.5.2-5.

Table 8.2.1.5.2-1: Test requirements for PUSCH, 50 MHz Channel Bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-1 | pos0 | No | [-1.2] |
|  |  |  |  |  | G-FR2-A3-13 | pos1 | No | [-1.4] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-1 | pos0 | Yes | [12.6] |
|  |  |  |  |  |  |  | No | [12.3] |
|  |  |  |  |  | G-FR2-A4-11 | pos1 | Yes | [11.3] |
|  |  |  |  |  |  |  | No | [11.7] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-1 | pos0 | Yes | [14.3] |
|  |  |  |  |  |  |  | No | [13.8] |
|  |  |  |  |  | G-FR2-A5-6 | pos1 | Yes | [13.8] |
|  |  |  |  |  |  |  | No | [13.8] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-6 | pos0 | No | [2.6] |
|  |  |  |  |  | G-FR2-A3-18 | pos1 | No | [2.2] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-6 | pos0 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
|  |  |  |  |  | G-FR2-A4-16 | pos1 | Yes | [20.4] |
|  |  |  |  |  |  |  | No | [19.2] |

Table 8.2.1.5.2-2: Test requirements for PUSCH, 100 MHz Channel Bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-2 | pos0 | No | [-1.3] |
|  |  |  |  |  | G-FR2-A3-14 | pos1 | No | [-1.6] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-2 | pos0 | Yes | [12.8] |
|  |  |  |  |  |  |  | No | [12.0] |
|  |  |  |  |  | G-FR2-A4-12 | pos1 | Yes | [11.8] |
|  |  |  |  |  |  |  | No | [11.3] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-2 | pos0 | Yes | [14.8] |
|  |  |  |  |  |  |  | No | [14.1] |
|  |  |  |  |  | G-FR2-A5-7 | pos1 | Yes | [14.3] |
|  |  |  |  |  |  |  | No | [13.8] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-7 | pos0 | No | [2.5] |
|  |  |  |  |  | G-FR2-A3-19 | pos1 | No | [2.3] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-7 | pos0 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
|  |  |  |  |  | G-FR2-A4-17 | pos1 | Yes | [19.6] |
|  |  |  |  |  |  |  | No | [19.3] |

Table 8.2.1.5.2-3: Test requirements for PUSCH, 50 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-3 | pos0 | No | [-1.0] |
|  |  |  |  |  | G-FR2-A3-15 | pos1 | No | [-1.3] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-3 | pos0 | Yes | [12.2] |
|  |  |  |  |  |  |  | No | [11.7] |
|  |  |  |  |  | G-FR2-A4-13 | pos1 | Yes | [11.5] |
|  |  |  |  |  |  |  | No | [11.2] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-3 | pos0 | Yes | [14.3] |
|  |  |  |  |  |  |  | No | [13.7] |
|  |  |  |  |  | G-FR2-A5-8 | pos1 | Yes | [13.8] |
|  |  |  |  |  |  |  | No | [13.6] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-8 | pos0 | No | [2.4] |
|  |  |  |  |  | G-FR2-A3-20 | pos1 | No | [2.3] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-8 | pos0 | Yes | [21.9] |
|  |  |  |  |  |  |  | No | [19.6] |
|  |  |  |  |  | G-FR2-A4-18 | Pos1 | Yes | [20.4] |
|  |  |  |  |  |  |  | No | [18.6] |

Table 8.2.1.5.2-4: Test requirements for PUSCH, 100 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-4 | pos0 | No | [-1.6] |
|  |  |  |  |  | G-FR2-A3-16 | pos1 | No | [-1.7] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-4 | pos0 | Yes | [12.5] |
|  |  |  |  |  |  |  | No | [11.2] |
|  |  |  |  |  | G-FR2-A4-14 | pos1 | Yes | [11.7] |
|  |  |  |  |  |  |  | No | [11.2] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-4 | pos0 | Yes | [14.1] |
|  |  |  |  |  |  |  | No | [13.6] |
|  |  |  |  |  | G-FR2-A5-9 | pos1 | Yes | [14.0] |
|  |  |  |  |  |  |  | No | [13.5] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-9 | pos0 | No | [2.5] |
|  |  |  |  |  | G-FR2-A3-21 | pos1 | No | [2.2] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-9 | pos0 | Yes | [21.9] |
|  |  |  |  |  |  |  | No | [20.1] |
|  |  |  |  |  | G-FR2-A4-19 | pos1 | Yes | [19.6] |
|  |  |  |  |  |  |  | No | [18.9] |

Table 8.2.1.5.2-5: Test requirements for PUSCH, 200 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-5 | pos0 | No | [-1.3] |
|  |  |  |  |  | G-FR2-A3-17 | pos1 | No | [-1.6] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-5 | pos0 | Yes | [11.9] |
|  |  |  |  |  |  |  | No | [11.6] |
|  |  |  |  |  | G-FR2-A4-15 | pos1 | Yes | [11.8] |
|  |  |  |  |  |  |  | No | [11.5] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-5 | pos0 | Yes | [14.7] |
|  |  |  |  |  |  |  | No | [14.1] |
|  |  |  |  |  | G-FR2-A5-10 | pos1 | Yes | [14.3] |
|  |  |  |  |  |  |  | No | [14.0] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-10 | pos0 | No | [2.5] |
|  |  |  |  |  | G-FR2-A3-22 | pos1 | No | [2.2] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-10 | pos0 | Yes | [22.3] |
|  |  |  |  |  |  |  | No | [21.5] |
|  |  |  |  |  | G-FR2-A4-20 | pos1 | Yes | [19.8] |
|  |  |  |  |  |  |  | No | [19.2] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

<end of the change>