3GPP TSG-RAN WG4 Meeting#92                	R4-1909364

Ljubljana, Slovenia, August 26 - 30, 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for RSS based RSRP measurements

Agenda item:	8.11.3.5

Document for:	Discussion and Agreement

Introduction

At RAN4 #90-Bis simulation assumptions for performance evaluation of RSS based RSRP measurements were revised to [1].

This contribution is an update of [2] and reports simulation results for an extended subset of scenarios, in particular for RSS-based RSRP measurement performance for non-colliding and colliding RSS scenarios as well as a performance comparison to CRS-based RSRP measurements for non-colliding scenarios. In particular, further results for shorter measurement periods of 160 ms and 320 ms, for UE with 2 RX antennas as well as for EPA 1 Hz and ETU 1Hz propagation channels are added. In section 2 the revised simulation assumptions are summarized and measurement procedure details are described. Section 3 depicts simulation results and section 4 a discussion. Section 5 draws the conclusion. Changes versus [2] are marked in blue.

RSS design and simulation assumptions

RSS reference signals were introduced for MTC in Rel-15 with the objective to provide improved performance for UEs in idle mode in (e)DRX operation, waking up and rapidly synchronizing to the serving cell thus achieving lower energy consumption compared to the CRS based measurement. The RSS signal is defined in TS 36.211 in subclause 6.11.3 and the mapping to RE’s in subclause 6.11.3.2. The mapping to RE’s for one PRB and 1 subframe is shown in Figure 1. In fact, RSS reference signals are much denser than CRS reference signals which is expected to also improve the RSRP measurement performance which is investigated in this contribution.

Figure 1: RSS mapping to RE’s (shown 1 PRB and 1 subframe, 2 antenna ports) [3]

In 1 subframe, there are 120 RE’s for RSS (yellow) per PRB. All CRS reference signals are punctured. In the specified RSS measurement bandwidth of 2 consecutive PRBs, there are hence 240 RE’s for RSS.

The revised simulation assumptions are summarized in Table 1 and further comments included.

Table 1: Revised simulation parameters for RSS based RSRP measurement studies [1]

| Parameters | Value | Comments |
| --- | --- | --- |
| RSS bandwidth | 2 consecutive resource blocks |  |
| Sample duration | {1, 2} subframes | Implementation dependent (NOTE 4) Equals term ‘RSS duration’ in this contribution. Results are also included for 8 and 40 subframes (previous assumption). 120 RS REs per PRB in each of the subframes were used. |
| RSS periodicity | 160 ms | Option 1: non-colliding RSS Option 2: colliding RSS |
| Ratio of RSS RE energy to CRS RE energy | 0 dB |  |
| Power level of neighbour cell relative to serving cell | {0, -3} dB |  |
| Measurement bandwidth | 2 resource blocks | RSRP measured over the 2 RSS PRBs. RSRP measured over 6 central PRBs for CRS. |
| System bandwidth | 6 resource blocks |  |
| L1 measurement period | Single shot (160 ms), 320 ms, 480 ms and 800 ms | Single shot measurement includes time domain averaging over the RSS duration (i.e. over 1 or 2 subframes and for previous results 8 or 40 subframes, respectively), that means 1 sample is used (equals 160 ms MP). |
| Measurement sampling rate |  | Implementation dependent (NOTE 1) |
| L3 filtering | Disabled |  |
| Transmit antenna | 1 |  |
| Receive antennas | 1, 2 | Single and double Rx branches, respectively for previous simulation results (8 and 40 subframes) Single and double RX antenna for 1 and 2 subframes. |
| Mobility | Stationary UEs, mobile UEs |  |
| Propagation conditions | AWGN, ETU and EPA | For comparison to CRS based RSRP performance, only EPA 5 is used. |
| Doppler Frequency for stationary UEs: ETU and EPA | 1 Hz and 1 Hz, respectively | results not included |
| Doppler Frequency for mobile UEs: ETU and EPA | ETU 30 Hz and EPA 5 Hz, ETU 1 Hz and EPA 1 Hz | results included |
| Channel estimation techniques | Measurement averaging techniques used currently for category M1 | Implementation dependent (NOTE 2) |
| Number of RSS subframes used for measurement | number of subframes per RSS duration (1, 2, 8 or 40) * number of samples (1, 2, 3 or 5) | total subframes per L1 measurement period For previous simulations (8,40 subframes) only 1, 3 and 5 samples were evaluated. |
| CP length | Normal |  |
| Carrier frequency | 2 GHz |  |
| Simulation duration | 5000 RSRP samples except section 3.1: 2500 RSRP samples | Equals following number of RSS instances: - 5 000 for single shot over 1 subframe - 10 000 for single shot over 2 subframes - 25 000 for 5 samples average over 1 subframe - 50 000 for 5 samples average over 2 subframes For results in section 3.1, number of RSS instances: - 20 000 for single shot over 8 subframe - 100 000 for single shot over 40 subframes - 60 000 for 3 samples average over 8 subframe - 300 000 for 3 samples average over 40 subframes - 100 000 for 5 samples average over 8 subframe - 500 000 for 5 samples average over 40 subframes |
| SNR [dB] | -15, -12, -9, -6, -3, 0, 5 | Setting for 1 and 2 subframes For 8, 40 subframes: -15, -12.5, -10, -7.5, -5, 0, 5 AWGN noise |
| Performance Metrics | Delta RSRP (= estimated RSRP – ideal RSRP) | The ideal RSRP is the true value that does not include any channel estimation noise. |
|  | RSRP error = MAX(Delta RSRP(CDF=95%); -Delta RSRP(CDF=5%)); | used for previous and revised simulation assumptions |
| NOTE 1: Companies are requested to provide the details of the measurement sampling rate for interpretation and comparison of the results NOTE 2: Companies are requested to provide the details of the RS averaging techniques for interpretation and comparison of the results. NOTE 3: The number of RSS subframes used for measurement is implementation dependent, and can range from 1 to the RSS duration. Companies are requested to provide the details of the RSS subframes used for interpretation and comparison of the results. Note 4: The presented results shall be based on at least 10 000 RSS instances. |  |  |

Simulation results

Non-colliding RSS scenario, results for 8 and 40 subframes

In this section simulations results are reported for 1 RX antenna and 2 RX antennas for the simulation settings agreed at RAN4#90 and depicted in [2]. In particular the delta RSRP has been evaluated and the 90% percentile around the median of the distribution is recorded, i.e. the maximum absolute value of the 5% and 95% points of the delta RSRP CDF are selected.

1 RX antenna

The following results were obtained for single RX antenna, shown in Tables 2 to 4 for AWGN, EPA 5 Hz and ETU 30 Hz propagation channels.

Table 2: RSRP error [dB] for AWGN.

| Propagation channel | SNR [dB] | RSS duration: 8 subframes |  |  | RSS duration: 40 subframes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms |
| AWGN | -15 | 4.28 | 3.11 | 2.15 | 1.92 | 1.07 | 0.86 |
|  | -12.5 | 2.71 | 1.59 | 1.20 | 1.13 | 0.63 | 0.51 |
|  | -10 | 1.49 | 0.87 | 0.65 | 0.66 | 0.37 | 0.30 |
|  | -7.5 | 0.90 | 0.54 | 0.39 | 0.41 | 0.23 | 0.17 |
|  | -5 | 0.61 | 0.35 | 0.27 | 0.25 | 0.14 | 0.11 |
|  | 0 | 0.27 | 0.15 | 0.12 | 0.12 | 0.07 | 0.06 |
|  | 5 | 0.14 | 0.08 | 0.06 | 0.06 | 0.04 | 0.03 |

Table 3: RSRP error [dB] for EPA 5 Hz.

| Propagation channel | SNR [dB] | RSS duration: 8 subframes |  |  | RSS duration: 40 subframes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms |
| EPA 5 Hz | -15 | 10.85 | 3.56 | 2.35 | 5.22 | 1.58 | 1.12 |
|  | -12.5 | 7.93 | 1.93 | 1.53 | 3.45 | 0.93 | 0.61 |
|  | -10 | 5.83 | 1.26 | 0.82 | 1.96 | 0.55 | 0.36 |
|  | -7.5 | 3.75 | 0.77 | 0.49 | 1.09 | 0.33 | 0.21 |
|  | -5 | 2.4 | 0.49 | 0.32 | 0.61 | 0.19 | 0.14 |
|  | 0 | 0.67 | 0.21 | 0.13 | 0.24 | 0.09 | 0.06 |
|  | 5 | 0.3 | 0.1 | 0.07 | 0.1 | 0.04 | 0.03 |

Table 4: RSRP error [dB] for ETU 30 Hz.

| Propagation channel | SNR [dB] | RSS duration: 8 subframes |  |  | RSS duration: 40 subframes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms |
| ETU 30 Hz | -15 | 6.05 | 3.23 | 2.47 | 2.53 | 1.2 | 0.97 |
|  | -12.5 | 4.19 | 1.96 | 1.36 | 1.47 | 0.75 | 0.51 |
|  | -10 | 2.5 | 1.01 | 0.81 | 0.88 | 0.43 | 0.33 |
|  | -7.5 | 1.69 | 0.67 | 0.48 | 0.51 | 0.27 | 0.23 |
|  | -5 | 1.07 | 0.42 | 0.31 | 0.36 | 0.19 | 0.16 |
|  | 0 | 0.52 | 0.23 | 0.17 | 0.2 | 0.11 | 0.08 |
|  | 5 | 0.31 | 0.15 | 0.12 | 0.16 | 0.08 | 0.07 |

In addition to the RSRP error, delta RSRP CDF curves for EPA 5 Hz and ETU 30 Hz for RSS duration of 8 subframes and 40 subframes are shown in Figure 2 and Figure 3 for the SNR points -15 dB, -10 dB and -5 dB.

Figure 2: CDF curves for delta RSRP (EPA 5 Hz and ETU 30 Hz for RSS duration=8 subframes, 1 RX)

Figure 3: CDF curves for delta RSRP (EPA 5 Hz and ETU 30 Hz for RSS duration=40 subframes, 1 RX)

2 RX antennas

The following results were obtained for two RX antennas, shown in Tables 5 to 7 for AWGN, EPA 5 Hz and ETU 30 Hz propagation channels.

Table 5: RSRP error [dB] for AWGN.

| Propagation channel | SNR [dB] | RSS duration: 8 subframes |  |  | RSS duration: 40 subframes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms |
| AWGN | -15 | 3.40 | 2.19 | 1.90 | 1.78 | 1.11 | 0.85 |
|  | -12.5 | 2.17 | 1.37 | 1.11 | 1.09 | 0.69 | 0.54 |
|  | -10 | 1.42 | 0.85 | 0.71 | 0.68 | 0.42 | 0.31 |
|  | -7.5 | 0.96 | 0.56 | 0.45 | 0.43 | 0.25 | 0.20 |
|  | -5 | 0.61 | 0.37 | 0.29 | 0.29 | 0.17 | 0.13 |
|  | 0 | 0.30 | 0.17 | 0.14 | 0.14 | 0.08 | 0.06 |
|  | 5 | 0.16 | 0.09 | 0.07 | 0.07 | 0.04 | 0.03 |

Table 6: RSRP error [dB] for EPA 5 Hz.

| Propagation channel | SNR [dB] | RSS duration: 8 subframes |  |  | RSS duration: 40 subframes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms |
| EPA 5 Hz | -15 | 5.67 | 2.26 | 1.47 | 2.51 | 0.97 | 0.71 |
|  | -12.5 | 3.66 | 1.33 | 0.94 | 1.42 | 0.62 | 0.48 |
|  | -10 | 1.96 | 0.84 | 0.61 | 0.82 | 0.34 | 0.25 |
|  | -7.5 | 1.22 | 0.51 | 0.36 | 0.49 | 0.21 | 0.16 |
|  | -5 | 0.71 | 0.32 | 0.23 | 0.3 | 0.14 | 0.1 |
|  | 0 | 0.31 | 0.15 | 0.12 | 0.13 | 0.07 | 0.05 |
|  | 5 | 0.16 | 0.07 | 0.06 | 0.06 | 0.04 | 0.03 |

Table 7: RSRP error [dB] for ETU 30 Hz.

| Propagation channel | SNR [dB] | RSS duration: 8 subframes |  |  | RSS duration: 40 subframes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms |
| ETU 30 Hz | -15 | 4.27 | 2.08 | 1.63 | 1.65 | 0.91 | 0.71 |
|  | -12.5 | 2.6 | 1.2 | 0.97 | 1.01 | 0.54 | 0.43 |
|  | -10 | 1.57 | 0.7 | 0.64 | 0.59 | 0.36 | 0.27 |
|  | -7.5 | 0.91 | 0.45 | 0.4 | 0.39 | 0.22 | 0.17 |
|  | -5 | 0.63 | 0.33 | 0.26 | 0.27 | 0.16 | 0.12 |
|  | 0 | 0.33 | 0.18 | 0.14 | 0.15 | 0.1 | 0.07 |
|  | 5 | 0.21 | 0.12 | 0.1 | 0.12 | 0.07 | 0.06 |

In addition to the RSRP error, delta RSRP CDF curves for EPA 5 Hz and ETU 30 Hz for RSS duration of 8 subframes and 40 subframes are shown in Figure 4 and Figure 5 for the SNR points -15 dB, -10 dB and -5 dB.

Figure 4: CDF curves for delta RSRP (EPA 5 Hz and ETU 30 Hz for RSS duration=8 subframes, 2 RX)

Figure 5: CDF curves for delta RSRP (EPA 5 Hz and ETU 30 Hz for RSS duration=40 subframes, 2 RX)

Non-colliding RSS scenario, results for 1 and 2 subframes

In this section, simulations results are reported for the non-colliding RSS scenario with simulation settings agreed in [1] for 1 RX antenna.

1 RX antenna

The following results were obtained for single RX antenna, shown in Tables 8 to 12 for AWGN, EPA 5 Hz, ETU 30 Hz, EPA 1 Hz and ETU 1 Hz propagation channels.

Table 8: RSRP error [dB] for AWGN.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  |  | RSS duration: 2 subframes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 measurement period |  |  |  | L1 measurement period |  |  |  |
|  |  | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) |
| AWGN | -15 | 6.11 | 4.95 | 4.38 | 4.28 | 4.98 | 4.36 | 4.37 | 4.04 |
|  | -12 | 4.55 | 4.38 | 4.07 | 3.31 | 4.64 | 3.57 | 2.73 | 2.24 |
|  | -9 | 3.70 | 2.77 | 2.22 | 1.53 | 2.70 | 1.88 | 1.53 | 1.14 |
|  | -6 | 2.17 | 1.51 | 1.13 | 0.92 | 1.48 | 1.02 | 0.83 | 0.59 |
|  | -3 | - | 0.85 | 0.72 | - | - | 0.59 | 0.49 | - |
|  | 0 | 0.75 | 0.54 | 0.45 | 0.33 | 0.54 | 0.38 | 0.32 | 0.24 |
|  | 5 | 0.39 | - | - | 0.18 | 0.28 | - | - | 0.12 |

Table 9: RSRP error [dB] for EPA 5 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  |  | RSS duration: 2 subframes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 measurement period |  |  |  | L1 measurement period |  |  |  |
|  |  | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) |
| EPA 5 Hz | -15 | 15.53 | 9.16 | 6.88 | 4.88 | 13.99 | 7.75 | 5.37 | 3.76 |
|  | -12 | 12.50 | 6.47 | 4.30 | 3.31 | 10.78 | 4.80 | 3.22 | 2.26 |
|  | -9 | 8.95 | 4.01 | 2.72 | 1.95 | 8.62 | 2.92 | 1.94 | 1.30 |
|  | -6 | 6.83 | 2.26 | 1.79 | 1.06 | 5.43 | 1.66 | 1.12 | 0.81 |
|  | -3 | - | 1.27 | 0.99 | - | - | 0.97 | 0.65 | - |
|  | 0 | 2.35 | 0.84 | 0.58 | 0.38 | 1.56 | 0.55 | 0.39 | 0.27 |
|  | 5 | 0.89 | - | - | 0.19 | 0.62 | - | - | 0.14 |

Table 10: RSRP error [dB] for ETU 30 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  |  | RSS duration: 2 subframes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 measurement period |  |  |  | L1 measurement period |  |  |  |
|  |  | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) |
| ETU 30 Hz | -15 | 11.35 | 7.64 | 6.06 | 4.45 | 9.84 | 6.09 | 4.65 | 3.83 |
|  | -12 | 8.57 | 4.90 | 3.81 | 3.07 | 7.17 | 3.88 | 3.21 | 2.57 |
|  | -9 | 5.73 | 3.01 | 2.72 | 2.05 | 4.47 | 2.47 | 2.00 | 1.24 |
|  | -6 | 3.60 | 2.00 | 1.58 | 1.10 | 2.39 | 1.40 | 1.05 | 0.75 |
|  | -3 | - | 1.30 | 0.92 | - | - | 0.88 | 0.67 | - |
|  | 0 | 1.43 | 0.74 | 0.60 | 0.43 | 1.00 | 0.57 | 0.41 | 0.31 |
|  | 5 | 0.71 | - | - | 0.25 | 0.55 | - | - | 0.17 |

Table 11: RSRP error [dB] for EPA 1 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  |  | RSS duration: 2 subframes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 measurement period |  |  |  | L1 measurement period |  |  |  |
|  |  | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) |
| EPA 1 Hz | -15 | 15.19 | 10.67 | 8.30 | 6.24 | 14.41 | 9.18 | 6.97 | 5.13 |
|  | -12 | 12.94 | 7.45 | 5.85 | 3.85 | 11.15 | 5.94 | 4.35 | 2.73 |
|  | -9 | 9.93 | 5.16 | 3.47 | 2.36 | 8.53 | 3.85 | 2.38 | 1.58 |
|  | -6 | 6.69 | 2.94 | 1.92 | 1.30 | 5.23 | 2.10 | 1.40 | 0.89 |
|  | -3 | 4.03 | 1.64 | 1.13 | 0.69 | 3.28 | 1.14 | 0.77 | 0.54 |
|  | 0 | 2.21 | 0.90 | 0.67 | 0.44 | 1.73 | 0.68 | 0.48 | 0.33 |
|  | 5 | - | - | - | - | - | - | - | - |

Table 12: RSRP error [dB] for ETU 1 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  |  | RSS duration: 2 subframes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 measurement period |  |  |  | L1 measurement period |  |  |  |
|  |  | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) |
| ETU 1 Hz | -15 | 11.40 | 8.49 | 6.88 | 5.58 | 9.94 | 7.09 | 5.61 | 4.04 |
|  | -12 | 8.74 | 5.84 | 4.37 | 3.25 | 7.22 | 4.56 | 3.43 | 2.57 |
|  | -9 | 5.67 | 3.57 | 3.06 | 2.23 | 4.36 | 2.62 | 2.19 | 1.63 |
|  | -6 | 3.44 | 2.42 | 1.76 | 1.40 | 2.63 | 1.69 | 1.21 | 0.87 |
|  | -3 | 2.09 | 1.39 | 1.04 | 0.81 | 1.65 | 1.01 | 0.71 | 0.51 |
|  | 0 | 1.36 | 0.89 | 0.62 | 0.53 | 0.99 | 0.65 | 0.48 | 0.35 |
|  | 5 | - | - | - | - | - | - | - | - |

In addition to the RSRP error, delta RSRP CDF curves for EPA 5 Hz and ETU 30 Hz for RSS duration of 1 subframe and 2 subframes are shown in Figure 6 and Figure 7 for the SNR points -15 dB, -12 dB, -9 dB and -6 dB.

Figure 6: CDF curves for delta RSRP (EPA 5 Hz and ETU 30 Hz for RSS duration=1 subframe, 1 RX)

Figure 7: CDF curves for delta RSRP (EPA 5 Hz and ETU 30 Hz for RSS duration=2 subframes, 1 RX)

2 RX antennas

The following results were obtained for two RX antennas, shown in Tables 13 to 15 for AWGN, EPA 5 Hz and ETU 30 Hz propagation channels.

Table 13: RSRP error [dB] for AWGN.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  |  | RSS duration: 2 subframes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 measurement period |  |  |  | L1 measurement period |  |  |  |
|  |  | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) |
| AWGN | -15 | 6.61 | 5.36 | 4.80 | 3,95 | 5.42 | 4.37 | 3.74 | 3,08 |
|  | -12 | 4.39 | 3.48 | 3.01 | 2,51 | 3.50 | 2.66 | 2.30 | 1,97 |
|  | -9 | 2.89 | 2.18 | 1.83 | 1,45 | 2.18 | 1.68 | 1.36 | 1,11 |
|  | -6 | 1.85 | 1.35 | 1.15 | 0,92 | 1.38 | 1.00 | 0.84 | 0,68 |
|  | -3 | 1.21 | 0.91 | 0.74 | 0,58 | 0.91 | 0.63 | 0.53 | 0,41 |
|  | 0 | 0.82 | 0.59 | 0.50 | 0,38 | 0.60 | 0.42 | 0.34 | 0,28 |
|  | 5 | - | - | - | - | - | - | - | - |

Table 14: RSRP error [dB] for EPA 5 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  |  | RSS duration: 2 subframes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 measurement period |  |  |  | L1 measurement period |  |  |  |
|  |  | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) |
| EPA 5 Hz | -15 | 9.89 | 6.45 | 5.24 | 4,06 | 8.61 | 5.13 | 3.94 | 3,00 |
|  | -12 | 7.10 | 4.05 | 3.18 | 2,28 | 5.71 | 3.08 | 2.18 | 1,72 |
|  | -9 | 4.63 | 2.36 | 1.75 | 1,27 | 3.38 | 1.73 | 1.26 | 0,94 |
|  | -6 | 2.62 | 1.28 | 1.02 | 0,74 | 1.93 | 0.94 | 0.73 | 0,59 |
|  | -3 | 1.41 | 0.81 | 0.65 | 0,48 | 1.07 | 0.58 | 0.43 | 0,35 |
|  | 0 | 0.84 | 0.52 | 0.43 | 0,33 | 0.61 | 0.37 | 0.28 | 0,22 |
|  | - | - | - | - | - | - | - | - | - |

Table 15: RSRP error [dB] for ETU 30 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  |  | RSS duration: 2 subframes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 measurement period |  |  |  | L1 measurement period |  |  |  |
|  |  | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) |
| ETU 30 Hz | -15 | 8.82 | 6.07 | 4.92 | 3,86 | 7.22 | 4.78 | 3.80 | 3,00 |
|  | -12 | 6.01 | 3.72 | 2.99 | 2,23 | 4.64 | 2.76 | 2.15 | 1,64 |
|  | -9 | 3.68 | 2.14 | 1.71 | 1,30 | 2.71 | 1.50 | 1.25 | 0,94 |
|  | -6 | 2.11 | 1.26 | 1.00 | 0,76 | 1.44 | 0.91 | 0.69 | 0,55 |
|  | -3 | 1.18 | 0.80 | 0.65 | 0,47 | 0.87 | 0.57 | 0.49 | 0,36 |
|  | 0 | 0.82 | 0.54 | 0.43 | 0,32 | 0.62 | 0.41 | 0.31 | 0,25 |
|  | 5 | - | - | - | - | - | - | - | - |

Colliding RSS scenarios, results for 1 and 2 subframes

In this section, simulations results are reported for the colliding RSS scenarios for the simulation settings agreed in [1] for 1 RX antenna. It is noted that full collision of the interferer with differently initialised RSS sequence is assumed.

1 RX antenna, relative interferer power: -3 dB

The following results were obtained for single RX antenna, shown in Tables 16 to 18 for AWGN, EPA 5 Hz and ETU 30 Hz propagation channels.

Table 16: RSRP error [dB] for AWGN.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  | RSS duration: 2 subframes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms |
| AWGN | -15 | 6.25 | - | 4.41 | 5.03 | - | 3.60 |
|  | -12 | 4.37 | - | 2.95 | 4.70 | - | 2.02 |
|  | -9 | 3.94 | - | 1.49 | 2.51 | - | 1.09 |
|  | -6 | 2.04 | - | 0.91 | 1.38 | - | 0.73 |
|  | 0 | 0.87 | - | 0.51 | 0.68 | - | 0.42 |
|  | 5 | 0.55 | - | 0.35 | 0.44 | - | 0.30 |

Table 17: RSRP error [dB] for EPA 5 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  | RSS duration: 2 subframes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms |
| EPA 5 Hz | -15 | 15.87 | - | 4.80 | 14.17 | - | 3.64 |
|  | -12 | 12.30 | - | 3.22 | 11.28 | - | 2.45 |
|  | -9 | 9.75 | - | 1.93 | 7.96 | - | 1.40 |
|  | -6 | 7.28 | - | 1.18 | 5.89 | - | 0.90 |
|  | 0 | 3.52 | - | 0.64 | 3.11 | - | 0.54 |
|  | 5 | 2.43 | - | 0.47 | 2.04 | - | 0.42 |

Table 18: RSRP error [dB] for ETU 30 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  | RSS duration: 2 subframes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms |
| ETU 30 Hz | -15 | 11.41 | - | 4.49 | 10.07 | - | 3.94 |
|  | -12 | 8.50 | - | 3.10 | 6.92 | - | 2.43 |
|  | -9 | 6.04 | - | 1.76 | 4.58 | - | 1.29 |
|  | -6 | 3.90 | - | 1.11 | 2.90 | - | 0.85 |
|  | 0 | 1.76 | - | 0.55 | 1.40 | - | 0.43 |
|  | 5 | 1.22 | - | 0.39 | 1.01 | - | 0.38 |

In addition to the RSRP error, delta RSRP CDF curves for EPA 5 Hz and ETU 30 Hz for RSS duration of 1 subframe and 2 subframes are shown in Figure 8 and Figure 9 for the SNR points -15 dB, -12 dB, -9 dB and -6 dB.

Figure 8: CDF curves for delta RSRP (EPA 5 Hz and ETU 30 Hz for RSS duration=1 subframe, 1 RX)

Figure 9: CDF curves for delta RSRP (EPA 5 Hz and ETU 30 Hz for RSS duration=2 subframes, 1 RX)

1 RX antenna, relative interferer power: 0 dB

The following results were obtained for single RX antenna, shown in Tables 19 to 22 for AWGN, EPA 5 Hz, ETU 30 Hz and EPA 1 Hz propagation channels.

Table 19: RSRP error [dB] for AWGN.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  | RSS duration: 2 subframes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms |
| AWGN | -15 | 6.27 | - | 4.18 | 5.09 | - | 3.65 |
|  | -12 | 4.41 | - | 2.81 | 4.00 | - | 2.15 |
|  | -9 | 3.62 | - | 1.61 | 2.58 | - | 1.24 |
|  | -6 | 2.08 | - | 1.10 | 1.47 | - | 0.85 |
|  | 0 | 1.04 | - | 0.64 | 0.82 | - | 0.54 |
|  | 5 | 0.69 | - | 0.50 | 0.59 | - | 0.46 |

Table 20: RSRP error [dB] for EPA 5 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  | RSS duration: 2 subframes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms |
| EPA 5 Hz | -15 | 15.38 | - | 4.90 | 14.25 | - | 3.93 |
|  | -12 | 12.63 | - | 3.17 | 11.41 | - | 2.51 |
|  | -9 | 10.13 | - | 2.10 | 8.77 | - | 1.62 |
|  | -6 | 7.91 | - | 1.46 | 6.58 | - | 1.13 |
|  | 0 | 4.71 | - | 0.91 | 3.92 | - | 0.85 |
|  | 5 | 3.39 | - | 0.79 | 3.59 | - | 0.74 |

Table 21: RSRP error [dB] for ETU 30 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  | RSS duration: 2 subframes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms | L1 meas. period: single shot | L1 meas. period: 480 ms | L1 meas. period: 800 ms |
| ETU 30 Hz | -15 | 11.48 | - | 4.62 | 9.68 | - | 3.79 |
|  | -12 | 8.66 | - | 2.89 | 6.97 | - | 2.34 |
|  | -9 | 6.20 | - | 1.84 | 4.95 | - | 1.47 |
|  | -6 | 4.43 | - | 1.34 | 3.42 | - | 1.02 |
|  | 0 | 2.53 | - | 0.79 | 2.10 | - | 0.74 |
|  | 5 | 2.10 | - | 0.65 | 1.73 | - | 0.63 |

Table 22: RSRP error [dB] for EPA 1 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  |  | RSS duration: 2 subframes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 measurement period |  |  |  | L1 measurement period |  |  |  |
|  |  | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) |
| EPA 1 Hz | -15 | 16.26 | 9.37 | 6.96 | 5.46 | 14.63 | 8.24 | 5.49 | 4.10 |
|  | -12 | 12.68 | 6.74 | 4.77 | 3.34 | 10.98 | 5.58 | 3.79 | 2.65 |
|  | -9 | 10.15 | 4.68 | 3.01 | 2.27 | 8.53 | 3.47 | 2.62 | 1.87 |
|  | -6 | 7.72 | 3.19 | 2.13 | 1.52 | 6.94 | 2.47 | 1.70 | 1.23 |
|  | -3 | 5.67 | 2.44 | 1.65 | 1.16 | 5.16 | 1.90 | 1.38 | 0.99 |
|  | 0 | 4.79 | 1.95 | 1.39 | 0.94 | 3.95 | 1.66 | 1.21 | 0.87 |
|  | 5 | - | - | - | - | - | - | - | - |

In addition to the RSRP error, delta RSRP CDF curves for EPA 5 Hz and ETU 30 Hz for RSS duration of 1 subframe and 2 subframes are shown in Figure 10 and Figure 11 for the SNR points -15 dB, -12 dB, -9 dB and -6 dB.

Figure 10: CDF curves for delta RSRP (EPA 5 Hz and ETU 30 Hz for RSS duration=1 subframe, 1 RX)

Figure 11: CDF curves for delta RSRP (EPA 5 Hz and ETU 30 Hz for RSS duration=2 subframes, 1 RX)

Non-colliding scenario, comparison to CRS-based 	performance for 1 and 2 subframes

In this section, simulations results are reported for the non-colliding RSS scenario for CRS-based RSRP measurements with the same measurement duration of 1 or 2 subframes per sample and for 1 RX antenna. The CRS are distributed over 6 PRBs corresponding to the measurement bandwidth.

1 RX antenna

The following results were obtained for single RX antenna, shown in Table 23 for EPA 5 Hz propagation channel.

Table 23: RSRP error [dB] for EPA 5 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  |  | RSS duration: 2 subframes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 measurement period |  |  |  | L1 measurement period |  |  |  |
|  |  | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) |
| EPA 5 Hz | -15 | 18.61 | 12.58 | 10.38 | 8.22 | 17.55 | 11.21 | 9.28 | 6.77 |
|  | -12 | 15.50 | 9.35 | 7.31 | 5.47 | 14.46 | 8.15 | 5.94 | 4.27 |
|  | -9 | 13.03 | 6.61 | 4.67 | 3.68 | 11.57 | 5.14 | 3.76 | 3.25 |
|  | -6 | 9.85 | 4.51 | 3.06 | 2.63 | 8.12 | 3.14 | 2.32 | 1.89 |
|  | -3 | - | 2.69 | 2.15 | - | - | 2.03 | 1.55 | - |
|  | 0 | 5.20 | 1.87 | 1.21 | 0.91 | 3.32 | 1.21 | 0.97 | 0.65 |
|  | 5 | 1.96 | - | - | 0.44 | 1.42 | - | - | 0.33 |

The difference in RSRP error versus RSS-based RSRP measurement performance for the same sample duration is recorded in Table 24.

Table 24: Difference in RSRP error [dB] to RSS-based RSRP for EPA 5 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  |  | RSS duration: 2 subframes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 measurement period |  |  |  | L1 measurement period |  |  |  |
|  |  | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) |
| EPA 5 Hz | -15 | 3.08 | 3.42 | 3.50 | 3.34 | 3.56 | 3.46 | 3.91 | 3.01 |
|  | -12 | 3.00 | 2.88 | 3.01 | 2.16 | 3.68 | 3.35 | 2.72 | 2.01 |
|  | -9 | 4.08 | 2.60 | 1.95 | 1.73 | 2.95 | 2.22 | 1.82 | 1.95 |
|  | -6 | 3.02 | 2.25 | 1.27 | 1.57 | 2.69 | 1.48 | 1.20 | 1.08 |
|  | -3 | - | 1.42 | 1.16 | - | - | 1.06 | 0.90 | - |
|  | 0 | 2.85 | 1.03 | 0.63 | 0.53 | 1.76 | 0.66 | 0.58 | 0.38 |
|  | 5 | 1.07 | - | - | 0.25 | 0.80 | - | - | 0.19 |

In addition to the RSRP error, delta RSRP CDF curves for EPA 5 Hz for RSS duration of 1 subframe and 2 subframes are shown in Figure 12 and Figure 13 for the SNR points -15 dB, -12 dB, -9 dB and -6 dB.

Figure 12: CDF curves for delta RSRP (EPA 5 Hz for sample duration=1 subframe, 1 RX)

Figure 13: CDF curves for delta RSRP (EPA 5 Hz for sample duration=2 subframes, 1 RX)

2 RX antennas

The following results were obtained for two RX antennas, shown in Table 25 for EPA 5 Hz propagation channel.

Table 25: RSRP error [dB] for EPA 5 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  |  | RSS duration: 2 subframes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 measurement period |  |  |  | L1 measurement period |  |  |  |
|  |  | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) |
| EPA 5 Hz | -15 | 13.56 | 9.79 | - | - | 11.77 | 8.35 | - | - |
|  | -12 | 10.60 | 7.07 | - | - | 9.21 | 5.71 | - | - |
|  | -9 | 7.44 | 4.60 | - | - | 6.35 | 3.63 | - | - |
|  | -6 | 5.32 | 2.76 | - | - | 4.12 | 2.09 | - | - |
|  | -3 | 2.98 | 1.77 | - | - | 2.22 | 1.29 | - | - |
|  | 0 | 1.82 | 1.14 | - | - | 1.38 | 0.81 | - | - |
|  | 5 | - | - | - | - | - | - | - | - |

The difference in RSRP error versus RSS-based RSRP measurement performance for the same sample duration is recorded in Table 26.

Table 26: Difference in RSRP error [dB] to RSS-based RSRP for EPA 5 Hz.

| Propagation channel | SNR [dB] | RSS duration: 1 subframe |  |  |  | RSS duration: 2 subframes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L1 measurement period |  |  |  | L1 measurement period |  |  |  |
|  |  | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) | single shot | 320 ms (2 samples) | 480 ms (3 samples) | 800 ms (5 samples) |
| EPA 5 Hz | -15 | 3.67 | 3.34 | - | - | 3.16 | 3.22 | - | - |
|  | -12 | 3.50 | 3.02 | - | - | 3.50 | 2.63 | - | - |
|  | -9 | 2.81 | 2.24 | - | - | 2.97 | 1.90 | - | - |
|  | -6 | 2.70 | 1.48 | - | - | 2.19 | 1.15 | - | - |
|  | -3 | 1.57 | 0.96 | - | - | 1.15 | 0.71 | - | - |
|  | 0 | 0.98 | 0.62 | - | - | 0.77 | 0.44 | - | - |
|  | 5 | - | - | - | - | - | - | - | - |

Discussion

Simulation results for the non-colliding RSS scenario contained in Section 3.1 with RSS duration of 8 or 40 subframes indicate,

- that both a longer RSS duration (i.e. 40 subframes versus 8 subframes) and also a longer L1 measurement period (480 ms or 800 ms versus single shot, i.e. 160 ms) serve well to considerably reduce the delta RSRP value, in particular for SNR values below 0 dB;

- that for averaging over 5 samples the RSRP error remains always below 2.5 dB for RSS duration of 8 subframes and below 1.2 dB for RSS duration of 40 subframes.

- that a further remarkable reduction is observed for EPA 5 Hz and ETU 30 Hz channels for the case of 2 RX antennas, i.e. for 5 samples averaging, the RSRP error remains always below 2 dB for RSS duration of 8 subframes and below 1 dB for RSS duration of 40 subframes.

Simulation results for the non-colliding RSS scenario contained in Section 3.2 with RSS duration of 1 or 2 subframes indicate, that the measurement performance majorly depends on the SNR steadily improving along the SNR in the considered range and in particular below 0 dB, and further:

- for UE with single RX antenna for the considered propagation channels:

- that EPA 1 Hz is the most challenging propagation channel.

- that single shot with 1 or 2 subframes is not delivering sufficient measurement performance.

- that measuring over 2 subframes gives superior performance over 1 subframe up to around 1.5 dB.

- that averaging over 2 samples limits the RSRP error to 2.1 dB for SNR down to -6 dB and to 3.9 dB down to -9 dB, respectively (2 subframes).

- that averaging over 3 samples limits the RSRP error to 1.4 dB for SNR down to -6 dB and to 2.4 dB down to -9 dB, respectively (2 subframes).

- that averaging over 5 samples limits the RSRP error to 2.8 dB for SNR down to -12 dB and to 5.2 dB down to SNR of -15 dB (2 subframes).

- for UE with dual RX antennas for a subset of the considered propagation channels:

- that single shot with 1 or 2 subframes again is not delivering sufficient measurement performance.

- that measuring over 2 subframes gives superior performance over 1 subframe up to around 1.5 dB.

- that averaging over 2 samples limits the RSRP error to 1.8 dB for SNR down to -9 dB, to 3.1 dB down to -12 dB, and to 5.2 dB down to -15 dB, respectively (2 subframes).

- that averaging over 3 samples limits the RSRP error to 1.4 dB for SNR down to -9 dB, to 2.3 dB down to -12 dB, and to 4.0 dB down to -15 dB, respectively (2 subframes).

- that averaging over 5 samples limits the RSRP error to 2.0 dB for SNR down to -12 dB and to 3.1 dB down to -15 dB, respectively (2 subframes).

Simulation results for colliding RSS scenarios contained in Section 3.3 with RSS duration of 1 or 2 subframes indicate,

- that the colliding scenarios have only small performance impact. Practically the same performance is achieved as for the non-colliding RSS scenario, however a small improvement below 1.5 dB for SNR below – 9 dB, and a small degradation of up to 1 dB above SNR of -9 dB is observed (comparison of Table 22 against Table 11).

Simulation results for the non-colliding scenario contained in Section 3.4 using legacy CRS-based measurement method for RSRP with the same measurement duration of 1 or 2 subframes per sample indicate, that the CRS-based measurement performance over the same measurement period is inferior to RSS-based RSRP measurement method, and further:

- for UE with single RX antenna for the considered propagation channel (EPA 5 Hz):

- that for averaging over 2 samples (2 subframes), the performance gain in terms of lower RSRP error at the same SNR point is in the range of 1.5 to 3.5 dB below -6 dB and in the range of 0.6 to 1.5 dB above -6 dB, steadily decreasing with increasing SNR.

- that for averaging over 3 samples (2 subframes), the performance gain in terms of lower RSRP error at the same SNR point is in the range of 1.2 to 3.9 dB below -6 dB and in the range of 0.6 to 1.2 dB above -6 dB, steadily decreasing with increasing SNR.

- that for averaging over 5 samples (2 subframes), the performance gain in terms of lower RSRP error at the same SNR point is in the range of 1.1 to 3.0 dB below -6 dB and in the range of 0.2 to 1.1 dB above -6 dB, steadily decreasing with increasing SNR.

- that the same behaviour is observed for sampling over 1 subframe or 2 subframes, i.e. the performance gain diminishes by around 0.18 dB in average and up to 1.1 dB for 2 subframes compared to 1 subframe.

- for UE with dual RX antennas for the considered propagation channel (EPA 5 Hz):

- that for averaging over 2 samples (2 subframes), the performance gain in terms of lower RSRP error at the same SNR point is in the range of 1.1 to 3.2 dB below -6 dB and in the range of 0.4 to 1.1 dB above -6 dB, steadily decreasing with increasing SNR.

- that the same behaviour is observed for sampling over 1 subframe or 2 subframes, i.e. the performance gain diminishes by around 0.26 dB in average and up to 0.5 dB for 2 subframes compared to 1 subframe.

The following observations can be made at this point in time:

Observation 1: The RSS based RSRP measurement performance improves along increasing SNR (i.e. coverage), i.e. the L1 measurement period for achieving a given RSRP measurement performance, i.e. RSRP accuracy level, can be shorter for higher SNR.

Observation 2: The performance benefit of longer RSS durations such as 8 subframes or 40 subframes for RSS periodicity of 160 ms and increased L1 measurement period, imply a higher measurement latency and furthermore an increased UE power consumption. This impact should be taken into account. The performance can be further improved with 2 RX antennas, yielding for averaging over 800 ms an RSRP error of at most 2 dB or 1 dB for RSS duration of 8 subframes or 40 subframes, respectively.

Observation 3: The performance benefit of measuring either over 1 or 2 subframes at RSS periodicity of 160 ms and then performing averaging over a given number of samples yields an energy efficient measurement procedure, as the UE may go to sleep in between both RSS instances, exhibiting good performance with RSRP error below 5.2 dB even in extreme coverage for UE with 1 RX antenna (most challenging propagation channel is EPA 1 Hz). For UE with 2 RX antennas, for the considered subset of radio channels, the RSRP error is considerably lower, since 5.2 dB in extreme coverage is achieved for averaging over 320 ms (2 samples), and 3.1 dB for averaging over 800 ms (5 samples). A performance gain of up to around 1.5 dB is observed for measurement in 2 subframes versus 1 subframe.

Observation 4: The impact of RSS collision scenarios on RSS-based RSRP measurement performance is found to be very limited.

Observation 5: The RSS-based RSRP measurement performance is superior over CRS-based RSRP measurement performance by 1.1 to 3.9 dB for enhanced coverage and by 0.2 to 1.5 dB for normal coverage for UE with single RX antenna. For UE with 2 RX antennas, the gains are practically same.

Observation 6: The defined RSRP error serves well in qualifying the RSRP measurement performance.

Observation 7: Existing intra-frequency absolute RSRP accuracy performance for UE category M1 is specified in TS 36.133, subclause 9.1.21.1 for CE mode A and in subclause 9.1.21.3 for CE mode B and is reproduced in Table 27 below.

Table 27: RSRP Intra frequency absolute accuracy for UE category M1 in TS 36.133.

| Normal condition | RSRP intra frequency absolute accuracy |
| --- | --- |
| CE mode A, Ês/Iot -6 dB | 7 dB |
| CE mode B, Ês/Iot -12 dB | 7 dB |
| CE mode B, -15≤Ês/Iot≤-12 dB | 8 dB |

Existing intra-frequency absolute RSRP accuracy performance for non-BL CE UE category is specified in TS 36.133, subclause 9.1.26.1 for CE mode A and in subclause 9.1.26.3 for CE mode B and is reproduced in Table 28 below.

Table 28: RSRP Intra frequency absolute accuracy for non-BL CE UE category in TS 36.133.

| Normal condition | RSRP intra frequency absolute accuracy |
| --- | --- |
| CE mode A, Ês/Iot -6 dB | 5.5 dB |
| CE mode B, Ês/Iot -12 dB | 6 dB |
| CE mode B, -15≤Ês/Iot≤-12 dB | 8 dB |

As elaborated in above observations, with RSS-based RSRP the measurement performance can be improved over CRS-based RSRP performance serving as basis for deriving new requirements in TS 36.133.

Based on the above discussion, for measurement averaging over 5 samples (800 ms) and 1 sample over 2 successive subframes, an RSRP error of up to 5 dB in normal coverage and up to 6 dB in enhanced coverage is found to be achievable for UE with 1 RX antenna, corresponding to 2 dB tightening. For UE with 2 RX antennas, an RSRP error of up to 2.5 dB in normal coverage and up to 5 dB in enhanced coverage is found to be achievable, corresponding to 3 dB tightening, which takes into account a margin for evaluating only a subset of propagation channels.

Thus, the following proposal is made:

It is proposed to consider improvements of the existing intra-frequency absolute RSRP accuracy performance for UE category M1 and similar improvements for non-BL CE UE category for RSS-based RSRP measurement accuracy in TS 36.133 according to Table 29 and Table 30 below and derive the setting of measurement parameters for the respective test, based on measurement averaging over 5 samples (800 ms) and 1 sample over 2 successive subframes.

Table 29: Proposed RSRP Intra frequency absolute accuracy for UE category M1 in TS 36.133 for RSS-based RSRP.

| Normal condition | RSRP intra frequency absolute accuracy |
| --- | --- |
| CE mode A, Ês/Iot -6 dB | 5 dB |
| CE mode B, Ês/Iot -12 dB | 5 dB |
| CE mode B, -15≤Ês/Iot≤-12 dB | 6 dB |

Table 30: Proposed RSRP Intra frequency absolute accuracy for non-BL CE UE category in TS 36.133 for RSS-based RSRP.

| Normal condition | RSRP intra frequency absolute accuracy |
| --- | --- |
| CE mode A, Ês/Iot -6 dB | 2.5 dB |
| CE mode B, Ês/Iot -12 dB | 3 dB |
| CE mode B, -15≤Ês/Iot≤-12 dB | 5 dB |

It is proposed that the network may configure different sets of measurement parameters as alternative to the setting in proposal 1, i.e. according to required RSRP accuracy, required latency of the measurement, UE power consumption and timing constraints, etc., but no requirements will be specified in TS 36.133 for these alternative settings. Hence signalling support for such measurement configurations must be ensured.

Conclusion

This contribution lists simulation results for the non-colliding and colliding RSS scenario for a variety of scenarios, in particular for RSS-based RSRP measurement performance for non-colliding and colliding RSS scenarios as well as a performance comparison to CRS-based RSRP measurements for non-colliding scenarios. From the results, obtained so far, following observations are made.

Observation 1: The RSS based RSRP measurement performance improves along increasing SNR (i.e. coverage), i.e. the L1 measurement period for achieving a given RSRP measurement performance, i.e. RSRP accuracy level, can be shorter for higher SNR.

Observation 2: The performance benefit of longer RSS durations such as 8 subframes or 40 subframes for RSS periodicity of 160 ms and increased L1 measurement period, imply a higher measurement latency and furthermore an increased UE power consumption. This impact should be taken into account. The performance can be further improved with 2 RX antennas, yielding for averaging over 800 ms an RSRP error of at most 2 dB or 1 dB for RSS duration of 8 subframes or 40 subframes, respectively.

Observation 3: The performance benefit of measuring either over 1 or 2 subframes at RSS periodicity of 160 ms and then performing averaging over a given number of samples yields an energy efficient measurement procedure, as the UE may go to sleep in between both RSS instances, exhibiting good performance with RSRP error below 5.2 dB even in extreme coverage for UE with 1 RX antenna (most challenging propagation channel is EPA 1 Hz). For UE with 2 RX antennas, for the considered subset of radio channels, the RSRP error is considerably lower, since 5.2 dB in extreme coverage is achieved for averaging over 320 ms (2 samples), and 3.1 dB for averaging over 800 ms (5 samples). A performance gain of up to around 1.5 dB is observed for measurement in 2 subframes versus 1 subframe.

Observation 4: The impact of RSS collision scenarios on RSS-based RSRP measurement performance is found to be very limited.

Observation 5: The RSS-based RSRP measurement performance is superior over CRS-based RSRP measurement performance by 1.1 to 3.9 dB for enhanced coverage and by 0.2 to 1.5 dB for normal coverage for UE with single RX antenna. For UE with 2 RX antennas, the gains are practically same.

Observation 6: The defined RSRP error serves well in qualifying the RSRP measurement performance.

Observation 7: Existing intra-frequency absolute RSRP accuracy performance for UE category M1 is specified in TS 36.133, subclause 9.1.21.1 for CE mode A and in subclause 9.1.21.3 for CE mode B and is reproduced in Table 27 below.

Table 27: RSRP Intra frequency absolute accuracy for UE category M1 in TS 36.133.

| Normal condition | RSRP intra frequency absolute accuracy |
| --- | --- |
| CE mode A, Ês/Iot -6 dB | 7 dB |
| CE mode B, Ês/Iot -12 dB | 7 dB |
| CE mode B, -15≤Ês/Iot≤-12 dB | 8 dB |

Existing intra-frequency absolute RSRP accuracy performance for non-BL CE UE category is specified in TS 36.133, subclause 9.1.26.1 for CE mode A and in subclause 9.1.26.3 for CE mode B and is reproduced in Table 28 below.

Table 28: RSRP Intra frequency absolute accuracy for non-BL CE UE category in TS 36.133.

| Normal condition | RSRP intra frequency absolute accuracy |
| --- | --- |
| CE mode A, Ês/Iot -6 dB | 5.5 dB |
| CE mode B, Ês/Iot -12 dB | 6 dB |
| CE mode B, -15≤Ês/Iot≤-12 dB | 8 dB |

As elaborated in above observations, with RSS-based RSRP the measurement performance can be improved over CRS-based RSRP performance serving as basis for deriving new requirements in TS 36.133.

Based on the above discussion, for measurement averaging over 5 samples (800 ms) and 1 sample over 2 successive subframes, an RSRP error of up to 5 dB in normal coverage and up to 6 dB in enhanced coverage is found to be achievable for UE with 1 RX antenna, corresponding to 2 dB tightening. For UE with 2 RX antennas, an RSRP error of up to 2.5 dB in normal coverage and up to 5 dB in enhanced coverage is found to be achievable, corresponding to 3 dB tightening, which takes into account a margin for evaluating only a subset of propagation channels.

Thus, the following proposals are made:

- It is proposed to consider improvements of the existing intra-frequency absolute RSRP accuracy performance for UE category M1 and similar improvements for non-BL CE UE category for RSS-based RSRP measurement accuracy in TS 36.133 according to Table 29 and Table 30 below and derive the setting of measurement parameters for the respective test, based on measurement averaging over 5 samples (800 ms) and 1 sample over 2 successive subframes.

Table 29: Proposed RSRP Intra frequency absolute accuracy for UE category M1 in TS 36.133 for RSS-based RSRP.

| Normal condition | RSRP intra frequency absolute accuracy |
| --- | --- |
| CE mode A, Ês/Iot -6 dB | 5 dB |
| CE mode B, Ês/Iot -12 dB | 5 dB |
| CE mode B, -15≤Ês/Iot≤-12 dB | 6 dB |

Table 30: Proposed RSRP Intra frequency absolute accuracy for non-BL CE UE category in TS 36.133 for RSS-based RSRP.

| Normal condition | RSRP intra frequency absolute accuracy |
| --- | --- |
| CE mode A, Ês/Iot -6 dB | 2.5 dB |
| CE mode B, Ês/Iot -12 dB | 3 dB |
| CE mode B, -15≤Ês/Iot≤-12 dB | 5 dB |

It is proposed that the network may configure different sets of measurement parameters as alternative to the setting in proposal 1, i.e. according to required RSRP accuracy, required latency of the measurement, UE power consumption and timing constraints, etc., but no requirements will be specified in TS 36.133 for these alternative settings. Hence signalling support for such measurement configurations must be ensured and parameter settings are FFS.

References

- R4-1904808, Update of simulation assumptions for RSS based RRM measurements for MTC, source: Ericsson, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell

- R4-1906665, Simulation results for RSS based RSRP measurements, source: Nokia, Nokia Shanghai Bell

- R4-1901582, Discussions on RSS based RRM measurements for MTC, source: Ericsson