3GPP TSG-RAN WG4 #92	R4- 1909401

Ljubljana, SI, EU, 26th – 30th August 2019

Source:	Ericsson

Title:	Initial Simulation Results

Agenda item:	9.11.2

Document for:	Discussion

# 1.	Introduction

In RAN4#91, initial simulation assumptions have been made in order to start the study phase for the feasibility study of 256 QAM in FR2.  The following parameters have been considered as part of the results presented in this contribution.  The parameters are a subset of the overall agreed parameters captured in [1].

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257) and 39 GHz (n260) |
| CBW | 50 MHz, 100MHz, 200MHz |
| SCS | 60kHz, 120 kHz; |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency Static |
| MCS | 64QAM: MCS 23, 24, 26, 28 in TS 38.214 Table 5.1.3.1-1 256QAM: MCS 21, 23, 25, 27 in TS 38.214 Table 5.1.3.1-2, fixed MCSs |
| Precoding | Precoding configuration defined in 38.101-4 Section 7.2 for fading channels and Section 7.5 for static channel; follow PMI |
| Symbol type | CP-OFDM |
| HARQ | None |
| Antenna configuration | Fading channel: 1x2 for Rank1 Static channel: 1x2 for Rank1 |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol |
| PTRS configuration | KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): PN model config1: example1 (BS) + example1(UE) |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: [1.0%-5.0%], rxEVM: [1.0%-5.0%] Option 1: txEVM < rxEVM; Option 2: no restriction |
| Other parameters | follow assumptions in 38.101-4 Section 7.2 for fading channels (e.g., case 2-6) and Section 7.5 for static channels |

# 2.	Discussion

In this contribution, the first set of results are presented for the study phase of the 256 QAM in FR2.  As the simulation assumptions in the WF [1] show that there are many permutations and approaches that could be used to study if benefits exist for introducing a higher modulation in the millimetre wave frequencies in NR.

As part of the study a companion paper [2] discusses further the complications on testability for this feature when it comes to receiver demodulation and the required SINR.  This contribution will not go into detail regarding receiver demodulation needs, but rather focus on the overall throughput that can be achieved.  Looking at the parameters, the higher order MCS (256 QAM MCS 25 and 27) are not presented here as initial results yielded little to no throughput.  Although, to keep ease in simulation time, HARQ was not applied and it may be possible to see more tangible throughput numbers.

The following results show the throughput performance at 256 QAM.

Figure 1: SCS 60 kHz, MCS 23

Comparably, when looking at 64 QAM throughput performance is better than 256 QAM when the expected EVM at transmitter and receiver is 5%.  The overall performance of 256 QAM at low SNR is rather sensitive to any added receiver and/or transmitter noise.  The fading channel conditions also provide some aspects to the results below.  Further simulations using HARQ could help this aspect; no link adaptation was simulated for this scenario.

Figure 2: 256 QAM and 64 QAM throughput performance comparison

# 3.	Conclusions

In this paper, preliminary simulation results were presented.  The agreed list of simulation parameters were extensive and as such were not presented here.  However, the conclusion of this contribution shows that there is potential that this feature may not provide additional benefit to that of using the current highest modulation (64 QAM) defined in FR2.

# 4.	References

R4-1907711, “Simulation assumption of feasibility study for FR2 DL 256QAM”, China Telecom

R4-1909403, “Feasibility of UE demodulation testing”, Ericsson