3GPP TSG-RAN WG4 Meeting #92 	R4-1909526

# Ljubljana, Slovenia, Aug 25th – 30th 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | XXX | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.141-2 Radiated test requirements for NR multi-slot PUCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) NR multi-slot PUCCH performance requirements are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update section 8.3.6 for NR multi-slot PUCCH performance requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR multi-slot PUCCH performance tests can’t be performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.104 |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<start of the change>

### 8.3.6	Performance requirements for multi-slot PUCCH format

### 8.3.6.1	Performance requirements for multi-slot PUCCH format 1

#### 8.3.6.1.1	NACK to ACK detection

###### 8.3.6.1.1.1	Definition and applicability

The performance requirement of PUCCH format 1 for NACK to ACK detection is determined by the two parameters: probability of false detection of the ACK and the NACK to ACK detection probability. The performance is measured by the required SNR at probability of the NACK to ACK detection equal to 0.1% or less. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK at particular bit position when input is only noise. Each false bit detection is counted as one error.

The NACK to ACK detection probability is the probability of detecting an ACK bit when an NACK bit was sent on particular bit position. Each NACK bit erroneously detected as ACK bit is counted as one error. Erroneously detected NACK bits in the definition do not contain the NACK bits which are mapped from DTX, i.e. NACK bits received when DTX is sent should not be considered.

A test for a specific combination of SCS and channel bandwidth is only applicable if the BS declares to support it.

For a BS supporting multiple combinations of SCS and channel bandwidth, the applicable rule is 8.1.2.2.

###### 8.3.6.1.1.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.1.7.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.2.7.

##### 8.3.6.1.1.3	Test purpose

The test shall verify the receiver’s ability not to falsely detect NACK bits as ACK bits under multipath fading propagation conditions for a given SNR.

###### 8.3.6.1.1.4	Method of test

8.3.6.1.1.4.1	Initial Conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see sub-clause 4.9.1

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (D.31).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

8.3.6.1.1.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1) Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2) Align the manufacturer declared coordinate system orientation of the BS with the test system.

3) Set the BS in the declared direction to be tested.

4) Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5) The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in Table 8.3.6.1.1.4.2-1.

Table 8.3.6.1.1.4.2-1: Test Parameters for multi-slot PUCCH format 1

| Parameter | Test |
| --- | --- |
| nrofBits | 2 |
| nrofPRBs | 1 |
| nrofSymbols | 14 |
| startingPRB | 0 |
| intraSlotFrequencyHopping | disabled |
| interSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| pucch-GroupHopping | neither |
| hoppingId | 0 |
| initialCyclicShift | 0 |
| startingSymbolIndex | 0 |
| Index of orthogonal sequence (time-domain-OCC) | 0 |
| Number of slots | 2 |

6) The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7) Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.6.1.1.5.1 and 8.3.6.1.1.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in Table 8.3.6.1.1.4.2-2

Table 8.3.6.1.1.4.2-2 AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | -83.5 dBm - ΔOTAREFSENS/ 4.5MHz |
|  |  | 10 | -80.3 dBm - ΔOTAREFSENS/ 9.36MHz |
|  |  | 20 | -77.2 dBm - ΔOTAREFSENS/ 19.08MHz |
|  | 30 kHz | 10 | -80.6 dBm - ΔOTAREFSENS/ 8.64MHz |
|  |  | 20 | -77.4 dBm - ΔOTAREFSENS/ 18.36MHz |
|  |  | 40 | -74.2 dBm - ΔOTAREFSENS/ 38.16MHz |
|  |  | 100 | -70.1 dBm - ΔOTAREFSENS/ 98.28MHz |
| BS type 2-O | 60 kHz | 50 | [EISREFSENS_50M + 12dBm / 47.52 MHz] |
|  |  | 100 | [EISREFSENS_50M + 15 dBm / 95.04 MHz] |
|  |  | 50 | [EISREFSENS_50M + 12 dBm / 46.08 MHz] |
|  |  | 100 | [EISREFSENS_50M + 15 dBm / 95.04 MHz] |
|  |  | 200 | [EISREFSENS_50M + 18 dBm / 190.08 MHz] |

8) The signal generator sends random codeword from applicable codebook, in regular time periods. The following statistics are kept: the number of ACK bits detected in the idle periods and the number of NACK bits detected as ACK.

##### 8.3.6.1.1.5	Test Requirement

###### 8.3.6.1.1.5.1	Test Requirement for BS type 1-O

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of NACK bits falsely detected as ACK shall be less than 0.1% for the SNR listed in Tables 8.3.6.1.1.5.1-1.

Table 8.3.6.1.1.5.1-1: Required SNR for multi-slot PUCCH format 1 with 30kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel bandwidth (MHz) / SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | 40 MHz |
| 1 | 8 | Normal | TDLC-300-100 Low | [ TBD ] |

###### 8.3.6.1.1.5.2	Test Requirement for BS type 2-O

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of NACK bits falsely detected as ACK shall be less than 0.1% for the SNR listed in Tables 8.3.6.1.1.5.2-1.

Table 8.3.6.1.1.5.2-1: Required SNR for multi-slot PUCCH format 1 with 30kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel bandwidth (MHz) / SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | 40 MHz |
| 1 | 8 | Normal | TDLC-300-100 Low | [ TBD ] |

##### 8.3.6.1.2	ACK missed detection

###### 8.3.6.1.2.1	Definition and applicability

The performance requirement of PUCCH format 1 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

A test for a specific combination of SCS and channel bandwidth is only applicable if the BS declares to support it.

For a BS supporting multiple combinations of SCS and channel bandwidth, the applicable rule is defined in 8.1.2.2.

###### 8.3.6.1.2.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.1.7.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.2.7.

###### 8.3.6.1.2.3	Test purpose

The test shall verify the receiver’s ability to detect ACK bits under multipath fading propagation conditions for a given SNR.

###### 8.3.6.1.2.4	Method of test

###### 8.3.6.1.2.4.1	Initial Conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see sub-clause 4.9.1

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (D.31).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

###### 8.3.6.1.2.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in Table 8.3.6.1.2.4.2-1.

Table 8.3.6.1.2.4.2-1: Test Parameters for multi-slot PUCCH format 1

| Parameter | Test |
| --- | --- |
| nrofBits | 2 |
| nrofPRBs | 1 |
| nrofSymbols | 14 |
| startingPRB | 0 |
| intraSlotFrequencyHopping | disabled |
| interSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| pucch-GroupHopping | neither |
| hoppingId | 0 |
| initialCyclicShift | 0 |
| startingSymbolIndex | 0 |
| Index of orthogonal sequence (time-domain-OCC) | 0 |
| Number of slots | 2 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex TBD.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.6.1.2.5.1 and 8.3.6.1.2.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in Table 8.3.6.1.2.4.2-2.

Table 8.3.6.1.2.4.2-2 AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | -83.5 dBm - ΔOTAREFSENS/ 4.5MHz |
|  |  | 10 | -80.3 dBm - ΔOTAREFSENS/ 9.36MHz |
|  |  | 20 | -77.2 dBm - ΔOTAREFSENS/ 19.08MHz |
|  | 30 kHz | 10 | -80.6 dBm - ΔOTAREFSENS/ 8.64MHz |
|  |  | 20 | -77.4 dBm - ΔOTAREFSENS/ 18.36MHz |
|  |  | 40 | -74.2 dBm - ΔOTAREFSENS/ 38.16MHz |
|  |  | 100 | -70.1 dBm - ΔOTAREFSENS/ 98.28MHz |
| BS type 2-O | 60 kHz | 50 | [EISREFSENS_50M + 12dBm / 47.52 MHz] |
|  |  | 100 | [EISREFSENS_50M + 15 dBm / 95.04 MHz] |
|  |  | 50 | [EISREFSENS_50M + 12 dBm / 46.08 MHz ] |
|  |  | 100 | [EISREFSENS_50M + 15 dBm / 95.04 MHz ] |
|  |  | 200 | [EISREFSENS_50M + 18 dBm / 190.08 MHz] |

8) The signal generator sends a test pattern with the pattern outlined in Figure 8.3.6.1.2.4.2-1. The following statistics are kept: the number of ACKs detected in the idle periods and the number of missed ACKs.

Figure 8.3.6.1.2.4.2-1: Test signal pattern for PUCCH format 1 demodulation tests

##### 8.3.6.1.2.5	Test Requirement

###### 8.3.6.1.2.5.1	Test Requirement for BS type 1-O

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of correctly detected ACK bits shall be larger than 99% for the SNR listed in Tables 8.3.6.1.2.5.1-1.

Table 8.3.6.1.2.5.1-1: Required SNR for multi-slot PUCCH format 1 with 30kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel bandwidth (MHz) / SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | 40 MHz |
| 1 | 8 | Normal | TDLC-300-100 Low | [ TBD ] |

###### 8.3.6.1.2.5.2	Test Requirement for BS type 2-O

The fraction of NACK bits falsely detected as ACK shall be less than 0.1% for the SNR listed in Table 8.3.6.1.2.5.2-1.

Table 8.3.6.1.2.5.2-1: Required SNR for multi-slot PUCCH format 1 with 30kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel bandwidth (MHz) / SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | 40 MHz |
| 1 | 8 | Normal | TDLC-300-100 Low | [ TBD ] |

<end of the change>