3GPP TSG-RAN WG4 Meeting #92	R4-1909529

# Ljubljana, Slovenia, 26th August – 30th August 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.133 on inter-frequency SFTD measurement requirements (Section 9.3.8) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2019-07-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The inter frequency SFTD measurement core requirements were introduced in the last meeting. However there are brackets on the requirements. In addition some descriptions in the requirements need revision to be more accurate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove brackets in the requirements - Remove ‘NR’ in the requirements when unnecessary - Add ‘inter-frequency’ to neighbour cells |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter frequency SFTD measurement core requirements are not finalized. There are ambiguous descriptions in the requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Start of change #1 >

### 9.3.8	NR Inter frequency SFTD measurement requirements

#### 9.3.8.1	Introduction

This clause contains requirements for a UE supporting NR inter-freqency SFTD measurement and is applicable in RRC_CONNECTED state. The UE shall, depending on network request, perform inter-frequency SFTD measurement and report SFTD result with or without SS-RSRP. The overall delay includes RRC procedure delay defined in clause 12 in TS 38.331 [2] and SFTD measurement reporting delay in clause 9.3.8.3.

UE which fulfils the requirements in clause 9.3.8 is not supposed to fulfil the requirements defined in clause 9.2.5.4.

#### 9.3.8.2	SFTD Measurement delay

The requirements on SFTD measurement delay defined in this section are applicable under the side condition SCH Ês/Iot ≥ -3 dB for the inter-frequency neighbour cell. Depending on configuration, the SFTD measurement may be carried out with or without the support of configured measurement gaps. In the current release, indication on whether to carry out the SFTD measurement with or without measurement gaps is implicit and depending on whether measurement gaps are configured.

The UE shall be able to detect, identify and measure SFTD of up to 3 of the strongest inter-frequency neighbour cells on the carrier frequency provided in the SFTD measurement configuration. Further depending on the SFTD measurement configuration, the UE shall additionally report SS-RSRP for the one or more strongest cells. The UE may or may not be configured with [cellsForWhichToReportSFTD]. The UE does not expect [cellsForWhichToReportSFTD] to change during an ongoing SFTD measurement.

When no measurement gaps are provided, the UE shall be capable of finding the inter-frequency neighbour cells regardless of its SSB position in the SMTC period. The SFTD measurement shall be conducted with sustained connection to the PCell and activated SCell(s) in MCG. Depending on capability, the UE may be allowed to cause a certain amount of interruptions for reconfiguration of the radio receiver, as specified in clause 8.2.2.2.6.

When measurement gaps are provided, the UE shall be capable of finding the inter-frequency neighbour cells under the additional condition that the SSB at least occasionally falls within the measurement gap.

When no DRX is used, the UE shall be capable of determining SFTD within a physical layer measurement period of Tmeasure_SFTD1 as follows:

-	For SFTD measurements without measurement gaps, and without additional SS-RSRP reporting:

-	For carrier frequency in FR1: Tmeasure_SFTD1 = 14 SMTC periods

-	For carrier frequency in FR2: Tmeasure_SFTD1 = 112 SMTC periods

-	For SFTD measurements in measurement gaps, and without additional SS-RSRP reporting:

-	For carrier frequency in FR1: Tmeasure_SFTD1 = CSSFinter × 8 × max(MGRP, SMTC period)

-	For carrier frequency in FR2: Tmeasure_SFTD1 = CSSFinter × 64 × max(MGRP, SMTC period)

-	For SFTD measurements without measurement gaps, and with additional SS-RSRP reporting:

-	For carrier frequency in FR1: Tmeasure_SFTD1 = 19 SMTC periods

-	For carrier frequency in FR2: Tmeasure_SFTD1 = 152 SMTC periods

-	For SFTD measurements in measurement gaps, and with additional SS-RSRP reporting:

-	For carrier frequency in FR1: Tmeasure_SFTD1 = CSSFinter × 13 × max(MGRP, SMTC period)

-	For carrier frequency in FR2: Tmeasure_SFTD1 = CSSFinter × 104 × max(MGRP, SMTC period)

where CSSFinter is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

When DRX is used, the same Tmeasure_SFTD1 as for non-DRX applies, but the reporting delay depends on the DRX cycle length in use.

In case PCell is changed due to handover, the UE shall terminate the inter-frequency SFTD measurement.

The measurement accuracy for the SFTD measurement shall fulfill the requirement in clause 10.1.21.3. The measurement accuracy for additionally reported SS-RSRP shall fulfil the requirement in clauses 10.1.4.1 and 10.1.5.1 for neighbour cell in FR1 and FR2, respectively.

#### 9.3.8.3	SFTD Measurement reporting delay

The SFTD measurement reporting delay is defined as the time between a command that will trigger an SFTD measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

The SFTD measurement reporting delay shall be less than Tmeasure_SFTD1 defined in clause 9.3.8.2.

< End of change #1 >