3GPP TSG RAN WG4 Meeting #92	R4-1909606

Ljubljana, Slovenia, August 26-30, 2019

Agenda Item:	9.5.2

Source:	Futurewei

Title:	Discussion on simulations assumptions for power control for IAB

Document for:	Approval

# Introduction

With the agreements for IAB simulation studies made in RAN4#91 [1], it is possible to consider TR 36.826 as a preliminary guidance for establishing RF requirements for IAB even though there are different aspects between LTE relays and NR IAB-MTs. This contribution examines the TR 36.826 [3] and requests clarification for the NR IAB simulation studies.

# Discussion

## Power control

The WF R4-1907825 [1] mentioned using power control for the MT uplink transmission. However, the details of power control algorithm are not listed. One algorithm listed in TR 36.826 [3] is

Pt=Pmax×min1,maxRmin,PLPLx-ileγ

Where PL is the pathloss, PLx-ile is the x-percentile path loss value (including shadowing), 0<γ≤1, Pmax is the maximum power level, and Rmin is the minimum power reduction. Power control will be needed to manage interference levels and for the co-existence studies.

Proposal 1: Details of power control algorithm should be considered for simulation.

## Power levels

After reviewing the transmit power requirements in [1], there appears to be ambiguity in what the minimum power is for backhaul transmissions. Table 1 attempts to capture suggested values in [1] and [2].

Table 1. Maximum and minimum power values.

(Yellow highlight indicates values used in the NR IAB TR 38.874 that are different)

| Node / Link | NR FR1 |  | NR FR2 |  |
| --- | --- | --- | --- | --- |
|  | Minimum power [dBm] | Maximum power [dBm] | Minimum power [dBm] | Maximum power [dBm] |  |
| gNB / IAB donor (FR2) |  | 46 [1] 44 [2] |  | 33 [1] 44 [2] |  |
| IAB-MT TX power |  | 38 dB [1] |  | 33 [1] |  |
| UE | -40 (6.3.1-1) | 23 [1] 23 [2] | -13 | 22.4 EIRP, 13.4 conducted [1] 23 [2] |  |

Observation 1: It is unclear whether the IAB-MT TX power refers to MT to UE, MT to IAB donor, or both.

# Conclusions

This contribution examined aspects of the simulation assumption as how they related to the technical report generated during the IAB study (TR 38.874) as well as the TR for RF requirements for relays (TR 36.826). Some proposal and observations are:

Proposal 1: Details of power control algorithm should be considered for simulation.

Observation 1: It is unclear whether the IAB-MT TX power refers to MT to UE, MT to IAB donor, or both.

# References

R4-1907825, “WF on simulation assumptions for IAB co-existence study”, Qualcomm, RAN4#91, May 13-17, 2019

TR 38.874, “Study on Integrated Access and Backhaul”

TR 36.826, “Relay radio transmission and reception”