3GPP TSG-RAN WG4 Meeting #92	R4-1909618

Ljubljana, Slovenia, Aug 26th - 30th, 2019

Title: 	Discussion on test setup for BFD and link recovery tests

Source: 	Huawei, HiSilicon

Agenda item:	7.11.4.8

Document for:	Discussion

Introduction

The BFD and link recovery tests have been introduced into TS 38.133 for both FR1 and FR2. In this contribution, we will provide further discussion on the test setup for BFD and link recovery tests.

Discussion

For BFD and link recovery, the test consists of five successive time periods from T1 to T5, and the SNR variation during T1 to T5 can be shown as Figure 1.

Figure 1: SNR variation for BFD and link recovery testing

During T1 and T2, UE shall not trigger beam failure. During T3, the UE shall trigger beam failure. During T4 and T5, UE need to perform L1-RSRP measurements on CBD-RS in q1. The values of T1, T2 and T3 need to be longer than one BFD evaluation period. The values of T4 and T5 need to be longer than one CBD evaluation period.

The BFD evaluation period depends on BFD-RS period, SMTC period, MGRP and DRX configuration. In both FR1 and FR2 , the BFD measurements need to be punctured by gaps. In FR2, the BFD measurements also need to be punctured by SMTC measurements, and UE should perform beam sweeping operations. In current BFD and link recovery tests, the test setup can be summarized as Table 1.

Table 1: Test setup for BFD and link recovery testing

| FR | RS type | DRX Config | RS period (ms) | SMTC period (ms) | MGRP (ms) | DRX cycle (ms) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | SSB | Non-DRX | 20 | 20 | 40 | N/A |
|  |  | DRX | 20 | 20 | 40 | 640 |
|  | CSI-RS | Non-DRX | 10 | 20 | N/A | N/A |
|  |  | DRX | 10 | 20 | 40 | 640 |
| FR2 | SSB | Non-DRX | 20 | 20 | 40 | N/A |
|  |  | DRX | 20 | 20 | 40 | 640 |
|  | CSI-RS | Non-DRX | 10 | 20 | N/A | N/A |
|  |  | DRX | 10 | 20 | 40 | 640 |

For DRX tests, DRX cycle is configured as 640ms, the BFD evaluation period would be very long if the measurement gaps are configured. For FR2 tests, due to beam sweeping operation, the measurement sampels for BFD measurements would be 8 times than that for FR1. In order to avoiding evaluation period further scaled by SMTC, a longer SMTC period can be used. For example, SMTC period with 160ms is used for FR2 RLM tests. Obviously, 640ms DRX cycle will lead to an extremely long evaluation period. Hen, a shorter DRX cycle is suggested for FR2 tests. Hence, we have the following proposals:

Proposal 1: For BFD and link recovery tests, measurement gaps are not configured for all the DRX tests.

Proposal 2: For BFD and link recovery tests, SMTC with 160ms period is configured for all the FR2 tests.

Proposal 3: For BFD and link recovery tests, 40ms DRX cycle is configured for non-DRX tests in FR2.

According to the above proposals, the BFD and CBD evaluation periods in each test can be calculated as Table 2.

Table 2: BFD and CBD evaluation period for BFD and link recovery testing

| Case | FR | RS type | DRX Config | RS period (ms) | SMTC period (ms) | MGRP (ms) | DRX cycle (ms) | Factor P | Scaling 1.5 | Ptotal | Total Sample Number | Tevaluation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BFD | FR1 | SSB | Non-DRX | 20 | 20 | 40 | N/A | 2 | N/A | 2 | 10 | 200 |
|  |  |  | DRX | 20 | 20 | N/A | 640 | 1 | N/A | 1 | 5 | 3200 |
|  |  | CSI-RS | Non-DRX | 10 | 20 | N/A | N/A | 1 | N/A | 1 | 10 | 100 |
|  |  |  | DRX | 10 | 20 | N/A | 640 | 1 | N/A | 1 | 10 | 6400 |
|  | FR2 | SSB | Non-DRX | 20 | 160 | 40 | N/A | 2 | N/A | 2 | 80 | 1600 |
|  |  |  | DRX | 20 | 160 | N/A | 40 | 8/7 | 3/2 | 12/7 | 69 | 2760 |
|  |  | CSI-RS | Non-DRX | 10 | 160 | N/A | N/A | 16/15 | N/A | 16/15 | 86 | 860 |
|  |  |  | DRX | 10 | 160 | N/A | 40 | 16/15 | 3/2 | 8/5 | 128 | 5120 |
| CBD | FR1 | SSB | Non-DRX | 20 | 20 | 40 | N/A | 2 | N/A | 2 | 6 | 120 |
|  |  |  | DRX | 20 | 20 | N/A | 640 | 1 | N/A | 1 | 3 | 1920 |
|  |  | CSI-RS | Non-DRX | 10 | 20 | N/A | N/A | 1 | N/A | 1 | 3 | 30 |
|  |  |  | DRX | 10 | 20 | N/A | 640 | 1 | N/A | 1 | 3 | 1920 |
|  | FR2 | SSB | Non-DRX | 20 | 160 | 40 | N/A | 2 | N/A | 2 | 48 | 960 |
|  |  |  | DRX | 20 | 160 | N/A | 40 | 8/7 | N/A | 8/7 | 28 | 560 |
|  |  | CSI-RS | Non-DRX | 10 | 160 | N/A | N/A | 16/15 | N/A | 16/15 | 26 | 260 |
|  |  |  | DRX | 10 | 160 | N/A | 40 | 16/15 | N/A | 16/15 | 26 | 260 |

The values of T1-T5 for BFD and link recovery tests are summarized in Table 3.

Table 3: T1-T5 setup for BFD and link recovery testing

| Test | Time period |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | T1 | T2 | T3 | T4 | T5 | D1 |
| BFD and link recovery | 1s | 0.4s | 0.6s | 0.4s | 1.4s | 0.44s |

It can be observed that the above T1-T5 configurations are not suitable for some BFD and link recovery tests. The beamFailureInstanceMaxCount is configured as 2. Then, the values of T2 and T3 shall be longer than twice of BFD evaluation period. The value of time period D1 equals to one CBD evaluation period plus 40ms.

The values of T1-T5 for each BFD and link recovery test can be updated as Table 1.

Table 4: Updated T1-T5 setup for BFD and link recovery testing

| FR | RS type | DRX Config | T1 | T2 | T3 | T4 | T5 | D1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | SSB | Non-DRX | 1s | 0.4s | 0.6s | 0.4s | 1.4s | 0.16s |
|  |  | DRX | 8s | 6.4s | 8s | 2s | 3s | 1.96s |
|  | CSI-RS | Non-DRX | 1s | 0.4s | 0.6s | 0.4s | 1.4s | 0.07s |
|  |  | DRX | 8s | 12.8s | 16s | 2s | 3s | 1.96s |
| FR2 | SSB | Non-DRX | 8s | 3.2s | 4s | 1s | 1.4s | 1s |
|  |  | DRX | 8s | 5.52s | 6s | 0.8s | 1.4s | 0.6s |
|  | CSI-RS | Non-DRX | 8s | 1.72s | 2s | 0.4s | 1.4s | 0.3s |
|  |  | DRX | 8s | 10.24s | 12s | 0.4s | 1.4s | 0.3s |

Proposal 4: The values of time period T1/T2/T3/T4/T5 and D1 for BFD and link recovery tests can be updated as Table 4.

Conclusions

This contribution provides the discussion on the currently test setup for BFD and link recovery tests, and the following are provided:

Proposal 1: For BFD and link recovery tests, measurement gaps are not configured for all the DRX tests.

Proposal 2: For BFD and link recovery tests, SMTC with 160ms period is configured for all the FR2 tests.

Proposal 3: For BFD and link recovery tests, 40ms DRX cycle is configured for non-DRX tests in FR2.

Proposal 4: The values of time period T1/T2/T3/T4/T5 and D1 for BFD and link recovery tests can be updated as Table 4.