3GPP TSG-RAN4 Meeting #92	R4-1909651

# Ljubljana, SI, 26th – 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on SCell activation requirements for Trs (section 8.3.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-06-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | So far the SCell activation requirements are defined based on SMTC period. However, the configuration of MO for SCell is not mandatory, so the requirements should take into account the case where SMTC period is not available. In addition, the overall SCell activation delay is defined in number of slots, but Tactivation_time and TCSI_Reporting are defined in ms. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace TSMTC with Trs in SCell activation requirements. Align the unit for activation delay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SCell activation requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 8.3.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this section shall apply for the UE configured with one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+ Tactivation_time+TCSI_ReportingNR Slot length, where:

THARQ is the timing between DL data transmission and acknowledgement as specified in [7].

Tactivation_time is the SCell activation delay in millisecond.

If the SCell is known and belongs to FR1, Tactivation_time is:

-	[Trs_SCell + 5ms], if the SCell measurement cycle is equal to or smaller than [160ms].

-	[Trs_MAX + Trs_SCell + 5ms], if the SCell measurement cycle is larger than [160ms].

If the SCell is unknown and belongs to FR1, Tactivation_time is:

-	[2*Trs_MAX + 2*Trs_SCell + 5ms] provided the SCell can be successfully detected on the first attempt.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, provided that the SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in section 3.6.3, Tactivation_time is [Trs_SCell + 5ms].

If the SCell being activated belongs to FR2 and there is at least one active serving cell on that FR2 band, if the UE is not provided with any SMTC for the target SCell, Tactivation_time is 3ms.

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1:

-	If the target SCell is known to UE, Tactivation_time is:

-	[TMAC-CE,SCell + TFineTiming + 2ms], if UE receives the SCell activation command and TCI state activation command at the same time.

-	[max{ TMAC-CE,SCell, Tuncertainty} + TMAC-CE_TCI + TFineTiming + 2ms], if UE receives TCI state activation command after SCell activation command.

-	If the target SCell is unknown to UE:

-	[TMAC-CE,SCell + 24* TSMTC_SCell + TL1-RSRP,measure + TL1-RSRP,report + Tuncertainty + TMAC-CE,TCI + TFineTiming + [TCSI-RS_resource_configuration] +2ms]

Note 1: TMAC-CE,SCell, TMAC-CE,TCI, and [TCSI-RS_resource_configuration] shall be determined in RAN4#92 meeting.

Where,

Trs_MAX:

-	In FR1, in case of intra-band SCell activation, Trs_MAX is the longer between Trs_SCell and SMTC periodicity of active serving cells provided the cell specific reference signals from the active serving cells and the SCells being activated are available in the same slot; in case of inter-band SCell activation, Trs_MAX is Trs_SCell.

Trs_SCell: SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs_SCell is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs_SCell is applied with Trs_SCell = 5ms assuming the SSB transmission periodicity is 5ms. There is no requirements if the SSB transmission periodicity is not 5ms.

TMAC-CE_TCI is the time for TCI activation for PDSCH and PDCCH.

TMAC-CE,SCell is the MAC-CE decoding time for SCell activation.

TFineTiming is the time period between UE finish decoding the last MAC CE message and the timing of first complete available SSB corresponding to the TCI state. For unknown case, the requirement is only defined provided that the MAC CE for PDCCH TCI, MAC CE for PDSCH TCI and MAC CE for CSI-RS CQI reporting is after the L1-RSRP measurement reporting.

Tuncertainty is the time period between reception of SCell activation MAC-CE and TCI activation MAC-CE for known case. For unknown case, uncertainty is the time between the first L1-RSRP reporting and when UE receives TCI activation MAC-CE.

TL1-RSRP,measure is L1-RSRP measurement delay as defined in section 9.5 assuming M=1.

TL1-RSRP,report is L1-RSRP reporting delay as defined in section 9.5.

[TCSI-RS_resource_configuration] is the time for CSI-RS resource configuration for CQI reporting.

TCSI_reporting in millisecond is the delay including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in [2].

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max([5] measCycleSCell,  [5] DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in section 9.2 and 9.3.

-	the SSB measured during the period equal to max([5] measCycleSCell, [5] DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in section 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

- During the period equal to [4s] for UE supporting power class1 and [3s] for UE supporting power class 2/3/4 before UE receives MAC-CE command for TCI activation:

  - the UE has sent a valid L3-RSRP measurement report with index

  - SCell  activation command is assumed to be received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

- During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in section 9.2 and 9.3, and the TCI state is selected based on one of the reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown.If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the activation command, TSMTC_Scell follows smtc1 or smtc2 according to the physical cell ID of the target cell being activated. TSMTC_MAX follows smtc1 or smtc2 according to the physical cell IDs of the target cells being activated and the active serving cells.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in [2] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The interruption on PSCell or any activated SCell in SCG for EN-DC mode specified in section 8.2 shall not occur before slot n+1+[THARQ] and not occur after slot n+1+[THARQ +3ms + TSMTC_MAX + TSMTC_duration ].

The interruption  on PCell or any activated SCell in MCG for NR standalone mode specified in section 8.2 shall not occur before slot n+1+[THARQ] and not occur after slot n+1+[THARQ +3ms + TSMTC_MAX + TSMTC_duration].

Starting from the slot specified in section 4.3 of [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

<End of Change 1>