3GPP TSG-RAN4 Meeting #92	R4-1909703

# Ljubljana, SI, 26th – 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6631 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to interruption requirements due to measurement on dormant SCell R15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_euCA-Core |  |  |  |  |  | Date: |  |  | 2019-06-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently the interruption for dormant SCell measurement is only defined for CQI measurement, but RRM measurement could also cause interruption. In addition, the length of interruption is not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update interruption requirements due to measurement on dormant SCell to include both RRM and CQI measurement. The length of interruption is also specified. Since MBSFN on active serving cell is not considered, the interruption length should be 1ms for intra-band CA and 2ms or inter-band CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interruption requirements due to measurement on dormant SCell are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.8.2.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.8.2.17	Interruptions during CQI and RRM measurement on dormant SCell

For a UE capable of supporting dormantSCellState [2] and configured with one or more number of dormant SCell(s), an interruption on PCell or any activated SCell(s) due to the periodic CQI measurements and RRM measurement of the dormant SCell(s) is allowed with up to 0.5% probability of missed ACK/NACK.

Each interruption shall not exceed

-	1 subframe, if the PCell or the activated SCell is not in the same band as any of the dormant SCells, or

-	2 subframes, if the PCell or the activated SCell is in the same band as any of the FDD dormant SCells, and the PCell or the activated SCell is not configured with the MBSFN subframes;

-	5 subframes, otherwise.

The interruption is for both uplink and downlink of PCell and activated SCell(s).

<End of Change 1>