3GPP TSG-RAN4 Meeting #92	R4-1909707

# Ljubljana, SI, 26th – 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6635 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding interruption length requirements for dormant state transition R15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_euCA-Core |  |  |  |  |  | Date: |  |  | 2019-06-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The state transition around dormat SCell includes 4 types - Dormant -> activated - Dormant -> de-activated - Activated -> dormant - De-activated -> dormant The interruption requirements are currently only partially specified for the first two types. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete the interruption requirements for the first 2 types of transitions. Add new requriements for the last 2 types of transitions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interruption requirements for state transitions around dormant SCell are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.8.2.14-16, new section 7.8.2.16a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.8.2.14	Interruptions at SCell activation/deactivation of dormant SCell for intra-band CA

When an intra-band FDD dormant SCell is activated or deactivated as defined in TS 36.321 [17] the UE capable of supporting dormantSCellState [2] is allowed an interruption of up to 2 subframes on PCell during the activation/deactivation defined in Section 7.7, provided that MBSFN subframes are not configured in the PCell. This interruption is for both uplink and downlink of PCell.

When an intra-band TDD dormant SCell is activated or deactivated as defined in TS 36.321 [17] the UE capable of supporting dormantSCellState [2] is allowed an interruption of up to 5 subframes on PCell during the activation/deactivation defined in Section 7.7, provided that MBSFN subframes are not configured in the PCell. This interruption is for both uplink and downlink of PCell.

7.8.2.15	Interruptions at SCell activation/deactivation of dormant SCell for inter-band CA

When an inter-band dormant SCell is activated or deactivation as defined in TS 36.321 [17] the UE that is capable of supporting dormantSCellState [2] and requires interruption is allowed an interruption of up to 1 subframe on PCell during the activation/deactivation defined in Section 7.7. This interruption is for both uplink and downlink of PCell.

7.8.2.16	Interruptions at SCell activation/deactivation of multiple dormant SCells

When any number of SCells in a dormant state between one and six is activated or deactivated using the same MAC control element as defined in TS 36.321 [17], the UE capable of supporting dormantSCellState [2] is allowed an interruption on PCell and on any activated SCell during the SCell activation/deactivation procedure [17] as follows:

-	an interruption on PCell:

-	of up to 1 subframe, if the PCell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to 2 subframes, if the PCell is in the same band as any of the FDD SCells being activated or deactivated, and the PCell is not configured with the MBSFN subframes;

-	of up to 5 subframes, otherwise.

-	an interruption on any activated SCell:

-	of up to 1 subframe, if the activated SCell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to 2 subframes, if the activated SCell is in the same band as any of the FDD SCells being activated or deactivated and the activated SCell is not configured with the MBSFN subframes.

-	of up to 5 subframes, otherwise.

7.8.2.16a	Interruptions at SCell hibernation

When any number of SCells between one and six in a activated or deactivated state is hibernated using the same MAC control element as defined in TS 36.321 [17], the UE capable of supporting dormantSCellState [2] is allowed an interruption on PCell and on any activated SCell during the SCell activation procedure [17] as follows:

-	an interruption on PCell:

-	of up to 1 subframe, if the PCell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to 2 subframes, if the PCell is in the same band as any of the FDD SCells being activated or deactivated, and the PCell is not configured with the MBSFN subframes;

-	of up to 5 subframes, otherwise.

-	an interruption on any activated SCell:

-	of up to 1 subframe, if the activated SCell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to 2 subframes, if the activated SCell is in the same band as any of the FDD SCells being activated or deactivated and the activated SCell is not configured with the MBSFN subframes.

-	of up to 5 subframes, otherwise.

<End of Change 1>