3GPP TSG-RAN WG4 RAN4#92	R4-1909713

Ljubljana, Slovenia, August 26th – August 30th, 2019

Agenda item:	9.1.5.12

Source: 	Qualcomm Incorporated

Title: 	Timing Requirements in NR-U

Document for:	Discussion

# Introduction

3GPP has started discussing NR based access to unlicensed spectrum (NR-U) [1]. NR-U standard will allow UE and gNB to access unlicensed spectrum with a NR based standard. While trying to access the spectrum, the UE and gNB may need to back off to meet listen-before-talk (LBT) requirements [1, 2]. Current RRM requirements, as defined in 38.133 [3], are defined for licensed spectrum and do not consider the scenarios of unlicensed spectrum. This contribution focuses on some aspects of UL RRM in NR-U, e.g., UL timing accuracy.

# Initial UL transmission timing accuracy requirements of Rel-15

Rel-15 provides the following text [3] while defining the initial UL transmission timing requirement of the UE.

“The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS or it is the PRACH transmission.

The UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus .

……..

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

“

The UL transmission timing accuracy ensures that the gNB can decode the UL transmission of the UE. UE needs a periodic DL reference signal so that it can run its time tracking loop and maintain UL transmission timing accuracy requirements. As the above text shows, Rel-15 enforces UE to maintain its initial UL transmission accuracy requirement provided that at least one SSB is available at the UE during the last 160 ms.

Observation 1: In Rel-15, UE is obligated to maintain its initial UL transmission accuracy requirement provided that at least one SSB is available at the UE during the last 160 ms.

# Backoff due to LBT in Unlicensed Spectrum

During one of the previous RAN1 meetings, following was agreed [2]

Agreement: For initiation of a gNB transmission: - LBT other than Cat 4 is not used for DRS multiplexed with unicast data - LBT other than Cat 4 is not used for PDCCH and/or PDSCH transmission outside of DRS. Note: - This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

The above agreement suggests that gNB may have to back off before transmitting discovery reference signals to meet LBT requirement. Hence, network may not be able to transmit discovery reference signals with a fixed periodicity and ensure that at least one discovery reference signal is transmitted within 160 ms prior to an UL transmission.

As a result, UE will not be able to maintain the initial UL transmission timing accuracy requirement, as defined in table 7.1.2-1 of 38.133.

Observation 2:  In NR-U networks, gNB may have to back off before transmitting discovery reference signals to meet LBT requirement.

Observation 3: In NR-U networks, network may not be able to ensure that at least one discovery reference signal is transmitted within 160 ms prior to an UL transmission.

- As a result, UE may not be able to maintain the UL transmission timing accuracy requirement, as defined in table 7.1.2-1 of 38.133.

# UL transmission accuracy in NR-U Networks

Specification needs to handle the scenarios where UE may not be able to maintain the UL transmission timing accuracy requirement if no discovery reference signal were transmitted during the last 160 ms. Several solutions can be considered:

- Option 1- Relaxation of UL transmission timing accuracy requirements based on the unavailability of discovery reference signal:

In this option, specification may define different sets of UL transmission timing accuracy requirements. Depending on the availability of discovery reference signal, one out of these multiple UL transmission timing accuracy requirements will be applied.

Assuming that the discovery reference signals were available during the last X ms, the correlation between the value of X and UL transmission timing error limit Te may look like the following:

| X | UL transmission timing error | Comment |
| --- | --- | --- |
| 160 | Te1 | Same as Te, as defined in table 7.1.2-1 of 38.133 |
| 320 | Te2 | More relaxed than Te1 |
| 640 | Te3 | More relaxed than Te2 |
| … | … | … |

In this option, network will need to be able to decode UL signals with different initial UL transmission timing accuracies based on the unavailability of DL reference signal.

- Option 2- Cancellation of UL transmission timing accuracy requirements based on the unavailability of discovery reference signal:

In this option, specification may simply cancel the UL transmission timing accuracy requirement if no discovery reference signal were available during the last 160 ms. In other words, UE will fulfil UL transmission timing accuracy requirement only if a discovery reference signal were available during the last 160 ms. If no DL discovery reference signal were available, UE does not have to maintain any UL transmission timing accuracy requirement.

- Option 3 – Not scheduling UL transmission if no discovery reference signal were available during the last Y ms:

In option 2, UE will not be mandated to maintain any UL transmission timing accuracy if no discovery reference signal were available during the last 160 ms. In many cases, gNB will not be able to decode UL transmission from the UE in these cases. Hence, instead of option 2, UE may simply skip transmission of non-scheduled UL when there were no DL RS available during the last Y ms.

- Option 4 – Transmitting PRACH based on last available discovery reference signal if no discovery reference signal were available during the last Z ms:

In this option, UE will cancel all uplink transmissions and transmit PRACH based on the last available DL RS if it doesn’t detect any DL RS during the last Z ms. This will allow UE to receive an updated timing command from gNB in Msg2.

A combination of these different options may also be needed to tackle different scenarios.

Proposal 1: RAN4 discusses following options based on the unavailability of discovery reference signal in NR-U networks:

- UL transmission timing accuracy requirement gets relaxed if no DL RS is available during the last 160 ms

  - Detailed values of relaxed timing accuracy requirements are FFS

- UL transmission timing accuracy requirement is not applicable if no DL RS is available during the last X ms

  - Value of X is FFS

- UE doesn’t transmit non-scheduled UL if no DL RS is available during the last X ms.

  - Value of X is FFS.

- UE transmits RACH and cancels all UL transmissions if no DL RS is available during the last Y ms.

  - Values of Y is FFS.

# Maximum receive and transmission timing difference requirements in NR-U Networks

The maximum receive timing difference (MRTD) and maximum transmission timing difference requirements (MTTD) are designed based on cell synchronization requirements and cell deployment assumptions. NR-U networks will be mostly deployed in small cells. Rel-15 requirements were designed assuming several possible cell deployment scenarios, including small cells. Hence, MRTD and MTTD requirements in NR-U networks can remain as those of Rel-15.

Proposal 2: MRTD and MTTD requirements in NR-U networks are same as those of Rel-15 requirements.

# Conclusion

Observation 1: In Rel-15, UE is obligated to maintain its initial UL transmission accuracy requirement provided that at least one SSB is available at the UE during the last 160 ms.

Observation 2:  In NR-U networks, gNB may have to back off before transmitting discovery reference signals to meet LBT requirement.

Observation 3: In NR-U networks, network may not be able to ensure that at least one discovery reference signal is transmitted within 160 ms prior to an UL transmission.

- As a result, UE may not be able to maintain the initial UL transmission timing accuracy requirement, as defined in table 7.1.2-1 of 38.133.

Proposal 1: RAN4 discusses following options based on the unavailability of discovery reference signal in NR-U networks:

- UL transmission timing accuracy requirement gets relaxed if no DL RS is available during the last 160 ms

  - Detailed values of relaxed timing accuracy requirements are FFS

- UL transmission timing accuracy requirement is not applicable if no DL RS is available during the last X ms

  - Value of X is FFS

- UE doesn’t transmit non-scheduled UL if no DL RS is available during the last X ms.

  - Value of X is FFS.

- UE transmits RACH and cancels all UL transmissions if no DL RS is available during the last Y ms.

  - Values of Y is FFS.

Proposal 2: MRTD and MTTD requirements in NR-U networks are same as those of Rel-15 requirements.

# References

- 3GPP TR 38.889 V16.0.0, Study on NR-based access to unlicensed spectrum.

- Chairman’s notes, 3GPP TSG RAN WG1 Meeting #96

- 3GPP TS38.133 V15.4.0, Requirements for support of radio resource management (Release 15)