3GPP TSG-RAN WG4 Meeting #92	R4-1909722

# Ljubljana, SI, 26th– 30th Aug, 2019

Source: 	Huawei, HiSilicon

Title:	TP for R16 MTC TR: PRB Alignment

Agenda Item:	8.11.2

Document for:	Approval

# 1	Introduction

In last RAN4#92 meeting, a WF for Rel-16 LTE-MTC coexistence with NR is agreed [1], where it specifies that the PRB alignment should be captured in the TR. This paper proposes the TP for this topic.

# 2	References

[1] R4-1907849, WF for Rel-16 LTE-MTC coexistence with NR

# 3	Text Proposal

## <Start of Text Proposal>

## 7.2 PRB alignment

LTE-MTC reuses LTE PSS/SSS/PBCH channel for initial access, which occupies the central 6 PRBs plus unused DC sub-carrier. Consider the case of SCS=15 kHz and channel raster=100 kHz for both NR and LTE-MTC. A total of 7 consecutive NR PRBs in the DL need to be reserved for the LTE synchronisation signals owing to the unused DC sub-carrier. When the LTE-MTC narrowband does not include the DC sub-carrier, the LTE-MTC PRBs may be aligned with NR PRBs if the following conditions are satisfied:

- The LTE DC sub-carrier coincides with the first sub-carrier of a NR PRB;

- The LTE-MTC narrowband is allocated to the lower side of the DC sub-carrier.

## <End of Text Proposal>