3GPP TSG- RAN4 Meeting #92	R4-1909739

# Ljubljana, Slovenia, 26th - 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Parameters, Test case A.7.7.1.1 FR2 Intra-frequency SS-RSRP accuracy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Many of the OTA related test parameters in Test case A.7.7.1.1 FR2 Intra-frequency SS-RSRP accuracy are TBD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the OTA related test parameters in Test case A.7.7.1.1: - It is only possible to have two useful sub-tests, one with Io close to the maximum of -50dBm, and one with SSB_RP close to minimum - Both sub-tests are set with Cell 2 Es/IotBB close to the minimum value of -6dB - In the sub-test with high Io, applied noise Noc and signal Es are used to give a well-defined SNR - In the sub-test with SSB_RP close to minimum, Es only is used, to replicate the condition used in the core requirement - Values are provided for Power class 3 bands (NR_TDD_FR2_T and NR_TDD_FR2_Y - Es/Noc is moved into the OTA related test parameters table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case A.7.7.1.1 FR2 Intra-frequency SS-RSRP accuracy remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.7.1.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

## A.7.7	Measurement Performance requirements

### A.7.7.1	SS-RSRP

#### A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

##### A.7.7.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.2.1.1and 10.1.2.1.1 for intra-frequency measurements.

##### A.7.7.1.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.7.7.1.1.2-1. Both absolute and relative accuracy of SS-RSRP intra frequency measurements are tested by using the parameters in Table A.7.7.1.1.2-2 and A.7.7.1.1.2-3. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. The TCI status for Cell 1 is defined in Table [TBD] and TRS configuration for Cell 1 is defined in Table [TBD].

Table A.7.7.1.1.2-1: SS-RSRP Intra frequency SS-RSRP supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.1.1.2-2: SS-RSRP Intra frequency general test parameters

| Parameter | Unit | Test 1 |  |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 |  |  | Cell 1 | Cell 2 |
| Cell ID |  | TBD | TBD |  |  | TBD | TBD |
| SSB ARFCN |  | freq1 |  |  |  | freq1 |  |
| Duplex mode |  | TDD |  |  |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  |  |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  |  |  | 100: NRB,c = 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 | - |  |  | DLBWP.0.1 | - |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 | - |  |  | DLBWP.1.1 | - |
| Uplink initial BWP configuration |  | ULBWP.0.1 | - |  |  | ULBWP.0.1 | - |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 | - |  |  | ULBWP.1.1 | - |
| DRX cycle configuration |  | Not applicable | - |  |  | Not applicable | - |
| TRS configuration |  | TRS.2.1 TDD | - |  |  | TRS.2.1 TDD | - |
| TCI state |  | TCI.State.0 | - |  |  | TCI.State.0 | - |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - |  |  | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - |  |  | CR.3.1 TDD | - |
| Control channel RMC |  | CCR.3.1 TDD | - |  |  | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 |  |  | OP.1 | OP.1 |
| SSB configuration |  | SSB.1 FR2 | SSB.1 FR2 |  |  | SSB.1 FR2 | SSB.1 FR2 |
| SMTC configuration |  | SMTC.1 | SMTC.1 |  |  | SMTC.1 | SMTC.1 |
| Time offset with Cell 1 | s | - | 3 |  |  | - | 3 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 |  |  | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |  |  | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Propagation conditions |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Void Note 4: Void |  |  |  |  |  |  |  |

Table A.7.7.1.1.2-3: SS-RSRP Intra frequency OTA related test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  |  | According to section A.3.15.1 |  | According to section A.3.15.1 |  |
| Note1 |  | dBm/15kHzNote4 | -95.53 |  | N/A |  |
| Note1 |  | dBm/SCSNote4 | -86.5 |  | N/A |  |
|  |  | dB | 5.0 | 0.5 | N/A |  |
| Es | NR_TDD_FR2_A | dBm/SCSNote4 | N/A |  |  |  |
|  | NR_TDD_FR2_B |  |  |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |  |  |
|  | NR_TDD_FR2_T |  |  |  | -109.0 | -109.0 |
|  | NR_TDD_FR2_Y |  |  |  | -106.4 | -106.4 |
| SSB_RPNote2 | NR_TDD_FR2_A | dBm/SCS | -81.5 | -86.0 |  |  |
|  | NR_TDD_FR2_B |  |  |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |  |  |
|  | NR_TDD_FR2_T |  |  |  | -109.0 | -109.0 |
|  | NR_TDD_FR2_Y |  |  |  | -106.4 | -106.4 |
| BB Note6 | NR_TDD_FR2_A | dB |  |  | -5.98 | -5.98 |
|  | NR_TDD_FR2_B |  |  |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |  |  |
|  | NR_TDD_FR2_T |  | 1.70 | -5.71 |  |  |
|  | NR_TDD_FR2_Y |  | 1.67 | -5.72 |  |  |
| IoNote2 | NR_TDD_FR2_A | dBm/95.04 MHz Note4 | -50.28 |  |  |  |
|  | NR_TDD_FR2_B |  |  |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |  |  |
|  | NR_TDD_FR2_T |  |  |  | -77.0 |  |
|  | NR_TDD_FR2_Y |  |  |  | -74.0 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Void Note 6: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19] |  |  |  |  |  |  |

##### A.7.7.1.1.3	Test Requirements

The SS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in clauses 10.1.3.1.1 and relative accuracy requirements in clause 10.1.3.1.2.

<< End of changes >>