3GPP TSG-RAN WG4 Meeting #92	R4-1909742

# Ljubljana, Slovenia, August 26th – 30th , 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for UE transmit timing test case in EN-DC FR2 (Section A.5.4.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Editorial corrections - Added TRS configuration - Added OTA params table - Changed SRS periodicity to align with DRx |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editorial corrections - Added TRS configuration - Added OTA params table - Changed SRS periodicity to align with DRx |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test confgurrations for UE transmit timing test cases will be incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR…CR... TS38.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### Start of changes

#### A.5.4.1.1	NR UE Transmit Timing Test for FR2

##### A.5.4.1.1.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

Supported test configurations are shown in Table 5.4.1.1.1-1.

Table A.5.4.1.1.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz |
| 2 | LTE TDD, NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz |

The test consists of E-UTRA PCell and NR PSCell. The configuration for E-UTRA is given in A.3.7.2.1. Tables A.5.4.1.1.1-2 and A.5.4.1.1.1-2A define the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.5.4.1.1.1-3.

Table A.5.4.1.1.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  |  | Test2 |  | Band Group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1,2 | Freq1 |  |  | Freq1 |  |  |
| Duplex Mode |  | 1,2 | TDD |  |  |  |  |  |
| TDD configuration |  | 1,2 |  | TDDConf.1.2 |  |  |  |  |
| BWchannel | MHz | 1,2 | 100: NRB,c = 66 |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |
| TRS Configuration |  | 1,2 | TRS.2.1 TDD |  |  |  |  |  |
| TCI State |  | 1,2 | CSI-RS.Config.0 |  |  |  |  |  |
| DRx Cycle | ms | 1,2 | N/A |  |  |  | DRX.5Note5 |  |
| PDSCH Reference measurement channel |  | 1,2 | SR.3.1 TDD |  |  |  |  |  |
| CORESET Reference Channel |  | 1,2 | CR.3.1 TDD |  |  |  |  |  |
| OCNG Patterns |  | 1,2 | OCNG pattern 1 |  |  |  |  |  |
| SSB Configuration |  | 1,2 | SSB.2 FR2 |  |  |  |  |  |
| SMTC Configuration |  | 1,2 | SMTC.1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2 | 120 |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2 | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Propagation condition |  | 1,2 | AWGN |  |  |  |  |  |
| SRS Config |  | 1,2 | Config1Note6 |  | Config2Note6 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRx related parameters are given in Table A.3.3.5-1 Note 6: SRS configs are given in Table A.5.4.1.1.1-3 |  |  |  |  |  |  |  |  |

Table A.5.4.1.1.1-2A: OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | According to section A.3.15.1 |  |
| Note1 | dBm/15kHzNote4 | -112 |  |
| Note1 | dBm/SCSNote3 | -103 |  |
|  | dB | 4 |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -99 |  |
|  | dB | 4 |  |
| IoNote2 | dBm/95.04 MHz Note4 | -68.5 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone |  |  |  |

Table A.5.4.1.1.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | Config1 | Config 2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | sl1 | sl1 |  |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1,0 | sl2560,0 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | Any 10 bit number |

Table A.5.4.1.1.1-4: Void

##### A.5.4.1.1.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1)	Set up E-UTRA PCell according to parameters given in Table A.3.7.2.2-1 and setup NR PSCell according to parameters given in Table A.5.4.1.1.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB.

a.	The NTA offset value (in Tc units) is 13792

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in Table A.5.4.1.1.2-1

Table A.5.4.1.1.2-1 Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 240 | +8*64Tc | +4*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in Section 7.1.2 Table 7.1.2-3 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first detected path (in time) of DL SSB. Skip this step for test 2 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment

End of changes