3GPP TSG-RAN WG4 Meeting #92	R4-1909766

# Ljubljana, Slovenia, August 26th – 30th , 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for L1-RSRP performance test case in EN-DC FR2 (Section A.5.7.4) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Editorial corrections - Requirement statement in SSB based RSRP was incorrect - Removed TBD’s in OTA params |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editorial corrections - Requirement statement in SSB based RSRP corrected - Removed TBD’s in OTA params |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test confgurrations for L1-RSRP performance test cases will be incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR…CR... TS38.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### Start of changes

### A.5.7.4	L1-RSRP measurement for beam reporting

#### A.5.7.4.1	SSB based L1-RSRP measurement

##### A.5.7.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.5.2 and section 10.1.20.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.5.7.4.1.1-1.

The AoA setup for this test is Setup 1 as defined in section A.3.15.

Table A.5.7.4.1.1-1: Applicable NR configurations for FR2 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.5.7.4.1.2	Test parameters

In this set of test cases there are two cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2). The test parameters and applicability for Cell 1 are defined in A.3.7.2. The test parameters for the Cell 2 are given in Table A.5.7.4.1.2-1 and Table A.5.7.4.1.2-2 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.5.7.4.1.2-1 and Table A.5.7.4.1.2-2.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.5.7.4.1.2-1: FR2 SSB based L1-RSRP general test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~4 |  | freq1 | freq1 |
| Duplex mode | 1~4 |  | TDD | TDD |
| TDD Configuration | 1~4 |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | 1~4 | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1~4 |  | SR.3.1 TDD | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1~4 |  | CR.3.1 TDD | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1~4 |  | CCR.3.1 TDD | CCR.3.1 TDD |
| SSB configuration | 1,2 |  | SSB.1 FR2 | SSB.1 FR2 |
|  | 3,4 |  | SSB.2 FR2 | SSB.2 FR2 |
| OCNG Patterns | 1~4 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.3 ULBWP.1.3 | DLBWP.1.3 ULBWP.1.3 |
| TRS Configuration | 1~4 |  | TRS.2.1 TDD | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~4 |  | TCI.State.2 | TCI.State.2 |
| SMTC configuration | 1~4 |  | SMTC.1 | SMTC.1 |
| reportConfigType | 1~4 |  | periodic | periodic |
| reportQuantity | 1~4 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS | 1~4 |  | 2 | 2 |
| L1-RSRP reporting period | 1~4 |  | slot640 | slot640 |
| Propagation condition | 1~4 |  | AWGN | AWGN |
| EPRE ratio of PSS to SSS | 1~4 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

Table A.5.7.4.1.2-2: FR2 SSB based L1-RSRP OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 NOTE 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SSB0 | SSB1 | SSB0 | SSB1 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  |
|  | 1~4 | dBm/15kHz | -100 |  | n.a. |  |
|  | 1,2 | dBm/SSB SCS | -91 |  | n.a. |  |
|  | 3,4 |  | -88 |  | n.a. |  |
|  | 1~4 | dB | 10 | -2 | n.a. |  |
| SS-RSRPNote1 | 1,2 | dBm/SCS | -81 | -93 | As in Table B.2.4-2 |  |
|  | 3,4 |  | -78 | -90 | As in Table B.2.4-2 |  |
| IoNote1 | 1~4 | dBm/ 95.04MHz | -51.57 |  | SS-RSRP+28.98 |  |
|  | 1~4 | dB | 10 | -2 | n.a. |  |
| Note 1: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 3: No additional noise is added by the test system in Test 2. |  |  |  |  |  |  |

##### A.5.7.4.1.3	Test Requirements

The L1-RSRP measurement accuracy for SSB#0 and SSB#1 of Cell 2 shall fulfil the requirements in sections 10.1.20.1.

. The reported L1-RSRP value shall include the Rx antenna gain in the range of TBD.

#### A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off

##### A.5.7.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.5.3 and section 10.1.20.2 for L1-RSRP measurements based on CSI-RS with the testing configurations for NR cells in Table A.5.7.4.2.1-1.

The AoA setup for this test is Setup 1 as defined in section A.3.15.

Table A.5.7.4.2.1-1: Applicable NR configurations for FR2 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.5.7.4.2.2	Test parameters

In this set of test cases there are two cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2). The test parameters and applicability for Cell 1 are defined in A.3.7.2. The test parameters for the Cell 2 are given in Table A.5.7.4.2.2-1 and Table A.5.7.4.2.2-2 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.5.7.4.2.2-1 and Table A.5.7.4.2.2-2.

There is no measurement gap configured in the test. Before the test, UE is configured one CSI-RS resource set with two CSI-RS resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB.

Table A.5.7.4.2.2-1: FR2 CSI-RS based L1-RSRP general test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 | freq1 |
| Duplex mode | 1~2 |  | TDD | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1~2 |  | SR.3.1 TDD | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1~2 |  | CR.3.1 TDD | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1~2 |  | CCR.3.1 TDD | CCR.3.1 TDD |
| SSB configuration | 1~2 |  | SSB.1 FR2 | SSB.1 FR2 |
| OCNG Patterns | 1~2 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 | DLBWP.1.3 ULBWP.1.3 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 | TCI.State.2 |
| SMTC configuration | 1~2 |  | SMTC.1 | SMTC.1 |
| CSI-RS | 1~2 |  | CSI-RS.3.2 TDD | CSI-RS.3.2 TDD |
| reportConfigType | 1~2 |  | periodic | periodic |
| reportQuantity | 1~2 |  | cri-RSRP | cri-RSRP |
| Number of reported RS | 1~2 |  | 2 | 2 |
| L1-RSRP reporting period | 1~2 |  | slot640 | slot640 |
| Propagation condition | 1~2 |  | AWGN | AWGN |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

Table A.5.7.4.2.2-2: FR2 CSI-RS based L1-RSRP OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 NOTE 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CSI-RS0 | CSI-RS1 | CSI-RS0 | CSI-RS1 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  |
|  | 1~2 | dBm/15kHz | -100 |  | n.a. |  |
|  | 1~2 | dBm/SSB SCS | -91 |  | n.a. n.a. |  |
|  | 1~2 | dB | 10 | -2 | n.a. |  |
| CSI-RS-RSRPNote1 | 1~2 | dBm/SCS | -81 | -93 | As in Table B.2.4-2 |  |
| IoNote1 | 1~2 | dBm/ 95.04MHz | -59.86 |  | SS-RSRP+28.98 |  |
|  | 1~2 | dB | -51.57 | -2 | n.a. |  |
| Note 1: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 3: No additional noise is added by the test system in Test 2. |  |  |  |  |  |  |

##### A.5.7.4.2.3	Test Requirements

The L1-RSRP measurement accuracy for CSI-RS#0 and CSI-RS#1 of Cell 2 shall fulfil the requirements in sections 10.1.20.2. The reported L1-RSRP value shall include the Rx antenna gain in the range of TBD.

End of changes