3GPP TSG-RAN WG4 Meeting #92	R4-1909813

Ljubljana, Slovenia, 26 August – 30 August 2019

Source:	Ericsson, AT&T

Title:	TP for TR 37.716-21-11: 12A-66A_n2A and 12A-66A-66A_n2A

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

Rel-16 LTE-NR DC combinations LTE 2 band + NR 1 band combination are defined in the WID [1]. This contribution is a text proposal for TR 37.716-21-11 to add 12A-66A_n2A and 12A-66A-66A_n2A.

# 2. Text Proposal

# ---Start of changes---

## 5.1.x	12A-66A_n2A
12A-66A-66A_n2A

### 5.1.x.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 12-66_n2 | CA_12-66 | n2 | DC_66_n2 |

### 5.1.x.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_12A-66A_n2A | DC_12A_n2A DC_66A_n2A | CA_12A-66A | n2 |
| DC_12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A | CA_12A-66A-66A | n2 |

### 5.1.x.3		∆TIB and ∆RIB values

For DC_12-66_n2, the TIB,c and RIB,c values are reused from the LTE combination CA_2-12-66, and are given in the tables below.

Table 5.1.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n2 | 12 | 0.8 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-66_n2 | 12 | 0.5 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

### 5.1.x.4	REFSENS requirements

Co-existence analysis from DC_12_n2 shows that there is no impact from DC_12_n2 UL to Band 66 DL.

Co-existence analysis from DC_66_n2 shows that there is no impact from DC_66_n2 UL to Band 12 DL.

---End of changes---

# Reference

[1]	RP-191124, “Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei