3GPP TSG-RAN WG4 Meeting #92	R4-1909819

Ljubljana, Slovenia, 26 August – 30 August 2019

Source:	Ericsson, AT&T

Title:	TP for TR 37.716-21-11: 14A-66A_n260M and 14A-66A-66A_n260M

Agenda item:			10.4.3

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.716-21-11 to add 14A-66A_n260M and 14A-66A-66A_n260M combinations as defined in the WID [1] for 2 LTE + 1 NR.

# 2. Text Proposal

# ---Start of changes---

## 5.2.x	DL_14A-66A_n260M_UL_14A_n260M
DL_14A-66A_n260M_UL_66A_n260M
DL_14A-66A-66A_n260M_UL_14A_n260M
DL_14A-66A-66A_n260M_UL_66A_n260M

### 5.2.x.1	Operating bands for EN-DC

Table 5.2.x.1-1 Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_14-66_n260 | CA_14-66 | n260 | No |
| DC_14-66-66_n260 | CA_14-66-66 | n260 | No |

### 5.2.x.2	Configuration for DC

Table 5.2.x.2-1: Inter-band EN-DC configurations including FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_14A-66A_n260A DC_14A-66A_n260G DC_14A-66A_n260H DC_14A-66A_n260I DC_14A-66A_n260J DC_14A-66A_n260K DC_14A-66A_n260L DC_14A-66A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M | CA_14A-66A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_14A-66A-66A_n260A DC_14A-66A-66A_n260G DC_14A-66A-66A_n260H DC_14A-66A-66A_n260I DC_14A-66A-66A_n260J DC_14A-66A-66A_n260K DC_14A-66A-66A_n260L DC_14A-66A-66A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M | CA_14A-66A-66A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |

### 5.2.x.3	∆TIB and ∆RIB values

Table 5.2.x.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-66_n260, DC_14-66-66_n260 | 14 | 0.3 |
|  | 66 | 0.3 |
|  | n260 | 0 |

Table 5.2.x.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_14-66_n260, DC_14-66-66_n260 | 14 | 0 |
|  | 66 | 0 |
|  | n260 | 0 |

---End of changes---

# Reference

[1]		RP-191124, “Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei