3GPP TSG-RAN WG4 Meeting #92	R4-1909829

Ljubljana, Slovenia, 26 August – 30 August 2019

Source:	Ericsson, Telstra

Title:	TP to TR 37.716.21-21: Addition of DC_28A_n5A-n78A

Agenda item:	10.8.2

Document for:	Approval

# Background

This contribution provides text proposal DC_28A_n5A-n78A as defined in the revised WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_28_n5-n78

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n5-n78 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_28A_n5A-n78A | DC_28A_n5A DC_28A_n78A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

### 6.x.3	Co-existence studies

Co-existence analysis for DC_28_n5 shows that there is IMD5 impact from DC_28_n5 UL to Band n78 DL.

Co-existence analysis for DC_28_n78 shows that there is IMD5 impact from DC_28_n78 UL to Band n5 DL.

### 6.x.4	∆TIB and ∆RIB values

For DC_28_n5-n78 the TIB,c and RIB,c values are reused from DC_18-28_n78 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n5-n78 | 28 | 0.5 |
|  | n5 | 0.5 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n5-n78 | n78 | 0.5 |

### 6.x.5	MSD

MSD values for impact from UL 28_n5 into DL n78 are reused from DC_3A_n28A-n78A and DC_3A_n40A-n78A who also have IMD5 impact to DL n78.

MSD values for impact from UL 28_n78 into DL n5 are reused from DC_18A-28A_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A_n5A-n78A | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n5 | 829 | 5 | 25 | 874 | 3.8 | IMD5 ¦fn78 -4*fB28¦ |
|  | n78 | 3721 | 10 | 50 | 3721 | N/A | N/A |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3721 | 10 | 50 | 3721 | 4.7 | IMD5 ¦fn5 +4*fB28¦ |

##### ---End of changes---

# Reference

[1]	RP-190949, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics