3GPP TSG-RAN WG4 Meeting #92	R4-1909898

# Ljubljana, Slovenia,  26-30 Aug. 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-3: Addition of MSD for CA_n25(2A) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sprint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 8-16-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | MSD is missing for CA_n25(2A) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds MSD for CA_n25(2A) based on MSD for CA_25A-25A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MSD for CA_n25(2A) would remain undefined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | To avoid potential Note number collisions, temporary note numbers are hightlighted and need to be replaced by the rapporteur. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

#### 7.3A.2.2	Reference sensitivity power level for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with FDL_low < 2700 MHz and FUL_low < 2700 MHz with one uplink carrier and two or more downlink sub-blocks, throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.2-1, Table 7.3.2-2, and Table 7.3A.2.2-1 with the reference sensitivity power level increased by D RIBNC  given in Table 7.3A.2.2-1 for the SCC(s). For aggregation of more than two downlink FDD carriers with one uplink carrier the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.2.2-1. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3-1) configured.

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n25(2A)v | 25RB+25RB | 30.0 < Wgap ≤ 55.0 | 10w | 5.0 | FDD |
|  |  | 0.0 < Wgap ≤ 30.0 | 25w | 0.0 |  |
|  | 25RB+50RB | 25.0 < Wgap ≤ 50.0 | 10w | 4.5 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 25w | 0.0 |  |
|  | 25RB+75RB | 20 < Wgap ≤ 45 | 10w | 4.3 |  |
|  |  | 0 < Wgap ≤ 20 | 25w | 0 |  |
|  | 25RB+100RB | 15 < Wgap ≤ 40 | 10w | 4.1 |  |
|  |  | 0 < Wgap ≤ 15 | 25w | 0 |  |
|  | 50RB+25RB | 15.0 < Wgap ≤ 50.0 | 10x | 5.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 32w | 0.0 |  |
|  | 50RB+50RB | 10.0 < Wgap ≤ 45.0 | 10x | 5.0 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 32w | 0.0 |  |
|  | 50RB+75RB | 5 < Wgap ≤ 40 | 10x | 4.5 |  |
|  |  | 0 < Wgap ≤ 5 | 32w | 0 |  |
|  | 50RB+100RB | 0 < Wgap ≤ 35 | 10x | 4.2 |  |
|  | 75RB+25RB | 10 < Wgap ≤ 45 | 10y | 7.6 |  |
|  |  | 0 < Wgap ≤ 10 | 32w | 0 |  |
|  | 75RB+50RB | 5 < Wgap ≤ 40 | 10y | 6.7 |  |
|  |  | 0 < Wgap ≤ 5 | 32w | 0 |  |
|  | 75RB+75RB | 0 < Wgap ≤ 35 | 10y | 5.6 |  |
|  | 75RB+100RB | 0 < Wgap ≤ 30 | 10y | 4.8 |  |
|  | 100RB+25RB | 0 < Wgap ≤ 40 | 12z | 8 |  |
|  | 100RB+50RB | 0 < Wgap ≤ 35 | 12z | 6.7 |  |
|  | 100RB+75RB | 0 < Wgap ≤ 30 | 12z | 6.1 |  |
|  | 100RB+100RB | 0 < Wgap ≤ 25 | 12z | 5.7 |  |
| CA_n41(2A) | NOTE 1 | NOTE 2 | NOTE 3 | 0.0 | TDD |
| CA_n66(2A) | NOTE 1 | NOTE 2 | NOTE 3, NOTE 4 | 0.0 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE v: v 15 kHz SCS is assumed for UL and DL. NOTE w: w refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE x: x refers to the UL resource blocks shall be located at RBstart=33. NOTE y: y refers to the UL resource blocks shall be located at RBstart=44. NOTE z: z refers to the UL resource blocks shall be located at RBstart=62. |  |  |  |  |  |

<End of Changes>