3GPP TSG-RAN WG4 Meeting #92	R4-1910042

# Ljubljana, Slovenia, August 26th – 30th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR TCI state switching requirements (section 8.10) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CR capturing the TCI state switch requirements - In the known condition for TCI state, TCI state command should be received within 1280ms of the last transmission of the RS resource for beam reporting or measurement for the target TCI state. - In the known condition for TCI state, the TCI state shall also remain detectable with the same Spatial Rx parameter during the TCI state switching period. - Only L1-RSRP measurement time in TL1-RSRP should be considered in unknown TCI state switching. TReport should be 0. - RAN4 should differentiate the TCI state switching requirement for QCL-TypeD and other QCL types. The TL1-RSRP-meas =0 when TCI state switching is for {QCL-TypeA, QCL-TypeB, QCL-TypeC}. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR captures missing parts of the TCI state switching delay requirements in section 8.10. - Update definition of known TCI state. - Update T L1-RSRP with TReport = 0. - Update requirement for L1-RSRP measurement time with different QCL type. - Update the wording |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Section 8.10 Active TCI state switching delay is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Next Modified Subclause

## 8.10	Active TCI state switching delay

### 8.10.1	Introduction

The requirements in this section apply for a UE configured with one or more TCI state configurations on serving cell in MR-DC or standalone NR. UE shall complete the switch of active TCI state within the delay defined in this section.

### 8.10.2	Known conditions for TCI state

The TCI state is known if it has been meeting the following conditions:

-	TCI state switch command is received within [1280] ms of the last transmission of the RS resource for beam reporting or measurement for the target TCI state

-	The UE has sent at least 1 measurement report for the target TCI state

-	The TCI state remain detectable during the TCI state switching period

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	SNR of the TCI state is ≥ -3dB

Otherwise, the TCI state is unknown.

### 8.10.3	MAC-CE based TCI state switch delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ THARQ +3 ms +TOk*(Tfirst-SSB + TSSB-proc). The UE shall be able to receive on the old TCI state until slot n+ THARQ +3 ms +TOk*(Tfirst-SSB).

Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.321 [7];

Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE;

TSSB-proc = 2 ms;

TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise.

If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ THARQ +3 ms + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc). The UE shall be able to receive on the old TCI state until slot n+ THARQ +3 ms+ TL1-RSRP +TOuk*(Tfirst-SSB).

Where T L1-RSRP is the time for L1-RSRP measurement for Rx beam refinement, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

TOuk = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement

TL1-RSRP_Measurement_Period_SSB = 0 for SSB in FR2 and TL1-RSRP_Measurement_Period_CSI-RS = 0 for CSI-RS in FR2, provided that the TCI state switching involves QCL-TypeA, QCL-TypeB or CQL-TypeC only.

During MAC CE based TCI state switch the UE is allowed an interruption due to one shot timing adjustment on the serving or any activated serving cells as defined in clause 8.2.

### 8.10.4	DCI based TCI switch delay

If the target TCI state is known, when a UE is configured with the higher layer parameter tci-PresentInDCI which is set as 'enabled' for the CORESET scheduling the PDSCH at slot n, UE shall be able to receive PDSCH or transmit PUSCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+timeDurationForQCL, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], the value of timeDurationForQCL is defined in TS 38.306 [14].

The known condition for TCI state defined in clause 8.10.2 is applied.

During DCI based TCI state switch the UE is allowed an interruption due to one shot timing adjustment on the serving or any activated serving cells as defined in clause 8.2.

### 8.10.5	RRC based TCI state delay

If the target TCI state is known, upon receiving PDSCH carrying RRC activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc). Where TRRC_processing is the RRC processing delay, Tfirst-SSB, TSSB-proc and TOk are as defined in clause 8.10.3. The UE is not required to receive PDCCH/PDSCH or transmit PUCCH/PUSCH until the end of switching period.

If the target TCI state is unknown, upon receiving PDSCH carrying RRC activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ TRRC_processing +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc) . Where TRRC_processing is the RRC processing delay, Tfirst-SSB and TOuk are as defined in clause 8.10.3. The UE is not required to receive PDCCH/PDSCH or transmit PUCCH/PUSCH until the end of switching period.

During RRC based TCI state switch the UE is allowed an interruption due to one shot timing adjustment on the serving or any activated serving cells as defined in clause 8.2.

### 8.10.6	Active TCI state list update delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive PDCCH to schedule PDSCH with the new target TCI state no later than slot n+ THARQ +3ms +TOk*(Tfirst-SSB + TSSB-proc). Where THARQ, Tfirst-SSB, TSSB-proc and TOk are as defined in clause 8.10.3.

Next Modified Subclause