3GPP TSG-RAN WG4 Meeting #92                                       	          R4-1910185

Ljubljana, Slovenia, 20th to 26th Aug. 2019

Agenda Item:	7.5.2.1

Source: 	MediaTek Inc.

Title: 	Feasibility of asynchronous operation of CA_n78-n79 MSD due to cross band isolation

Document for:	Approval

- 1. Introduction

The CA_n78-n79 has been introduced in Rel-15 NR CA combinations. It was not considered to have simultaneous RX/TX operation when the CA combination was approved. The contribution is to revisit the combination whether there should be MSD due to cross band isolation when considering simultaneous RX/TX operation.

- 2. Discussion

Since n78 and n79 are TDD bands, the REFSENS was defined not including impairments due to TX side [3][4]. So MSD due to TX impairments for CA_n78-n79 operating with simultaneous RX/TX shall be revisited [2]. The analysis of separate antenna and shared antenna are performed below.

2.1 Separate antenna:

Figure 2.1-1 Reference architecture for CA_n78-n79 MSD with separate antenna

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| n79 filter rejection on n78 | [25] | dB |
| n78 filter rejection on n79 | [25] | dB |
| n78/n79 front-end loss | 4 | dB |
| Thermal noise at n78/n79 RX ANT port | -165.5 | dBm/Hz |
| Typical transmitter / receiver phase noise | -150 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 2.1-1 Typical receiver performance parameters for MSD analysis

In ref[2], the total isolation was assumed more than 55dB. We surveyed filter vendors, listed in table 2.1-2 and table 2.1-3, and found that the assumption is not realistic filter performance today. The filter rejection of n78/n79 from vendors and TR 38.814 are listed below:

| n78 Filter |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 4.4-5.0 [GHz] | TR38.813 | Vendor A | Vendor B | Vendor C | Vendor D |
| Attenuation [dB] | 22 | 10 | 23 | N/A | >32 |

Table 2.1-2 n78 Filter out of band rejection performance parameters for MSD analysis

| n79 Filter |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.3-3.8 [GHz] | TR38.814 | Vendor A | Vendor B | Vendor C | Vendor D |
| Attenuation [dB] | >15 | 8 | 23 | >28 | >35 |

Table 2.1-3 n79 Filter out of band rejection performance parameters for MSD analysis

From data from filter vendors, we have observation 1 as below,

Observation 1: From table 2.1-2 and 2.1-3, it is very difficult to achieve 40dB minimum isolation for filter vendors today.

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| n79 TX power at antenna port (Primary path) | 23 | 23 | dBm |
| n79 PA output noise power at n78 freq | -130 | -130 | dBm/Hz |
| n79 noise power at n78 PRX ANT port | -92.7 | NA | dBm/10MHz |
| n79 noise power at n78 DRX ANT port | NA | -92.7 | dBm/10MHz |
| Combined thermal noise at RX ant port | -159.5 |  | dBm/Hz |
| TX IM2 in n78 LNA input port | -76 | -76 | dBm |
| Total noise level refer to receiver LNA input port | -75.8 | -75.8 | dBm |
| Combined |  |  |  |
| MSD (10MHz BW) | 18.3 |  | dB |

Table 2.1-4 Link analysis for n78 10MHz REFSENS calculation

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| n78 TX power at antenna port (Primary path) | 23 | 23 | dBm |
| n78 PA output noise power at n79 freq | -130 | -130 | dBm/Hz |
| n78 noise power at n79 PRX Filter-ANT port | -86.2 | NA | dBm/40MHz |
| n78 noise power at n79 DRX Filter-ANT port | NA | -86.2 | dBm/40MHz |
| Combined thermal noise at RX ant port | -159.5 |  | dBm/Hz |
| TX IM2 in n79 LNA input port | -76 | -76 | dBm |
| Total noise level refer to receiver LNA input port | -75.1 | -75.1 | dBm |
| Combined |  |  |  |
| MSD (40MHz BW) | 12.6 |  | dB |

Table 2.1-5 Link analysis for n79 40MHz REFSENS calculation

From the above analysis, we can see large MSD due to cross band isolation. Even with best isolation numbers of the filters (32dB for n78 filter and 35dB for n79 filter), the MSD on n78/n79 would still be high to 10.9/5.9 dB.

Observation 2: Separate antenna would result in large MSD for CA_n78-n79 with simultaneous RX/TX operation.

2.2 Shared antenna:

Figure 2.2-1 Reference architecture for CA_n78-n79 MSD with Shred antenna

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| Diplexer out of band isolation | 15 | dB |
| n79 filter rejection on n78 | [25] | dB |
| n78 filter rejection on n79 | [25] | dB |
| n78/n79 front-end loss | 4 | dB |
| Thermal noise at n78/n79 RX ANT port | -165.5 | dBm/Hz |
| Typical transmitter / receiver phase noise | -150 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 2.2-1 Typical receiver performance parameters for MSD analysis

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| n79 TX power at antenna port (Primary path) | 23 | 23 | dBm |
| n79 PA output noise power at n78 freq | -130 | -130 | dBm/Hz |
| n79 noise power at n78 PRX ANT port | -91.9 | NA | dBm/10MHz |
| n79 noise power at n78 DRX ANT port | NA | -102.9 | dBm/10MHz |
| Combined thermal noise at RX ant port | -159.5 |  | dBm/Hz |
| TX IM2 in n78 LNA input port | -76 | -98 | dBm |
| Total noise level refer to receiver LNA input port | -75.8 | -92.1 | dBm |
| Combined |  |  |  |
| MSD (10MHz BW) | 3.2 |  | dB |

Table 2.2-2 Link analysis for n78 10MHz REFSENS calculation

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| n78 TX power at antenna port (Primary path) | 23 | 23 | dBm |
| n78 PA output noise power at n79 freq | -130 | -130 | dBm/Hz |
| n78 noise power at n79 PRX Filter-ANT port | -85.4 | NA | dBm/40MHz |
| n78 noise power at n79 DRX Filter-ANT port | NA | -96.4 | dBm/40MHz |
| Combined thermal noise at RX ant port | -159.5 |  | dBm/Hz |
| TX IM2 in n79 LNA input port | -76 | -98 | dBm |
| Total noise level refer to receiver LNA input port | -75.1 | -86.6 | dBm |
| Combined |  |  |  |
| MSD (40MHz BW) | 2.3 |  | dB |

Table 2.2-3 Link analysis for n79 40MHz REFSENS calculation

From above analysis we can see, even with shared antenna architecture (with the help of diplexer), the MSD would still be innegligible.

Obervation 3: Even with shared antenna architecture, MSD is still innegligible.

With the observation 1 and 2, we can see no matter what architecture the CA combination adopts, the MSD would be always required once the CA requires simultaneous RX/TX operation.

Proposal 1: MSD is needed for CA_n78-n79 if it requires simultaneous RX/TX operation.

Reference:

- R4-1906224 Handling of filter assumption of n77 and n78, WG4 Meeting #91

- R4-1800251 TP for TR 37.865-01-01 2DL/1UL CA_n78A-n79A

- TR 38.813 New frequency range for NR (3.3-4.2 GHz)

- TR 38.814 New frequency range for NR (4.4-5.0 GHz)