3GPP TSG-RAN WG4 RAN4#92 Meeting	R4-1910298

# Ljubljana, Slovenia Aug 26 – 30, 2019

Source:	Sprint

Title:	TP for TR 38.716-01-01: CA_n25(2A) REFSENS

Agenda item:	10.1.2

Document for:	Approval

# Introduction

This contribution is a text proposal for TR 38.716-01-01 for CA_n25(2A) REFSENS. The REFSENS is based on the LTE REFSENS for CA_25A-25A.

This revision of the TP uses the table format proposed in R4-1908552. Also, a change to the text in 38.101-1 and a not to the rapporteur is included, that the highlighted change needs to appear in

# Text Proposal

<Start of Text Proposal>

## 6.3	CA_2DL_n25(2A)_1UL_n25A

### 6.3.1	Channel bandwidths per operating band for CA

Table 6.3.1-1: Supported bandwidth combinations for CA_2DL_n25(2A)_1UL _n25A

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_n25(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  | 40 | 0 |

### 6.3.2	Co-existence studies

There are no co-existence issues for this combination.

### 6.3.3	REFSENS

REFSENS can be impacted by the PCC UL being closer to do the SCC DL than the nominal spacing.

NOTE to rapporteur: The paragraph below has highlighted necessary changes to 38.101-1  section 7.3A.2.2 that will need to be included in the big CR.

For intra-band non-contiguous carrier aggregation with FDL_low < 2700 MHz and FUL_low < 2700 MHz with one uplink carrier and two or more downlink sub-blocks, throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.2-1, Table 7.3.2-2, and Table 7.3A.2.2-1 with the reference sensitivity power level increased by D RIBNC  given in Table 7.3A.2.2-1 for the SCC(s). For aggregation of more than two or more downlink FDD carriers with one uplink carrier the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.2.2-1. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3-1) configured.

| CA configuration | SCS kHz | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) | 15 | 25RB+25RB | Wgap = 55.0 | 101 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. |  |  |  |  |  |  |

<End of Text Proposal>