3GPP TSG-RAN WG4 Meeting #92                                                               R4-1910489

Ljubljana, SI, 26th-30th August, 2019

Source: 	Huawei, HiSilicon

Title:	TP on specification and conformance impact

Agenda Item:	8.12.2

Document for:	Approval

1	Introduction

In last RAN4 meeting #91, following agreements on specification and conformance impact were approved for NB-IoT coexistence with NR.

- Agreements on specification and conformance impact

MSR BS supporting NR and LTE+NB-IoT (LTE in-band/guard band) should also meet current RF requirements when operating NR+(LTE)+NB-IoT (NR in-band) with following limitations:

    - Same NR channel bandwidth as LTE

    - NR uses 15 kHz SCS

    - NB-IoT carrier frequency kept when operating with LTE and NR, or shifted closer to NR carrier.

Therefore:

    - MSR BS could operate NB-IoT in NR in-band with above limitations.

    - No need to redo conformance testing in this case.

To remove any limitation:

    - Add support for NB-IoT operating in NR in-band in TS 38.104, TS 38.141-1 and TS 37.141

    - Rel-15 NR BS can declare to support Rel-16 NB-IoT RF requirements for NB-IoT coexistence with NR.

In this contribution we provide TP on NB-IoT operating in NR guard band.

Text proposal

5.6	Specification and conformance impact

MSR BS supporting NR and LTE+NB-IoT (LTE in-band/guard band) could also meet current RF requirements when operating NR+(LTE)+NB-IoT (NR in-band) with following limitations:

- Same NR channel bandwidth as LTE

- NR uses 15 kHz SCS

- NB-IoT carrier frequency kept when operating with LTE and NR, or shifted closer to NR carrier.

Therefore, MSR BS could operate NB-IoT in NR in-band with above limitations and no need to redo conformance testing in this case.

To remove any limitation, specification update is needed to add support for NB-IoT operating in NR in-band in TS 38.104, TS 38.141-1, TS 37.104 and TS 37.141 in Rel-16 the latest.

References

[1] R4-1907809, WF on coexistence of NB-IoT with NR, Huawei, HiSilicon, Ericsson, Nokia,

ZTE, DISH, T-Mobile USA, CHTTL