3GPP TSG RAN WG4 Meeting #92-Bis	R4-1910707

Chongqing, China, 14th – 18th Oct, 2019

3GPP TSG RAN WG1 Meeting #98						R1-1909874

Prague, CZ, August 26th – 30th, 2019

Title:	LS to RAN4 on NR V2X S-SSB design

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN1

To:	RAN4

Cc:

Contact Person:

#### Name:	Teng Ma

#### E-mail Address:	mateng@catt.cn

Attachments:	None

1. Overall Description:

In NR V2X synchronization discussion, RAN1 reached the following agreements:

Agreements: - In NR V2X, S-SSB bandwidth is 11RBs. - PSBCH spans 11RBs. - The S-SSB is designed following combination 1. - Length-127 M-sequences for S-PSS and length-127 Gold sequences for S-SSS - Two symbols are used for each of S-PSS and S-SSS, respectively. Agreements: - For the evaluation of PSBCH performance, the payload size of NR V2X PSBCH is 56 bits including 24 bits of CRC. Working assumption: - For the NR SLSS, - Same sequence is used for both symbols of S-PSS - Same sequence is used for both symbols of S-SSS Agreements: - NR S-SSB structure for NCP is as follows: - For the case of ECP, the structure is the same as the above except that the number of PSBCH symbols after S-SSS is only 6 Send LS to RAN4 with the above agreements, and add: - It is up to RAN4 to decide whether a transient period is necessary or not. If so, to address the transient period, one possibility is to shift the symbols starting the first S-SSS symbol by at least one symbol.

2. Actions:

To RAN WG4:

RAN1 respectfully requests RAN4 to take the above agreement into consideration in their future work.

It is up to RAN4 to decide whether a transient period is necessary or not. If so, to address the transient period, one possibility is to shift the symbols starting the first S-SSS symbol by at least one symbol.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #98bis                 October 14 – 18, 2019    		Chongqing, China

TSG-RAN WG1 Meeting #99                      November 18 – 22, 2019    		Reno, USA