3GPP TSG-RAN WG4 Meeting #92Bis	R4-1910725

# Chongqing, China, 14th– 18th October, 2019

Source:	Verizon, Nokia, Ericsson, Qualcomm

Title:	Band n48 CA bandwidth class and CA configuration

Agenda item:			 9.1.2

Document for:	Approval

- Introduction

RAN4 has defined the NR CA configuration for band n48 in Table 5.3.5-1 of [1]. The range of channel bandwidth is from 5, 10, 15, 20, 40, 50, 60, 80 90 to 100MHz.

Alternative requirement of band n48 is this band will be in operation for both licenses and general authorized unlicensed accesses, and certain requirement of NR CA configuration should be considered.

We provide requirements in this contribution for the intra-band band n48 CA operation.

- Discussion

Though band n48 is a common TDD band, the band operation environment is slight difference from others as it needs to support a mixed licenses and unlicensed accesses. The configurations requirement should reflect this type of operation.

  - NR CA bandwidth class for band n48

Both CA_n48B and CA_n48C configurations for CA operation are specified in Table 5.5A.1-1 of [1] in which two component carriers are allowed along with two bandwidth combination sets.

The defined NR CA bandwidth class in Table 5.5A.1-1 of [1] indicates the maximum aggregated bandwidth for the band n48 is either less than 50MHz or larger than 100MHz, but no an aggregated bandwidth allowed in between 50 to 100MHz. The gap of aggregated bandwidth from 50 to 100MHz is big as the total bandwidth of the band n48 is only 150MHz.

In addition, selection of NR CA configuration is another crucial requirement to stabilize the band operation. The environment of band n48 operation is to support a mixed licensed (or non-contention based) and unlicensed (or contention-based) accesses for the services to both operators and enterprises. For stabilizing the configured CA component carriers in instant service from competition of resource, multiple configurations (e.g., classes B to F) should be get avoid when the resource condition (either licensed or unlicensed) is slightly updated. This specific operation environment should be reflected in the defined NR CA bandwidth class.

For avoiding possible issues, we suggest to extend the CA bandwidth Class B to 100MHz as a specific requirement for the band n48, based on the objective specified in the work item [3], i.e., “Including generic and band specific requirements for intra-band non-contiguous CA…” for the band n48.

Proposal 1: Extend the Class B upper limit to 100MHz from 50MHz in Table 5.3A.5-1 of [1] for the aggregated channel bandwidth for band n48 operation.

  - CA configuration for band n48

Considering our proposal for the 20 - 40MHz ranges of uplink aggregated bandwidth in R4-1910724 [4] and taking account of the study result in the contribution [2], the set of n48 configurations in Table 5.5A.1-1 of [1] could be modified to the table below.

Proposal 2: Update the band n48 CA configuration jointly with the bandwidth Class B to support both uplink and downlink CA configuration.

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA for fallback group 1

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |  |
| CA_n41C | - | 40 | 80, 100 |  |  |  | 180 | 0 |  |
|  |  | 50 | 60, 80, 100 |  |  |  |  |  |  |
|  |  | 60 | 50, 60, 80, 100 |  |  |  |  |  |  |
|  |  | 80 | 40, 50, 60, 80, 100 |  |  |  |  |  |  |
|  |  | 100 | 40, 50, 60, 80 |  |  |  |  |  |  |
| CA_n48B | CA_n48B3 | 5, 10 | 10, 15, 20, 40 |  |  |  | 50 | 0 |  |
|  |  | 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |  |
|  |  | 40 | 5, 10 |  |  |  |  |  |  |
|  |  | 10 | 50, 60, 80, 90 |  |  |  | 100 | 1 |  |
|  |  | 15, 20 | 40, 50, 60, 80 |  |  |  |  |  |  |
|  |  | 40 | 40, 50, 60 |  |  |  |  |  |  |
|  |  | 50 | 50 |  |  |  |  |  |  |
| CA_n48C |  | 10, 15 | 90, 100 |  |  |  | 150 | 0 |  |
|  |  | 20 |  | 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  | 60, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 50 |  | 50, 60, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | 40, 50, 60, 80, 90 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 80 |  | 20, 40, 50, 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 90 |  | 10, 15, 20, 40, 50, 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 100 |  | 10, 15, 20, 40, 50 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66B | - | 5 2 | 20, 40 |  |  |  | 50 | 0 |  |
|  |  | 10 | 15, 20, 40 |  |  |  |  |  |  |
|  |  | 15 | 10, 15, 20 |  |  |  |  |  |  |
|  |  | 20 | 5 2, 10, 15 |  |  |  |  |  |  |
|  |  | 40 | 5 2, 10 |  |  |  |  |  |  |
| NOTE 1: Unless otherwise stated, minimum requirements are applicable irrespective of the order of the component carriers. NOTE 2: 5 MHz is not applicable for 30/60 kHz SCS. NOTE3: In uplink CA configuration, the aggregated bandwidth is restricted to 40MHz when carrier is configured as an downlink CA configuration |  |  |  |  |  |  |  |  |

- Conclusion

In our view, to extend the Class B upper limit for CA_n48B could support varied applications by more band combinations. Hence, our two proposals are

Proposal 1: Extend the Class B upper limit to 100MHz from 50MHz in Table 5.3A.5-1 of [1] for the aggregated channel bandwidth for band n48 operation.

Proposal 2: Update the band n48 CA configuration jointly with the bandwidth Class B to support both uplink and downlink CA configuration.

Reference

[1]	38.101-1 (16.1.0), User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16)

[2]	R4-1900055, TP to TR 38.173: n48 A-MPR, Nokia

[3] RP-192282, WID revision: RF requirements for NR frequency range 1 (FR1), Huawei

[4] R4-1910724, Requirement of n48 uplink intra-band CA, Verizon, Nokia, Ericsson, Qualcomm