3GPP TSG RAN WG4 Meeting #92bis	R4-1910744

Chongqing, China, 14 - 18 Oct 2019

Agenda item:	7.3.2

Source:	Sprint

Title:	TP for TR 36.716-03-01: LTE CA_25-26-41

Document for:	Approval

# 1	Introduction

This TP is to capture the basic aspects for LTE CA_25A-25A-26A-41D/E/F with single UL. The requirements for these combinations are the same as existing combinations LTE CA_25A-25A-26A-41A/C in TS 36.101.

# 2	Text proposal

----- < Start of Text Proposal for TR 38.716-03-01>-----

## 5.x	CA_25-26-41

### 5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  | Uplink CA configurations |  | E-UTRA Bands |  | 1.4 |  | 3 |  | 5 |  | 10 |  | 15 |  | 20 | Maximum aggregated bandwidth |  | Bandwidth combination set |
|  |  |  |  |  |  | MHz |  | MHz |  | MHz |  | MHz |  | MHz |  | MHz | [MHz] |  |  |
| CA_25A-25A-26A-41D |  | - |  | 25 |  | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  | 105 | 0 |  |  |
|  |  |  |  | 26 |  |  |  | yes |  | yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | See CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-26A-41E |  | - |  | 25 |  | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  | 125 | 0 |  |  |
|  |  |  |  | 26 |  |  |  | yes |  | yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | See CA_41E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-26A-41F |  | - |  | 25 |  | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  | 145 | 0 |  |  |
|  |  |  |  | 26 |  |  |  | yes |  | yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | See CA_41F Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.x.2	∆TIB and ∆RIB values

For CA_25-25-26-41, the TIB,c and RIB,c values are the same as for existing CA-25-26-41 combinations in TS 36.101.

### 5.x.3	REFSENS requirements

The REFSENS requirements for CA_25A-25A-26A-41D/E/F are the same as for existing CA combinations CA_25A-25A-26A-41A/C in TS 36.101.

----- < End of Text Proposal >-----

# 3	References

- RP-192083, WID revision: Rel-16 LTE inter-band CA for 3 bands DL with 1 band UL, Huawei, HiSilicon.

- R4-1904936, 36.716-03-01 - Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Advanced inter-band Carrier Aggregation (CA) Rel-16 for 3 bands Down Link (DL) / 1 band Up Link (UL) , Huawei, HiSilicon.