3GPP TSG-RAN WG4 Meeting #92bis                                                                                        R4-1910827

Chongqing, China, 14th - 18th Oct, 2019

Agenda item:            8.17.2.2

Source:	MediaTek Inc.

Title:	Discussion on PDSCH simulation assumptions for NR-HST UE

Document for:          Discussion

# 1	Introduction

It was agreed in RAN4#92 to further study the options regarding to following aspects:

- Feasible maximum Doppler frequency;

- DMRS configuration between 1+2 and 1+3; and

- Rank and MCS

In this paper, we present the simulation results of PDSCH for NR-HST channel and provide our observations and proposals for the HST test case setting.

# 2	Discussion on NR-HST test case setting

2.1 Maximum Doppler frequency

Based on the simulation assumption in [1], the following we provide the corresponding simulation results. Table 2.1 and Table 2.2 present the required SNR (in dB) at 70% throughput in HST-SFN channel for FDD and TDD cases, respectively, with 2 additional DMRS configuration. Table A.1 and Table A.2 (in Appendix) contains detailed parameter setting in the simulation.

Table 2.1 Required SNR (dB) @ 70% Tput for HST-SFN FDD cases

| Doppler shift | MCS | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Rank = 1 | Rank = 2 | Rank = 1 | Rank = 2 |
| For 500km/h |  |  |  |  |  |
| 1250 Hz | 4 | 1.17 | 4.87 | -0.85 | 2.05 |
|  | 13 | Error floor | Error floor | Error floor | Error floor |
|  | 17 | Error floor | Error floor | Error floor | Error floor |
| 972 Hz | 4 | -0.85 | 2.15 | -0.86 | 2.14 |
|  | 13 | 12.11 | 17.18 | 12.09 | 14.51 |
|  | 17 | Error floor | Error floor | Error floor | Error floor |
| 875 Hz | 4 | -1.43 | 1.53 | -4 | -0.36 |
|  | 13 | 8.43 | 12.22 | 5.97 | 10.31 |
|  | 17 | 29.82 | Error floor | 29.47 | Error floor |
| 780 Hz | 4 | -1.67 | 1.26 | -4.22 | -1.22 |
|  | 13 | 6.27 | 9.86 | 3.49 | 7.18 |
|  | 17 | 10.28 | 14.23 | 7.75 | 11.58 |
| For 350km/h |  |  |  |  |  |
| 875 Hz | 4 | -1.16 | 1.61 | -4.04 | -0.41 |
|  | 13 | 8.79 | 12.61 | 6.74 | 10.21 |
|  | 17 | Error floor | Error floor | Error floor | Error floor |
| 780 Hz | 4 | -1.64 | 1.25 | -4.27 | -1.1 |
|  | 13 | 6.63 | 9.87 | 3.96 | 7.52 |
|  | 17 | 10.27 | 14.44 | 8.14 | 11.83 |

Table 2.2 Required SNR (dB) @ 70% Tput for HST-SFN TDD cases

| Doppler shift | MCS | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Rank = 1 | Rank = 2 | Rank = 1 | Rank = 2 |
| For 500km/h |  |  |  |  |  |
| 1667 Hz | 4 | -1.34 | 1.75 | -4.12 | -0.68 |
|  | 13 | 10.26 | 14.39 | 9.03 | 12.88 |
|  | 17 | 19.27 | 24.25 | 20.12 | 24.66 |
| 1500 Hz | 4 | -1.71 | 1.43 | -4.31 | -1.08 |
|  | 13 | 7.77 | 11.18 | 5.55 | 9.64 |
|  | 17 | 11.69 | 17.28 | 10.41 | 14.44 |
| 1167 Hz | 4 | -1.86 | 1.24 | -4.55 | -1.2 |
|  | 13 | 7.47 | 11.37 | 4.76 | 8.73 |
|  | 17 | 11.81 | 17.71 | 9.6 | 14.56 |

For HST-SFN FDD case, we can observe that when maximum Doppler shift equals to 1250Hz, error floor appears in 16-QAM and 64-QAM due to strong ICI effect. Therefore, in light of the stability of demodulation, we don’t think it’s feasible to consider such a large Doppler frequency in the FDD test cases. For lower Doppler frequency such as 972Hz and 875Hz, it’s feasible to support up to 16-QAM, and even up to 64-QAM for 780Hz Doppler shift.

For HST-SFN TDD case, it has better tolerance to UE mobility due to the larger SCS. As a result, it can support up to 1667Hz Doppler which corresponds to the 3.6GHz carrier frequency and 500km/h train speed.

In single-tap HST channel, UE may percept the maximum Doppler shift as frequency error and try to compensate it by AFC. NR defined TRS to facilitate UE for time/frequency tracking. The theoretical maximum frequency offset tracking range with TRS based FOE (frequency offset estimator) are listed in the following table.

Table 2.3 Maximum frequency offset by TRS based FOE

| SCS | Max. frequency offset can be estimated |
| --- | --- |
| 15kHz | 1750 Hz |
| 30kHz | 3500 Hz |

The most crucial situation for UE frequency tracking happen during transient period when Doppler frequency changes from +fd to -fd. The frequency change can be up to 2×fd instantaneously, where fd is the maximum Doppler frequency. Taking 1250 Hz Doppler shift (option 1) as an example, the expected frequency error can be as large as 2500Hz, which is beyond the locking range of TRS FOE and will cause wrap around error. As a result, to ensure the robustness of UE frequency offset estimation, the maximum Doppler frequency for FDD cases (w. 15 kHz SCS) should not exceed 875Hz.

In the case of TDD, the maximum feasible Doppler frequency can up to 1667Hz, because 2×fd is within TRS FOE locking range.  This can be verified by the simulation results shown in Table 2.4 for FDD and TDD with 875Hz and 1667Hz, respectively.

Table 2.4 Required SNR (dB) @ 70% Tput for single-tap HST cases

| FDD 15kHz SCS |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Doppler shift |  | MCS | Rank = 1 |  |  |  | Rank = 2 |  |  |  |
|  |  |  | 1x2 |  | 1x4 |  | 2x2 |  | 2x4 |  |
|  |  |  | AddPos2 | AddPos3 | AddPos2 | AddPos3 | AddPos2 | AddPos3 | AddPos2 | AddPos3 |
| 875 Hz |  | 13 | 4.46 | 4.45 | 1.502 | 1.48 | 7.48 | 7.52 | 4.54 | 4.59 |
| TDD 30kHz SCS |  |  |  |  |  |  |  |  |  |  |
| 1667 Hz |  | 13 | 4.43 | 4.38 | 1.9 | 1.7 | 6.93 | 6.95 | 4.56 | 4.51 |

Proposal #1:

- For HST-SFN, the feasible maximum Doppler frequency is:

  - 975Hz for FDD; and

  - 1667Hz for TDD

- For single-tap HST channel, the feasible maximum Doppler frequency is:

  - 875Hz for FDD; and

  - 1667Hz for TDD

2.2 DMRS symbol number

We analyze the impact of different DMRS symbol numbers on demodulation performance. Figure 2.1 shows the BLER and Tput curve with different DMRS number under FDD cases with maximum Doppler shift = 972Hz, 2x4 antenna configuration and rank 2. From the BLER curves, 3 additional DMRS has 0.5dB SNR gain compared to 2 additional DMRS due to more accurate channel estimation results. However, 3 additional DMRS configuration increases the pilot overhead and thus reduce the throughput accordingly. Considering the complexity of UE demodulation, 2 additional DMRS is enough to handle the high Doppler spread and should be more practical for HST scenario.

Proposal #2: Use 2 additional PDSCH DMRS symbols (Option 1: 1+1+1) for NR-HST test cases

Figure 2.1 HST performance with different DMRS number

2.3 Rank and MCS

For 4-tap HST-SFN channel, we conducted some evaluations to investigate the throughput under different rank and MCS. We compare the absolute Tput values and choose the appropriate rank and MCS according to different scenarios.

The simulation results in Figure 2.2 shows the Tput under FDD cases with maximum Doppler shift = 875Hz and 2x2 antenna configuration. We can observe that for middle to high SNR range, Tput of rank 2 is better than that of rank 1. Besides, mcs13 (16-QAM) can provide better Tput than mcs17 (64-QAM). For low SNR range, rank 1 with 16-QAM is enough.

Figure 2.3 shows the Tput under TDD cases with maximum Doppler shift = 1500Hz and 2x2 antenna configuration. It can be observed that rank 2 universally provides better Tput over wide SNR range. For high SNR range, 64-QAM can achieve better Tput.

Figure 2.2 FDD performance with different Rank and MCS

Figure 2.3 TDD performance with different Rank and MCS

The same method for analyzing proper rank and MCS values can also be used for different Doppler and antenna configuration. We summarized the results in Table 2.5. It’s observed that the rank 2 with 16-QAM or 64-QAM are more suitable in the operating SNR range.

Proposal #3: Considering various operating SNR in HST, we propose to adopt the following Rank/MCS for PDSCH setting:

- Rank2, MCS4 (QPSK) for low SNR

- Rank2, MCS13 (16QAM) for medium-to-high SNR

Table 2.5 Propose Rank and MCS

| FDD (2x2) |  |  |  | FDD (2x4) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Doppler | SNR range | MCS | Rank | Doppler | SNR range | MCS | Rank |
| Very high (≈ 1250Hz) | 0~5 dB | QPSK | 2 | Very high (≈ 1250Hz) | 0~5 dB | QPSK | 2 |
| High (> 780Hz) | 5~20 dB | 16-QAM | 2 | High (> 780Hz) | 5~20 dB | 16-QAM | 2 |
| Middle (≤ 780Hz) | 5~12 dB | 16-QAM | 2 | Middle (≤ 780Hz) | 5~10 dB | 16-QAM | 2 |
|  | ≥12 dB | 64-QAM | 2 |  | ≥10 dB | 64-QAM | 2 |
| TDD (2x2) |  |  |  | TDD (2x4) |  |  |  |
| Doppler | SNR range | MCS | Rank | Doppler | SNR range | MCS | Rank |
| Very high (≈ 1667Hz) | 5~20 dB | 16-QAM | 2 | Very high (≈ 1667Hz) | 5~15 dB | 16-QAM | 2 |
|  | ≥20 dB | 64-QAM | 2 |  | ≥15 dB | 64-QAM | 2 |
| High (> 1167Hz) | 5~15 dB | 16-QAM | 2 | High (> 1167Hz) | 5~12 dB | 16-QAM | 2 |
|  | ≥15 dB | 64-QAM | 2 |  | ≥12 dB | 64-QAM | 2 |

# 3	Conclusions

In the contribution, we provide our simulation results of PDSCH HST tests and following proposals:

Proposal #1:

- For HST-SFN, the feasible maximum Doppler frequency is:

  - 975Hz for FDD; and

  - 1667Hz for TDD

- For single-tap HST channel, the feasible maximum Doppler frequency is:

  - 875Hz for FDD; and

  - 1667Hz for TDD

Proposal #2: Use 2 additional PDSCH DMRS symbols (Option 1: 1+1+1) for NR-HST test cases

Proposal #3: Considering various operating SNR in HST, we propose to adopt the following Rank/MCS for PDSCH setting:

- Rank2, MCS4 (QPSK) for low SNR

- Rank2, MCS13 (16QAM) for medium-to-high SNR

# 4	Reference

[1] R4-1910050, “WF on UE demodulation for NR HST”, CMCC.

Appendix

Table A.1 Simulation assumption for HST-SFN channel

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2x2; 2x4, |  |
| Precoding scheme | Random precoding based on type1 single panel codebook, bundle size 2 |  |
| Front loaded DMRS symbol | L0=2 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | - Option 1: DMRS 1+1+1 - Option 2: DMRS 1+1+1+1 |  |
| MCS | MCS 4; MCS 13; MCS 17 based on 64QAM table |  |
| Propagation condition | HST-SFN |  |
| TDD pattern | DDDDDDDSUU ,S(6D:4G:4U) |  |
| TRS periodicity | 10ms, 2 slot TRS |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds=1000m, Dmin=50m |  |
| Rank | Option 1: Rank = 1 Option 2: Rank = 2 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | - For 500km/h - Option 2: 1250Hz - Option 3: 875Hz - Option 4: 972Hz - Option 5: 780Hz - For 350km/h - Option 1: 875Hz - Option 2: 780Hz | - For 500km/h - Option 1: 1667Hz - Option 2: 1500Hz - For 350km/h - Option 1: 1167Hz |
| Testing metric | SNR @70% of maximum throughput |  |

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 1x2, 1x4 (rank 1); 2x2; 2x4 (rank 2) |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | - Option 1: DMRS 1+1+1 - Option 2: DMRS 1+1+1+1 |  |
| Precoding scheme | Random precoding based on type1 single panel codebook, bundle size 2 |  |
| TDD pattern: | DDDDDDDSUU ,S(6D:4G:4U) |  |
| Front loaded DMRS symbol | L0=2 |  |
| Propagation condition | HST-single tap |  |
| TRS periodicity | 10ms, 2 slot |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds= 300m, Dmin=2m |  |
| Rank | Option 1: Rank = 1 Option 2: Rank = 2 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | 875Hz | 1667Hz |
| Testing metric | SNR @70% of maximum throughput |  |

Table A.2 Simulation assumption for HST-single tap channel