3GPP TSG-RAN WG4 Meeting #92-Bis	R4-1910854

# Chongqing, China, 14th – 18th Oct, 2019

Source: 	Huawei, HiSilicon

Title:	Further consideration on adding wider channel bandwidths in Band n77/n78

Agenda Item:	9.19

Document for:	Discussion

1	Introduction

In last RAN plenary meeting #84, a new WI was approved to add wider channel bandwidths in NR band n77 and n78 [1].

In last RAN4#92 meeting, the CR for UE and BS were technical endorsed in [2] [3]. For this WI there is two things we need further consideration. 1) For 70 MHz should it be mandatory or optional in Rel-16? 2) How to make it optional support from Rel-15 for 25 MHz and 30 MHz?  In this contribution, we provide considerations and solutions on the two aspects.

2	Discussion

Optional support 25 MHz and 30 MHz in Rel-15

There are two approaches discussed in [4]. One approach is to add these channel bandwidths in Rel-15 38.101 and mark them as optional. The other is to include the channel bandwidth as a release independent feature in 38.307. From the discussion last meeting, it seems the first option might not be doable since Cat. B CR should be avoided in Rel-15 specifications. Hence it is proposed to add a sub-clause for channel bandwidth in TS 38.307 as below.

============= Proposed changes for TS 38.307 =============

5.4	Additional NR UE channel bandwidth for NR frequency range 1

Requirements for a Rel-15 UE for additional NR UE channel bandwidths compared to TS 38.101-1 of Rel-15 [2] are introduced via this clause.

Table 5.4-1: NR operating bands

| Feature | SCS (kHz) | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the UE channel bandwidth was introduced for the operating band) |
| --- | --- | --- | --- |
| UE channel bandwidths | 15, 30, 60 | Rel-15 |  |

Support of 70 MHz

As discussed, 25 MHz and 30 MHz UE channel bandwidth are already included for other NR bands in Rel-15, and the signalling can support these channel bandwidths. Meanwhile for 70 MHz channel bandwidth, UE capability signalling for 70 MHz CBW need to be introduced in RAN2 specifications. The remaining question is should it be mandatory or optional in Rel-16. It is noted RAN4 agreed that 90 MHz be a mandatory channel BW for NR UE from REL-16 onwards, considering Band n78 spectrum auctions in Europe. Operator acquired 70 MHz of spectrum in Germany in band n78. For the same reason we propose to define 70 MHz as a mandatory channel bandwidth for n77 and n78.

3	Conclusions

In this contribution we provide further consideration on the addition of new channel bandwidth for band n77 and n78. The corresponding CRs can be found in [7], [8] and LS to RAN2 can be found in [6].

4	References

[1] RP-191606, New WI proposal: Addition of channel bandwidths in Band n77 and n78

[2] R4-1910513, draft CR for TS 38.101-1: adding wider channel bandwidths, Huawei

[3] R4-1909209, draft CR for TS 38.104: adding wider channel bandwidths, Huawei

[4] R4-1909206,	Discussion on adding wider channel bandwidths in Band n77/n78, Huawei

[5] RP-181443, WF on IOdT Bits for SCS and Channel Bandwidths

[6] R4-1910858, LS to RAN2 on adding 70MHz UE CBW in Band n77/n78

[7] R4-1910857, CR for 38.307, additional UE channel bandwidth

[8] R4-1910855, CR for TS 38.101-1: adding wider channel bandwidths, Huawei